3GPP TSG-RAN WG2 Meeting #132	R2-2508543

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.3.1.2 (User plane)

Source:	Sony

Title:	Discussion on User plane QoS aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 the following study item was agreed with following objectives focused on user plane[1]:

- At RANP#108, the 6G study item was agreed [1], where mobility related part of the study item is captured as follows: Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

At RAN2#131bis, the following agreements were made:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2. 	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we share our views on QoS related issues like whether a single DRB or multiple DRBs should be supported in 6GR, L4S performance due to radio retransmissions, and improvement and QoS requirements for multimodality.

# 2 Discussion

## 2.1 Number of DRBs

There is one PDCP entity configured for each Radio Bearer in NR. For each radio bearer, compression and security functions are located in PDCP entity. For multiple DRB case, as currently specified for NR, a separate ROHC entity per radio bearer is used. ROHC is normally used for applications like voice where payload size is generally smaller than the header size.

If a single DRB carries all of the QoS flows and some of them needed ROHC and others don’t, then a single PDCP state machine might require a packet filter to separate the compressed and uncompressed header packets within the same state machine. One option is that uncompressed header traffic is also fed to the ROHC entity but this may not be optimal for ROHC performance. The other option is that a radio bearer is mapped to different PDCP entities. This extra effort for handling all QoS flows in a single DRB might be similar to creating separate multiple DRBs and in some cases introduce complexity.

Observation 1: Handling of ROHC, when a single RB carries traffic related to all QoS flows/parameters, requires further work and may introduce complexity while ROHC is already supported efficiently for multiple DRB case.

For security, input parameters like bearer id and count will be different for different DRBs. This might require extra processing in the UE implementation if multiple DRBs are supported.

Observation 2: Security input parameters remain the same when a single RB carries traffic related to all QoS flows. Some implementations may have extra burden for processing multiple input parameters per DRB (e.g. bearer id and count).

Each PDCP entity is mapped to either one (for RLC-AM mode) or two RLC entities (for RLC-UM/TM mode and separate for UL and DL) for non-DC scenario.it can be agnostic to if one RB is mapped to single or multiple PDCP entities. Similarly, MAC can be agnostic if single or multiple DRBs are used for all QOS flows.

Observation 3: mapping between PDCP entity and RLC entity(ies) can follow NR model and not an issue for single or multiple DRBs.

Based on above discussions, we think ROHC/EHC handling might be complex if a single DRB is allowed to carry traffic related to all QoS parameters. This was not deployed because there was no use case. So, we propose that RAN2 should not discuss a new solution when problem with NR framework is not clear:

Proposal 1	Current NR framework of supporting multiple DRBs is already flexible. A single DRB-based approach should not be studied.

## 2.2 L4S performance improvement

The congestion bit in IP header is set when there is an expected drop in the data rate so that the other end can lower the data rate. This is decided based on the buffer build up in application layer. However, L4S bit setting may be slow and occur after the congestion has already built up leading to setting the bit as useless. This can happen in one scenario where lower layers e.g. ARQ/HARQ are performing retransmissions. This may lead to a buffer build up in the application layer and congestion bit is still not set.  This delay may occur because of many reasons like pre-processing in PDCP/RLC layers where packets are already stored in L2 buffer with encrypted header and the bit in IP header can no longer be set. Also, UE modem may not provide an API to OS about HARQ retransmissions in real time.

Alternatively, gNB DU hosting HARQ and ARQ and the physical distance between this (DU) entity and application server may result in the information shared being irrelevant due to the delay in sharing the information. An early indication from lower layers to application layer about retransmissions may result in L4S bits being set early enough to maintain its usefulness.

Proposal 2	RAN2 to investigate cross layer optimization so that the congestion bit setting in IP header remains relevant.

## 2.3 Multimodality QoS requirements

At the last RAN2 meeting [2] it was agreed to study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications).

Immersive Reality is one of the main features driving and basis for 6G RAN requirement study. The QoS framework needs to be able to handle the increasing need and requirements for timely delivery of time critical communication and in particular packets belonging to the same XR service that supports interrelated multimodality streams, both in uplink and downlink direction. In 5G, RAN is aware of the streams which are interrelated but is not aware of the interrelated QoS requirements for these streams. In order to handle strict multimodality, e.g., relative latency, requirements between interrelated data streams from the application, RAN2 should study whether the information of the interrelated QoS requirements for the interrelated data streams should be known by RAN. One example is the relation between audio and video, where a relatively strict synchronization is needed, but other applications may require even stricter synchronization.

Proposal 3	RAN2 to study RAN Awareness of the interrelated QoS requirements for the multimodality streams.

# 3 Conclusion

In this contribution, we propose RAN2 to discuss and agree to the following proposals:

Proposal 1	Current NR framework of supporting multiple DRBs is already flexible. A single DRB-based approach should not be studied.

Proposal 2	RAN2 to investigate cross layer optimization so that the congestion bit setting in IP header remains relevant.

Proposal 3	RAN2 to study RAN Awareness of the interrelated QoS requirements for the multimodality streams.

# 4 References

- RP-251881. New WID: Study on 6G Radio

- Minutes RAN2#131bis, Prague.