3GPP TSG-RAN WG2 Meeting #132	R2-2508580

Dallas, USA, Nov. 17th - 21st

Agenda Item:	9.3.2

Source:	Mediatek Inc.

Title:	Discussion on LCM for RRM Prediction and Event Prediction

Document for:	Discussion, Decision

Introduction

In this contribution, we discuss the LCM for both RRM prediction and measurement event prediction. The content covers UE capability, applicability report, and inference configuration (including full inference configuration and inference parameters). We first raise a discussion related to the temporal domain case A, which we think it is recommended that it is supported only within the context of event prediction, rather than as a separate RRM prediction. Then, we discuss the UE capability and propose using separate capability bits for different UE capability (respective to three different use cases). Next detailed full inference configurations and corresponding IEs are provided. For applicability part, we focus on how to support option B. Applicability report configuration for both option A and B are then be discussed. Additionally, we also propose the UE autonomous fallback mechanism if UE detect inapplicable configuration. Furthermore, the timing of performing inference is also discussed.

Discussion

Temporal case A for event prediction

AI mobility encompasses a variety of use cases, including RRM prediction (such as temporal case A, case B, and inter-frequency prediction) and measurement event prediction (both direct and indirect). However, for temporal case A, it is recommended that it is supported only within the context of event prediction, rather than as a separate RRM prediction use case for following reasons:

- Evaluation of RAN2 showed that RRM measurement prediction (temporal case A) reporting to the network solely for early synchronization or preparation (HO modeling option 1 in event prediction) yields only limited performance gain.

- After an event trigger is predicted, the predicted values naturally follow the trend of the event, if the UE is required to report both prediction results/measurement reports and actual measurement results, it introduces additional signaling overhead without delivering meaningful new information.

- It is unclear how the network utilizes that information for handover decision, which is different from current network implementation.

Proposal 1: Temporal case A is not supported as a separate RRM measurement prediction use case. Temporal case A is by default supported for measurement event prediction.

UE Capability

Based on the discussion above, temporal case A is by default supported only for measurement event prediction and does not require separate support for RRM measurement prediction. Accordingly, we propose that UE capabilities for temporal prediction (case B), frequency prediction, and event prediction (case A) be represented by three distinct capability bits: one for RRM temporal domain case B, one for RRM inter-frequency prediction, and one for measurement event prediction (which includes temporal domain case A).

Proposal 2: RAN2 consider separate capability bits to indicate UE capability for (1) RRM temporal domain case B (2) RRM inter-freq prediction (3) measurement event prediction (including temporal case A).

For the capability enquiry and report, referring to the AI BM, the UE can report the corresponding capabilities through the UE-NR-Capability. For AI BM case, the capability indication bit is carried in Phy-Parameters/Phy-ParametersCommon/aiml-BM-Case1-r19/aiml-BM-Case2-r19. As there is no observed reason why this capability indication method would not be feasible for AI mobility use cases, we propose the AI mobility related capability bits can be carried in measAndMobParameters/MeasAndMobParametersCommon as shown below in TS 38.331.

Figure 1: TS38.331 UE-NR-Capability

Proposal 3: The UE reports AI MOB related capability to the NW via measAndMobParameters/MeasAndMobParametersCommon in UE-NR-Capability.

Inference configuration

RAN2 has agreed to reuse the legacy RRM measurement configuration framework as the baseline for AI mobility inference configuration. The legacy RRM measurement configuration contains the information for measurement objects and measurement reports. Here we list the related components for the AI mobility inference configuration as follows.

- Measurement object (MO)

- Report configuration

- Measurement identities

The information of measurement targets is carried by MO, which includes the timing and frequency locations, cells list, etc. The report configuration contains the report triggering condition, e.g., event-triggered, periodical, and the report quality, e.g., RSRP, SINR, RSSI (via ReportQuantityCell). A measurement/report task is identified by a measurement ID which contain one MO ID and one report configuration ID. This framework can also be used for AI mobility inference configuration.

However, it is clear that in AI mobility inference configuration, in addition to the MO, we need to identify the information of the inference target, i.e., the predicted object. It should contain the timing and frequency location of the predicted target. For inter-freq prediction use case, the predicted object is difference from the measurement object. For intra-freq use case, the measurement and predicted object can be the same. Besides that, the configuration should also contain predicted information for different sub-use cases, for example, the prediction window (PW) length for Temporal domain case A, and the skipping pattern for Temporal domain case B.

Observation 1: For UE-sided model RRM prediction, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, etc.

Similar to the legacy measurement, the report configuration provides the measurement report triggering condition and the report quantity, however, for AI mobility use cases, the measurement report can be generated based on not only real measurements but also predicted results. Also, similar to the measurement ID, for AI mobility inference, we can introduce “inference ID” to figure out the mapping of an MO, a predicted object, a report configuration, and an inference report configuration. The overall configuration structure can be shown in the following figure where the inference configuration contains the following components:

- Measurement object (via MeasObject): Describes the information of measurement/observed target.

- Report configuration (via ReportConfig): Describes the triggering condition and report quantity for the measurement report. The measurement report can be generated based on both real measurement and inference results.

- Predicted object (via PredObject): Describes the information of inference/predicted target (can reuse MeasObject if applicable).

- Inference identity (via InferenceId): Refers to an identity (can reuse MeasId if applicable) that provides the mapping of a measurement object, a predicted object, and a report configuration.

Figure 2: Overall structure of inference configuration.

Proposal 4: NW provide full inference configuration to UE via MeasConfig, which contains

- Measurement object (via MeasObject): Describes the information of measurement/observed target.

- Report configuration (via ReportConfig): Describes the triggering condition and report quantity for the measurement report.

- Predicted object (via PredObject): Describes the information of inference/predicted target (can reuse MeasObject if applicable).

- Inference identity (via InferenceId): Refers to an identity (can reuse MeasId if applicable) that provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 5: For intra-freq prediction and measurement event prediction for which prediction object and measurement object refer to the same one, the predicted object could be optional.

Applicability

Regarding the applicability report procedure, RAN2 has agreed to support both Option A and Option B. For Option A, the NW can provide a detailed full inference configuration to the UE through measConfig, and the UE can report the corresponding applicability based on the inferenceId mentioned in previous section. For Option B, the NW provides one or multiple sets of inference parameters to the UE through OtherConfig. However, RAN2 has not reached an agreement on how the UE should report applicability for each set of inference parameters and what the corresponding MOs for the inference parameters should be. Further discussion is required.

How to support Option B

In RAN2 #131 AI-PHY, RAN2 has the following agreement on OtherConfig for option B.

Agreements Other Config for option B - RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. - For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19. - For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

For Option B, NW will provide the one or multiple set of inference parameters to UE via OtherConfig. For AI-BM use case, the inference parameter set is configured per serving cell. In AI mobility, this is inapplicable. In our perspective, the minimal granularity is per MO. One possible modification is to have the inference parameters configured per MO. To enable the UE to know which set of parameters corresponds to which MO(s), there are two options:

- Assumption 1: RAN2 assumes the UE can still receive full configuration i.e. MO information from MeasConfig as Option A in step 3, in addition,  OtherConfig will carry MeasObjectId or InferenceId to link the inference parameters in the corresponding MO for option B

- Assumption 2: RAN2 assumes OtherConfig itself contains MO information and the corresponding mapping of sets of inference parameters.

For Assumption 1, the MO index can be easily connected to the set of inference parameters via MeasObjectId or InferenceId. However, this assumes that the detailed configuration of MeasConfig is still needed for Option B, ,which somewhat undermines the original advantage of Option B in reducing configuration overhead..

Another possibility is that, in step 3, the NW does not provide the UE with relevant MO information through MeasConfig. However, when the UE reports applicability for each set of inference parameters, OtherConfig must include which set of inference parameters corresponds to which MO(s). If this is the case, RAN2 needs to discuss how to carry the MO information in OtherConfig. Otherwise, the UE can only assume that all MOs must be supported.

Proposal 6: RAN2 discuss whether the UE need to have the MO configuration via MeasConfig in step 3. If not, FFS how to indicate the link between the inference parameters and corresponding MOs in OtherConfig.

Applicability Report Configuration

One UE received the configuration in Step 3, it can determine applicability for each full inference configuration (Option A) or each set of inference parameters (Option B).

For Option A, the inferenceId can be used to indicate the ID of a configuration for which the UE should report applicability. For Option B, following the AI-PHY configuration, RAN2 considers using applicabilitySetConfId to indicate the ID of a set of inference parameters for which the UE should report applicability.

For both Option A and Option B, the UE will report its applicability through (1) RRCReconfigurationComplete and (2) UAI, via ApplicabilityReportList.

Proposal 7: For applicability determination procedure,

- Option A: UE determine applicability for each full inference configuration contained in MeasConfig. InferenceId can be used to indicate the ID of the configuration for applicability report.

- Option B: UE determine applicability for each set of inference parameters contained in OtherConfig. applicabilitySetConfigId can be used to indicate the ID of the set of parameters for applicability report.

UE Autonomous Fallback

In RAN#131 meeting, RAN2 have the following agreement about UE detect some inference configuration is not applicable.

Agreements 1. Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility: - UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration. - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. - When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). ……..

We agree that the UE must wait for the NW to release inapplicable configurations and cannot release them on its own. However, we believe that when the UE finds some configurations to be not applicable, it should be able to autonomously fallback to alternative solutions, such as a non-AI approach. This means that the UE does not need to send inference reports for inapplicable configurations. In the AI MOB use case, unsuitable inference results can lead to HOF and RLF. Additionally, whether the inference report uses predicted or actual measurements is also up to UE implementation. Forcing the UE to continue sending inference reports for inapplicable inference configurations only increases system overhead.

Proposal 8: Same as AI-BM, the UE doesn’t autonomous release the inference configuration when it becomes inapplicable. But the UE can autonomously fallback to non-AI operation.

Inference Report for measurement event prediction

Different handover modeling approaches (e.g., option 1 or option 2) for temporal domain case A result in differences in the UE's inference reporting procedures and behavior. For event prediction in temporal case A with handover modeling option 1, the UE is required to report both predicted measurement reports (MRs) based on inference, and real measurement reports based on actual measurements. This dual reporting introduces substantial specification changes and increases signaling overhead in the current system. Moreover, despite these significant impacts on the specification, RAN2 evaluation results indicate that the performance gain achieved by HO modeling option 1 is very limited. In addition, implementing HO modeling option 1 requires the UE to perform continuous inference to detect whether early synchronization and preparation are needed, which results in considerable power consumption for the UE.

Observation 2: Handover modeling option 1 offers very limited performance gain, while causing significant impacts on current specifications, such as increased prediction measurement reporting and signaling overhead.

With handover modeling option 2 for event prediction in temporal case A, the UE can perform inference only when the entry condition is met. It then reports a predicted measurement event at the time instance it is predicted to be triggered. This procedure aligns with legacy MR report and handover processes: once the predicted measurement event (like a legacy A3 event) is received, the network can send the handover command immediately after completing handover preparation. A MeasObject/ReportConfig can correspond to one or more inference configurations. An inference configuration can reference an existing MeasObject/ReportConfig, reusing its measurement objects and reporting mechanisms, while only adding AI-related triggering and processing logic. Therefore, we need an identifier to establish the mapping relationship.

Observation 3: For measurement event prediction in temporal domain case A with HO modeling option 2, as well as in temporal case B, the measurement report can reuse the legacy MR format, resulting in minimal impact on the current specifications.

Proposal 9: Unless additional evidence emerges to justify the necessity of option 1, RAN2 at this stage focus the normative work for the LCM procedure in temporal case A on HO modeling option 2.

Proposal 10: For measurement event prediction with HO modeling option 2, the measurement report can reuse the legacy MR to include real measurement results.

When to perform inference

The other issue is about when to perform inference (At the RAN2 #131 meeting, we noticed that when discussing “when to perform inference”, some proposals refer to the timing of inference activation. Here we focus on the exact timing of performing inference). For RRM prediction use cases with 1st goal, i.e., Temporal case B and frequency domain prediction, the only requirement related to the timing of inference is the latency requirement of the measure report. For those cases, when to perform inference should be UE implementation, as long as UE can meet all the report requirements defined by RAN4. Whether the report requirement should be relaxed or modified for AI use cases can be FFS in RAN4.

Proposal 11: For RRM Temporal domain case B and frequency domain prediction, when to perform inference is up to UE implementation. The UE needs to meet the report requirements determined by RAN4.

Conclusion

Temporal case A for event prediction:

Proposal 1: Temporal case A is not supported as a separate RRM measurement prediction use case. Temporal case A is by default supported for measurement event prediction.

UE capability:

Proposal 2: RAN2 consider separate capability bits to indicate UE capability for (1) RRM temporal domain case B (2) RRM inter-freq prediction (3) measurement event prediction (including temporal case A).

Proposal 3: The UE reports AI MOB related capability to the NW via measAndMobParameters/MeasAndMobParametersCommon in UE-NR-Capability.

Inference configuration:

Observation 1: For UE-sided model RRM prediction, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, etc.

Proposal 4: NW provide full inference configuration to UE via MeasConfig, which contains

- Measurement object (via MeasObject): Describes the information of measurement/observed target.

- Report configuration (via ReportConfig): Describes the triggering condition and report quantity for the measurement report.

- Predicted object (via PredObject): Describes the information of inference/predicted target (can reuse MeasObject if applicable).

- Inference identity (via InferenceId): Refers to an identity (can reuse MeasId if applicable) that provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 5: For intra-freq prediction and measurement event prediction for which prediction object and measurement object refer to the same one, the predicted object could be optional.

Applicability:

How to support Option B:

Proposal 6: RAN2 discuss whether the UE need to have the MO configuration via MeasConfig in step 3. If not, FFS how to indicate the link between the inference parameters and corresponding MOs in OtherConfig.

Applicability Report configuration:

Proposal 7: For applicability determination procedure,

- Option A: UE determine applicability for each full inference configuration contained in MeasConfig. InferenceId can be used to indicate the ID of the configuration for applicability report.

- Option B: UE determine applicability for each set of inference parameters contained in OtherConfig. applicabilitySetConfigId can be used to indicate the ID of the set of parameters for applicability report.

UE Autonomous Fallback:

Proposal 8: Same as AI-BM, the UE doesn’t autonomous release the inference configuration when it becomes inapplicable. But the UE can autonomously fallback to non-AI operation.

Inference Report for measurement event prediction:

Observation 2: Handover modeling option 1 offers very limited performance gain, while causing significant impacts on current specifications, such as increased prediction measurement reporting and signaling overhead.

Proposal 9: Unless additional evidence emerges to justify the necessity of option 1, RAN2 at this stage focus the normative work for the LCM procedure in temporal case A on HO modeling option 2.

Proposal 10: For measurement event prediction with HO modeling option 2, the measurement report can reuse the legacy MR to include real measurement results.

When to perform inference:

Proposal 11: For RRM Temporal domain case B and frequency domain prediction, when to perform inference is up to UE implementation. The UE needs to meet the report requirements determined by RAN4.

Reference