3GPP RAN WG2 Meeting #132	R2-2508722

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	9.3.3

Source:	InterDigital, Inc.

Title:	Functionality management for UE sided model for RRM measurement event prediction

Document for:	Discussion

# 1. Introduction

The rel-19 study on AIML mobility was concluded in RAN2-131 and the agreements are captured in TR 38.744 [1]. Based on that study, a work item has been approved for rel-20 to specify the RRM measurement and measurement event prediction use cases, where the main RAN2 objectives of the work item for a UE sided model are [2]:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level prediction is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we discuss LCM and functionality management aspects for a UE-side model for RRM measurement event prediction. The focus here will be indirect measurement event prediction as the support of direct measurement event prediction is still not agreed and is expected to be decided after feasibility of core requirements has been studied in RAN4 [2].

# 2. Discussion

## 2.1 Adopting Rel-19 agreements in AIML PHY

Most of the LCM and related signalling discussions and agreements for AIML mobility during the study item phase in rel-19 used the AIML BM use case as a baseline (e.g., capability reporting, applicability determination and reporting, etc.,), as captured in the TR [1]. Additionally, the specification of the rel-19 work on AIML BM has also been finalized, with a few ASN.1 corrections left to be made (e.g., 38.300 corrections [3]). Considering that, we don’t think it is necessary to re-open the discussion for all the individual stage 2 aspects of AIML mobility.

Observation 1: The LCM and related signalling discussions and agreements for the AIML mobility study in rel-19 study were based on the AIML BM use case.

Observation 2: The rel-19 specification work on AIML BM use case has been finalized (with few ASN.1 corrections pending).

Observation 3: RAN2 should not discuss and re-confirm each agreement that was made during the AIML mobility study during rel-19.

Thus, the LCM and related signalling discussions in RAN2 should focus on identifying the aspects that can be directly adopted from the BM specifications and those that need further discussion. Thus, we propose:

Proposal 1: Rel-20 discussion on LCM and signalling for AIML mobility should start by identifying the aspects of rel-19 AIML BM use case that can be adopted as is and those that require further discussion.

The applicability determination and reporting for AIML BM case have been specified in rel-19. We have listed below the stage-2 description from the latest rel-19 38.300 [3], where we have marked the items that can be directly adopted for the AIML mobility use case in green and the ones that need discussion/update or are not applicable for the AIML mobility use case in red (i.e. aspects that are specific to AIML BM such as CSI configuration, CSI reporting activation/deactivation, etc.):

22.2.3	 Applicability Reporting

For UE-side model, the network provides inference configuration or inference-related parameters based on UE supported functionalities. The UE reports its applicable functionalities, inapplicable functionalities with its preference to release the configuration and subsequent changes of applicability status of functionalities to network. The basic procedure of applicability reporting is described as in Figure 22.2.3-1.

Figure 22.2.3-1: Initial applicability and applicability status change reporting [3]

1.	The network may inquire about the UE capability information.

2.	The UE may indicate its supported AI/ML functionalities to the network via UECapabilityInformation message.

3.	The network may provide inference configuration with Network-side additional conditions (i.e., associated ID) to UE via CSI report configuration or inference related parameters configuration.

4.	The UE determines the applicable AI/ML functionalities based on Network-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

NOTE 1:	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

5.	The UE reports its functionality applicability in RRCReconfigurationComplete message.

6.	When the inference configuration consists of periodic CSI report configuration, upon reporting the applicable functionalities, the UE autonomously activates the applicable AI/ML functionalities. When the inference configuration consists of semi-persistent CSI reporting and/or aperiodic CSI report configuration, upon reporting the applicable AI/ML functionalities, applicable AI/ML functionality can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

7.	When applicability of the functionality changes, the UE can report updated functionality applicability status in UEAssistanceInformation message. When an activated AI/ML functionality becomes inapplicable, the UE does not autonomously deactivate it, but the UE indicates to the network the change in the applicability. Upon reception of UE indication of the functionality becoming inapplicable, the network should deactivate or release this activated functionality.

NOTE 2:	Upon receiving one or more inference configuration(s), UE maintains all the inference configuration(s) no matter the inference configuration is applicable or inapplicable until the network releases it.

During handover, the UE may receive the inference configuration related to the target gNB via handover command. Then the UE reports its applicable/inapplicable AI/ML functionalities to the target gNB after the handover completion.

NOTE 3:	UAI can be sent from the source gNB to the target gNB to exchange applicability reporting referring to the configurations from the source gNB. The source gNB can send to the target gNB applicability information referring to the configurations from the source gNB, via handover preparation information message.

Proposal 2: For indirect RRM measurement event prediction, RAN2 to adopt the applicability determination and reporting framework for AIML BM that is captured in 38.300 (section 22.2.3), except for the aspects related to CSI configuration/reporting.

In the following sub sections, we discuss in detail the aspects that are particular to indirect RRM measurement event prediction that are different from AIML BM case.

## 2.2 UE capability

In indirect RRM measurement event prediction, a UE-sided AIML model first generates RRM measurement predictions (e.g., RSRP predictions) and the event prediction is generated by further post-processing of these results, without further involvement of the AIML model. Thus, a UE that capable of performing RRM measurements is implicitly also capable of indirect measurement event prediction.

Proposal 3: A UE that is capable of RRM measurement prediction is also capable of indirect RRM measurement event prediction.

## 2.3 Inference configuration/reporting

It is obvious that the inference and reporting configuration for the AIML mobility use case will be different from the AIML BM use case. There were various agreements captured on Sec. 6.1.2.1.2 in [1] regarding the inference configuration/reporting for RRM measurement prediction:

Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly.

Following inference parameters can be configured to UE for inference and assessing applicability:

-	PW length for intra-frequency temporal domain case A;

-	Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction;

-	Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement);

-	Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available);

-	Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction.

-	Optional associated ID.

It is up to UE’s implementation to decide on model-related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length.

NOTE 1:	MRRT or MRRS can be considered if it is required for defining performance requirement in RAN4

NOTE 2:	The detail (e.g. which use case(s)) of associated ID will be decided during WI phase

UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message:

-	For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported;

-	For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported;

-	For inter-frequency prediction, the latest predicted measurement result is reported.

In addition, the following information is provided in Sec. 6.1.3 [1], specifically for the case of RRM measurement event prediction:

On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction.

UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact.

Most of the agreements for RRM measurement prediction are also applicable for the case of indirect RRM measurement event prediction, as discussed above, this case depends on post-processing RRM measurement predictions.

Observation 4: Except for aspects like spatial prediction that are not within the scope of the Rel-20 WI, most of the agreements that are captured in TR 38.744 for inference configuration/reporting of RRM measurement prediction can be already confirmed in RAN2.

Observation 5: By extension, any such agreements are also applicable to the case of indirect RRM measurement event prediction.

Thus, we propose the following:

Proposal 4: RAN2 to confirm that the RRM measurement configuration and reporting framework based on the RRC layer is the baseline for inference configuration and reporting for indirect RRM measurement event prediction.

Proposal 5: RAN2 to confirm that the UE can be configured with the following parameters for the case of indirect RRM measurement event prediction:

- Prediction window length

- Event-related parameters (e.g., event type and other event-related parameters such as TTT, etc.)

- (optional) list of cells for which network expects inference report and their corresponding associated ID

Proposal 6: RAN2 to confirm that for indirect measurement event predictions, the UE can be configured with event triggered conditions and measurement reporting upon event prediction based on predicted measurements or a combination of actual and predicted measurements.

Proposal 7: RAN2 to confirm that for the case of indirect RRM measurement event prediction, the information in the measurement report sent upon event prediction includes:

- RRM measurement results (actual and/or predicted) based on the respective RRM measurement prediction sub use case:

  - For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in the PW

  - For intra-frequency temporal domain case B, the latest actual or predicted measurement result(s)

  - For inter-frequency prediction, the latest predicted measurement result.

- Time-related information about the predicted event (e.g., the expected time of occurrence)

## 2.4 Network side additional condition (associated IDs)

In the AIML BM case, the associated ID corresponds to the serving cell where the beams are being predicted. On the other hand, in the case of AIML mobility, the UE may be configured to perform simultaneous predictions for multiple cells. For example, a UE may perform temporal prediction for the serving cell and multiple neighbour cells to be used for indirect RRM measurement event prediction. Likewise, a UE may use RRM measurements from (at least) two cells at the input of its AIML model to evaluate whether an event (e.g., A3) will take place. In such cases, the UE may need to be provided with associated IDs for multiple cells, but the UE’s AIML model may have been trained only for a subset of these associated IDs.

Observation 6: There may be cases in indirect RRM measurement event prediction where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Like in the AIML BM use case, the UE can determine the applicability of the indirect RRM measurement event prediction functionality by considering model availability, NW-side additional conditions (i.e., the provided associated IDs), UE-side additional conditions (internally known by UE) and the provided inference configuration. However, now the provided associated IDs may not all be fully compatible with the training of the UE’s model. This may in turn influence the applicability determination for the configured AIML functionality.

Proposal 8: For indirect RRM measurement event prediction, the AIML configuration may be fully applicable, fully non-applicable, or partially applicable.

## 2.5 Performing inference

In the case of AIML BM, periodic CSI reporting is activated upon configuration, while semi-persistent reporting needs to be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. However, for performing RRM measurements, the activation is always upon the reception of the respective configuration.

One approach for the case of indirect RRM measurement event prediction is to apply legacy behaviour. That is, the UE will start performing the indirect RRM measurement event predictions if it has an AIML model that is applicable for the provided RRM measurement prediction configuration (i.e., UE can do the measurement prediction that can be post processed to predicted measurements). A disadvantage of such an approach is that the UE may perform unnecessary inferences/predictions even when it is not needed (e.g., when the UE is near the serving cell center). Even in legacy RRM framework, there are optional configurations such as s-measure that prevent the UE from performing unnecessary measurements, and UE will not evaluate events if it doesn’t have the required measurements for the event condition determination. Thus, it should be discussed whether such mechanisms are needed for controlling event inferences as well.

Observation 7: Performing indirect RRM measurement event predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

As such, we propose the following:

Proposal 9: The UE can be configured to start performing RRM measurement event predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfilment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

# 3. Conclusion

In this contribution, we discussed various LCM and functionality management aspects for indirect RRM measurement event prediction. The following observations and proposals were made:

[Adopting rel-19 agreements in AIML PHY]

Observation 1: The LCM and related signalling discussions and agreements for the AIML mobility study in rel-19 study were based on the AIML BM use case.

Observation 2: The rel-19 specification work on AIML BM use case has been finalized (with few ASN.1 corrections pending).

Observation 3: RAN2 should not discuss and re-confirm each agreement that was made during the AIML mobility study during rel-19.

Proposal 1: Rel-20 discussion on LCM and signalling for AIML mobility should start by identifying the aspects of rel-19 AIML BM use case that can be adopted as is and those that require further discussion.

Proposal 2: For indirect RRM measurement event prediction, RAN2 to adopt the applicability determination and reporting framework for AIML BM that is captured in 38.300 (section 22.2.3), except for the aspects related to CSI configuration/reporting.

[UE Capability]

Proposal 3: A UE that is capable of RRM measurement prediction is also capable of indirect RRM measurement event prediction.

[Inference configuration/reporting]

Observation 4: Except for aspects like spatial prediction that are not within the scope of the Rel-20 WI, most of the agreements that are captured in TR 38.744 for inference configuration/reporting of RRM measurement prediction can be already confirmed in RAN2.

Observation 5: By extension, any such agreements are also applicable to the case of indirect RRM measurement event prediction.

Proposal 4: RAN2 to confirm that the RRM measurement configuration and reporting framework based on the RRC layer is the baseline for inference configuration and reporting for indirect RRM measurement event prediction.

Proposal 5: RAN2 to confirm that the UE can be configured with the following parameters for the case of indirect RRM measurement event prediction:

- Prediction window length

- Event-related parameters (e.g., event type and other event-related parameters such as TTT, etc.)

- (optional) list of cells for which network expects inference report and their corresponding associated ID

Proposal 6: RAN2 to confirm that for indirect measurement event predictions, the UE can be configured with event triggered conditions and measurement reporting upon event prediction based on predicted measurements or a combination of actual and predicted measurements.

Proposal 7: RAN2 to confirm that for the case of indirect RRM measurement event prediction, the information in the measurement report sent upon event prediction includes:

- RRM measurement results (actual and/or predicted) based on the respective RRM measurement prediction sub use case:

  - For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in the PW

  - For intra-frequency temporal domain case B, the latest actual or predicted measurement result(s)

  - For inter-frequency prediction, the latest predicted measurement result.

- Time-related information about the predicted event (e.g., the expected time of occurrence)

[Network side additional condition (associated IDs)]

Observation 6: There may be cases in indirect RRM measurement event prediction where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Proposal 8: For indirect RRM measurement event prediction, the AIML configuration may be fully applicable, fully non-applicable, or partially applicable.

[Performing Inference]

Observation 7: Performing indirect RRM measurement event predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

Proposal 9: The UE can be configured to start performing RRM measurement event predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfilment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

# 4. References

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19), October 2025

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO/Interdigital, RAN#109, Sept 2025

- R2-2508466, Correction on AI for NR Air interface feature in TS38.300, Vivo (Rapporteur), RAN2-132, Dallas, November 2025