3GPP TSG RAN WG2 Meeting #132                          	   R2-2508753

Dallas, USA, 17th – 21st November 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

At RAN2 #131bis, the following design issue was discussed but couldn’t draw any conclusion [1].

Proposal 5 [H130] is not pursued. [H130] is Rejected - RRC rapporteur suggests to continue discussion on whether there is a need for clarification in the meeting minutes and if there is a need to activate adaptation via RRC configuration. - HW understands that if this RIL is rejected we also assume that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells Can come back in the next meeting to check whether we can confirm the understanding that a UE can receive DCI2-9 from Pcell for SSB adaptation status for deactivated Scells

In this contribution, we aim to further examine the unresolved issue raised during RAN2 #131bis regarding the support of SSB adaptation for deactivated SCells.

# Discussion

From our perspective, the key question is whether SSB adaptation should be supported when the SCell is in a deactivated state. This scenario closely resembles OD-SSB case 2, where both AO-SSB and OD-SSB share the same frequency allocation. Notably, the specification already supports OD-SSB indication for deactivated SCells, which suggests that the necessary signaling mechanisms are in place. Given this existing support, introducing additional design elements specifically for SSB adaptation in the context of deactivated SCells may not offer significant benefits. Furthermore, considering the current stage of the Release 19 timeline, where most design aspects are already well-defined, it would be difficult to justify further modifications without clear and compelling technical advantages.

Observation 1. Given the support for OD-SSB indication with deactivated SCells, introducing additional design for SSB adaptation in this context appears unnecessary.

SSB adaptation is signaled via DCI format 2-9, whereas SCell activation is conveyed through MAC CE. This differs from the OD-SSB scenario for deactivated SCells, where both the OD-SSB indication and SCell activation are signaled via MAC CEs. Coordinating signaling between DCI and MAC CE introduces additional complexity in terms of timing alignment, compared to managing signaling solely between MAC CEs. This increased complexity could result in additional workload for RAN4, particularly in ensuring consistent and efficient timing relationship across layers.

Observation 2. Supporting SSB adaptation for deactivated SCells adds evaluation complexity due to timing alignment between DCI and MAC CE, potentially causing more RAN4 work which is undesired at this late CR stage of Release 19.

Considering the above observations, we suggest the following proposal.

Proposal 2. Do not support SSB adaptation for deactivated SCell.

# Conclusion

In this contribution, we discussed whether SSB adaptation should be supported for deactivated SCell and concluded with the following observations and corresponding proposal.

Observation 1. Given the support for OD-SSB indication with deactivated SCells, introducing additional design for SSB adaptation in this context appears unnecessary.

Observation 2. Supporting SSB adaptation for deactivated SCells adds evaluation complexity due to timing alignment between DCI and MAC CE, potentially causing more RAN4 work which is undesired at this late CR stage of Release 19.

Proposal 2. Do not support SSB adaptation for deactivated SCells.

# References

- “R2-131bis_NES_NR-NTN_IoT-NTN_(Sergio)_ EOM”, RAN2 #131 bis, Prague, Czech Republic, Oct. 13th-17th, 2025.