3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508803

Dallas, USA, November 17th-21st, 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Multiple Reference locations reporting

Document for:	Discussion and Decision

# Introduction

In RAN2#132 meeting, following agreements were made regarding SMTC enhancements.

1.	[S024]: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available

2.	[H252]: We add the “N closest…” in the procedural text

3.	[H250]: Use “closest reference location information” to replace “location information” in the UAI reporting procedures, and clarify that this this information is for “assisted SMTC and measurement gap configuration” (can come back to this after the discussion on inter-frequency support)

4.	[V208/X251] Confirm that bitmap is used to indicate the N closest reference location.

5.	[Q200]: Location-based SMTC enhancement is not supported in moving cell scenario (to be captured in Stage2). We specify that smtc5 with multiple offsets is not broadcast in a Earth-moving cell scenario (to be captured in the smtc5 field description in system information)

6.	[C006]: We change the limitation as “The total number of configurable SMTCs offsets across smtc4list and smtc5list is 6.”

7.	[C009]: RAN2 understands that if the NW wants to reuse the periodicity in smtc1, the NW needs to configure smtc4 (we capture something accordingly in the field description)

8. 	[V204/H250] We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized. We introduce a new (per-UE) capability with signalling for the support in RRC connected of 2 periodicities in the inter-frequency case

9.	[X250/V205] The introduction of a new distance threshold (that is associated with the Rel-19 reference locations) is not pursued

In this document, we discuss remaining issues.

# Discussion

In the last meeting it was agreed to extend the solution what we have for the intra-frequency measurements case will be extended to inter-frequency measurements scenario. Unfortunately, this understanding was not made clear enough in the agreement. It is misunderstood that the UE can report a separate list of 4 closes reference locations for each frequency requested by the network.

In our view, transmitting report of closest 4 reference locations is to inform the network about the Area where the UE is located or has just entered. The area or region or shape of area is determined by the set of 4 closest reference locations. Based on the proximity, then network can configure the UE with proper measurement configurations, this includes both intra-frequency and inter-frequency measurement configurations. This is supposed to be in larger granularity of the geographical area without the issue of user’s privacy concern.

Figure 1 Risk of UE location exposure with too many closest reference locations report

If the network needs more granular location information from UE, it should request it after obtaining user consent. But if the network already has user consent, it can request UE’s coarse location information or UE’s GNSS location information. Therefore, instead of reporting multiple sets of 4 closest reference locations, UE should report coarse UE location information as for both network needs to obtain user consent.

If network has no user consent, it requests only one set of 4 closest reference locations from UE regardless of inter or intra-frequency measurement scenarios.

If network has user consent or does not require user consent based on location regulation, the UE reports coarse location information instead of multiple sets of 4 closet reference locations (or instead of 4 closet reference locations per frequency layer). In this case, network can simply include coarse UE location request indication.

In our view, the original intention of limiting to 4 SMTCs is to limit the number of satellites a UE can measure simultaneously. However, the SIB19 was extended without taking into account this. Now with the beam hopping enhancements, it is not clear whether 4 SMTCs is for the given single satellite beams or more than 4 satellites’ beams. We think it should be clarified that total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

Clarify that total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

# Conclusion

Following proposals are made.

Proposal 1	If network has no user consent, it requests only one set of 4 closest reference locations from UE regardless of inter or intra-frequency measurement scenarios.

Proposal 2	If network has user consent or does not require user consent based on location regulation, the UE reports coarse location information instead of multiple sets of 4 closet reference locations (or instead of 4 closet reference locations per frequency layer). In this case, network can simply include coarse UE location request indication.

Proposal 3	Clarify that total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.