3GPP TSG-RAN WG2 Meeting #132	                                                                     R2-2508831

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.3.3.2

Title:	Discussion on 6G energy efficiency

Source:	CMCC

Document for:	Discussion1

# 1	Introduction

In RAN#108 meeting, a new SI of study on 6G Radio was approved [1], one of the objectives is to study single technology framework considering energy efficiency/energy saving for both network and device:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

And in RAN2#131bis meeting, RAN2 started the discussion on 6G, following is the agreement related to energy efficiency[2]:

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

This paper is to provide our observation and view on energy efficiency in 6G.

# 2	Discussion

2.1	Lessons from 5G UE power saving/network energy saving solutions

In 5G, multiple UE power saving or network energy saving solutions were specified, considering different RRC states and different deployment scenarios, as summarized in the following table:

Table 1

|  | UE power saving | Network energy saving |
| --- | --- | --- |
| Rel-15 | - C-DRX/I-DRX - UAI for power saving | - On-demand SI - Intra-band SSB-less SCell |
| Rel-16 | - DCP - SCell dormancy - RRM Measurement relaxation - UAI for power saving - MIMO layer adaptation |  |
| Rel-17 | - SCell/SCG (de)activation - PEI - RLM/BFD relaxation - PDCCH skipping - SSSG switching - E-DRX for RedCap - RRM relaxation for RedCap |  |
| Rel-18 | - LR/LP-WUS (SI) | - Cell DTX/DRX - Inter-band SSB-less Scell - Spatial/Power domain adaptation - Inter-node beam selection |
| Rel-19 | - LR/LP-WUS (WI) | - On-demand SSB for scell - On-demand SIB1 - Common signaling(SSB, RACH and paging) adaptation |

According to the table, it can be noticed that with the introduction of LR/LP-WUS, there comes some function duplication on UE power saving, as summarized in the following:

- LP-WUS for RRC_IDLE UE and PEI: R19 LP-WUS for RRC_IDLE UE can be transmitted before PEI or paging. It has the same function as PEI, that to sub-group UEs to avoid paging false alarm, and it can be used together with PEI;

- LP-WUS for RRC_CONNECTED UE (option 1-2-1) and DCP: LP-WUS for RRC_CONNECTED UE (option 1-2-1) has the same function as DCP, that it can be used to indicate to the UE whether to wake up for PDCCH monitoring in the next C-DRX on-duration, and it can save UE’s power compared to always wake-up for PDCCH monitoring based on C-DRX pattern;

- LP-WUS for RRC_CONNECTED UE (option 1-2-2) and C-DRX: option 1-2-2 is used to indicate UE that there will be scheduling, and UE will start an active timer (not C-DRX on-duration) based on LP-WUS indication. What’s more, LP-WUS can be delivered in C-DRX on or off duration, in other words, it is not associated with C-DRX. Compared to C-DRX, it is more flexible and beneficial to service latency;

- Measurement relaxation or offloading to LR and measurement relaxation: For a UE supporting LR, the measurement  can be relaxed or offloaded. For measurement relaxation with LR, both measurement on serving cell and neighbor cell are relaxed, and LR is used by the UE for serving cell measurement. For measurement offloading, the measurement is offloaded to LR, in which UE only measures serving cell on LR and there’s no measurement on neighbor cell. Besides, 5G has specified RRM relaxation on neighbor cells based on MR.

Observation 1: LP-WUS/LR can achieve the same or similar function with PEI, DCP, C-DRX and measurement relaxation.

Two kinds of LP-WUS sequences were studied in 5G, including OOK-based sequence and OFDM-based sequence, which has implementation complexity and performance differences. Though LR/LP-WUS can achieve the same UE power saving purpose with lower power consumption compared with MR related solution, it has drawbacks like limited coverage, low data transmission capability and not supporting neighbor cell measurement, especially OOK based LP-WUS.

RAN1 has started the discussion on OFDM-based DL-WUS for 6G, following are RAN1 agreements related to DL-WUS[3]:

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities Agreement For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects: - Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s) - Transition energy and time for each of sleep state(s) - Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

It can be seen from RAN1’s agreements that RAN1 is to study solutions to overcome the shortcomings of LP-WUS identified in 5G. Considering that whether to support DL-WUS( LR/LP-WUS) mainly depends on RAN1’s evaluation, therefore, we propose that RAN2 first focus on the study of the energy efficiency solution irrespective with LR.

Proposal 1: RAN2 first focus on the study of the energy efficiency solution irrespective with LR/LP-WUS(DL-WUS).

2.2	Network energy saving mechanism

In 5G, one cell associated with one carrier (normal cell) is the typical deployment scenario, while in 6G, one cell associated with multi carrier (hyper cell) is proposed to utilize the fragmented spectrums[4]. From our perspective, hyper cell deployment is helpful to both network and UE’s power saving due to the following reasons: 1) For network, it can only broadcast common signalling including SSB, system information, etc., on the anchor carrier, but not all carriers associated with the cell to achieve energy saving; 2) From UE side, it may only perform measurement on anchor cell in RRC_IDLE state, but not all carriers to achieve UE’s power saving. Hence, hyper cell can be seen as a network energy saving solution.

Proposal 2: Hyper cell (one cell associated with multi-carrier) deployment can considered for network and UE’s energy saving.

What’s more, though the NES mechanisms aims to network energy saving, these can also reduce UE’s power consumption which is brought by the network’s changes. What’s more, based on the 5G discussion, and network energy saving mechanisms may bring with different UE’s behaviour when applying UE power saving mechanisms. And based on companies’ majority view, 6G should aim to develop solutions that can achieve joint energy saving in both network and UE side from day1. Therefore, we think network energy saving mechanism should be discussed at the beginning in 6G.

Observation 2: Apply NES solution can also bring UE power saving gain, and UE’s behavior with/without NES mechanism applied may have difference.

We think the following network energy saving mechanisms can be considered in 6G for both normal cell and hyper cell deployment:

- On-demand common signalling, including SSB, SIB1 and other SI:  From network side, with common signalling transmitted by on-demand way, the network can avoid or sparse the periodical transmission of common siganlling, then the network energy is saved. From UE side, this way can reduce UE’s unnecessary monitoring on common signalling, therefore, it is also helpful to UE power saving.

- Common signalling adaptation: In Rel-19, common signalling adaptation is supported for network energy saving, similar to on-demand common signalling transmission, the periodicity of common signalling may be enlarged, and the common signalling transmission is reduced. What’s more, this  mechanism can work with on-demand common signalling transmission.

- Cell DTX/DRX: In 5G, Cell DTX/DRX is a network energy saving mechanism to facilitate reducing gNB downlink transmission/uplink reception active time and it is only applied to RRC_CONNECTED UE. Based on Cell DTX/DRX pattern, RRC_CONNECTED UE can identify network’s active duration, during which the UE waits for to receive PDCCHs or SPS occasions, and transmit SR or CG. Combined with UE’s C-DRX, which is a commercial deployed UE power saving mechanism, the UE can minimize the PDCCH monitoring duration by identify the overlapping of cell DTX/DRX activation duration and UE’s C-DRX active time. Therefore, cell DTX/DRX can achieve joint network and UE energy saving. However, as mentioned, that in 5G, it is only applied to RRC_CONNECTED UEs, that’s to say, it has no impact to common signalling transmission, in 6G, we can consider the extension of Cell DTX/DRX to common signalling transmission, since the most energy consumption of the network comes from periodicity transmission.

Proposal 3:  For 6G network energy saving, the following solutions can be considered for both normal cell and hyper cell deployment:

- On-demand common signalling, including SSB, SIB1 and other SI

- Common signalling adaptation

- Cell DTX/DRX applied to any RRC states

2.3	UE power saving mechanism

When looking back to 5G energy saving discussion history, most discussion is around how to reduce UE’s power consumption in different RRC state. In the following, we discuss UE power saving solutions in different RRC states.

For RRC_CONNECTED UE, the main power consumption is for PDCCH monitoring, therefore, multiple solutions were specified to reduce PDCCH monitoring in different 5G releases, like C-DRX, DCP, SSSG and PDCCH skipping. Generally, these solutions can be supported in 6G for UE’s power saving when there’s data transmission. And from operate perspective, C-DRX that has been commercialized in our real network has good performance, can be the baseline for UE power saving.

Proposal 4: C-DRX can be considered as the baseline mechanism to reduce UE’s PDCCH monitoring in RRC_CONNECTED state. Also, DCP, SSSG and PDCCH skipping can work with C-DRX for further power saving.

What’s more, we should notice that there’s a case that RRC_CONNECTED UE has no traffic but remains the connection, like C-DRX non-active period, and it’s not suitable to release UE to RRC_IDLE state. Currently, there’s no extra definition on UE’s behavior for this case, and UE may act as the same as having data transmission, which is not energy efficiency. Though RRC_INACTIVE is supported in current 5G spec, and it be a solution for this case, RRC_INACTIVE state has its own shortcomings and it is not commercial deployed in 5G. Therefore, except for RRC_INACTIVE state, we can also consider the sub-state of RRC_CONNECTED for UE’s power saving for the case without data transmission, details can be found in our contribution[5].

Proposal 5: RRC_INACTIVE state or a new sub-state for UE power saving can be considered for the case of no-data transmission.

For RRC_IDLE UE, the main power consumption is for commons signalling monitoring and measurement, therefore, we need to consider how to reduce common signalling monitoring or measurement. For common signalling monitoring reducing, the joint energy saving mechanisms proposed in Proposal 3 can be used, and some wake up mechanisms for RRC_IDLE UEs such as PEI can be introduced. As to measurement reduction, 5G measurement relaxation mechanism can be considered as baseline in 6G.

Proposal 6: For RRC_IDLE UE power saving, except for the mechanisms in Proposal 3, wake up mechanisms like PEI can be considered.

Proposal 7: 5G measurement relaxation mechanism can be considered as baseline in 6G.

# 3	Conclusion

In this contribution, the following observations and proposals are provided.

Observation 1: LP-WUS/LR can achieve the same or similar function with PEI, DCP, C-DRX and measurement relaxation.

Proposal 1: RAN2 first focus on the study of the energy efficiency solution irrespective with LR/LP-WUS(DL-WUS).

Proposal 2: Hyper cell (one cell associated with multi-carrier) deployment can considered for network and UE’s energy saving.

Observation 2: Apply NES solution can also bring UE power saving gain, and UE’s behavior with/without NES mechanism applied may have difference.

Proposal 3:  For 6G network energy saving, the following solutions can be considered for both normal cell and hyper cell deployment:

- On-demand common signalling, including SSB, SIB1 and other SI

- Common signalling adaptation

- Cell DTX/DRX applied to any RRC states

Proposal 4: C-DRX can be considered as the baseline mechanism to reduce UE’s PDCCH monitoring in RRC_CONNECTED state. Also, DCP, SSSG and PDCCH skipping can work with C-DRX for further power saving.

Proposal 5: RRC_INACTIVE state or a new sub-state for UE power saving can be considered for the case of no-data transmission.

Proposal 6: For RRC_IDLE UE power saving, except for the mechanisms in Proposal 3, wake up mechanisms like PEI can be considered.

Proposal 7: 5G measurement relaxation mechanism can be considered as baseline in 6G.

# 4	References

[1] RP-251881, New SID: Study on 6G Radio;

[2] R2_131bis_Chairnotes;

[3] Chairman notes RAN1#122bis;

[4] R2-2508835, Discussion on 6G initial access and spectrum aggregation, CMCC;

[5] R2-2508845, Discussion on 6G RRC states, CMCC.