3GPP TSG-RAN2 Meeting #132			R2-2508847

Dallas, USA, Nov. 17th-21st

Agenda item:	10.3.1.2

Title:	Considerations on RAN-level QoS/QoE framework and Service-awareness RAN

Source:	CMCC

Document for:	Discussion

Introduction

RAN#109 approved the revised 6G SID [1], where one of the primary responsibilities of RAN2 is the study of 6G user plane architecture and protocol design.

In the last meeting [2], RAN2 has conducted preliminary discussions on 6GR UP aspects and reached some consensus. This contribution will further discuss 6GR RAN-level QoS/QoE framework and Service-awareness RAN.

Discussion

  - Characterization of traffic types for services

    - XR and multi-modal service

Based on the former study, XR services generate high and variable bit-rate traffic, typically with large downlink data for visual content and smaller uplink flows for control and sensor feedback. The traffic is often quasi-periodic and bursty, following rendering frame rates ranging from 30 to 120, which produce data bursts every few milliseconds. XR traffic is highly sensitive to latency, jitter, and packet loss, as transmission variations can immediately affect the immersive experience. It also shows multi-modal and asymmetric characteristics, combining multiple synchronized streams, e.g., video, audio, and control data, with generally higher downlink than uplink demand. Overall, XR traffic exhibits fluctuating data rates, short packet intervals, and strict timing requirements, distinguishing it from conventional mobile broadband services.

Particularly, XR and multi-modal interactive services involve simultaneous transmission of multiple types of modalities, including visual, auditory, haptic, and environmental sensing data. From the perspective of 6GR, these services impose stringent requirements on resource allocation, latency, synchronization, and reliability.

These services require multiple modalities to be transmitted in parallel and nearly synchronously, ensuring consistent perception across all data types. Haptic and force-feedback data typically demand very low latency and high update frequency, while visual and auditory modalities require high bandwidth with moderate latency constraints. Both downlink and uplink flows are critical: downlink often carries high-volume data such as video and multi-modal content, whereas uplink carries high-frequency small packets for control, feedback, or sensor information.

The interaction nature and multi-user scenarios introduce highly dynamic traffic patterns. Rapid changes in user actions, scene updates, or modality switching can create short-term traffic peaks and demand dynamic resource adaptation. Furthermore, multiple parallel streams per user or per session require coordination to maintain timing and reduce inter-stream jitter. High reliability is also essential, as packet loss or delays can significantly impact user perception and interaction quality.

Based on these characteristics, the key RAN-side features of XR and multi-modal services can be summarized as follows:

- Inter-PDU dependency: consecutive PDUs in certain modalities are causally or temporally dependent, requiring correct reception and ordering to ensure seamless rendering and avoid perceptual artifacts.

- High-frequency updates and low latency: certain modalities require extremely short packet intervals and low end-to-end delay, while other modalities still require controlled latency.

- Parallel homogeneous streams:  Multi-stream single-modality data, e.g., visual, auditory, haptic, and environmental data from several source are transmitted simultaneously and need to be associated with the same session.

- Heterogeneous bandwidth and latency requirements: some modalities, e.g., visual, need high bandwidth, whereas others, e.g., haptic, require lower bandwidth but higher update frequency.

- Strong synchronization requirement: differences in transmission timing and jitter among modalities must be tightly controlled.

With regarding to TR 38.838, some characteristics and metrics for XR and multi-modal service are summarized as follows.

Table 2.1.1-1: Characteristic for XR and multi-modal service

| Traffic | Characteristic | Dependency | Packet Rate | Packet Arrival Interval | Packet Latency Sensitivity |
| --- | --- | --- | --- | --- | --- |
| High-rate data flow | Streamed Service with continuous or bursty high-rate data for visual or multi-modal media | Independent | 100Mbps-10GBps | 1-10 ms | Moderate |
| High-frequency control flow | Frequent small packets for control, pose, sensing, or haptic feedback. | Control/pose flow impacts visual data | <100Mbps | 1-5 ms | Stringent |
| Synchronized Multi-modal flows | Multiple heterogeneous modalities (e.g., audio, video, haptic) requiring temporal synchronization. | Flows with the same MMSID are dependent | 10Mbps-10GBps | 1-10 ms | Stringent |
| Bursty flow | Temporarily increased data rate caused by scene update, modality change, or multi-user interaction. Haptic data | Usually impacts data in the opposite link | Small but variable | <10 ms, aperiodic | Stringent |

Observation 1: XR and multi-modal services in 6G exhibit highly dynamic, bursty, and asymmetric traffic with multiple parallel streams, short packet intervals, heterogeneous bandwidth and latency requirements, strict inter-stream synchronization, and high reliability needs.

Proposal 1: The QoS/QoE framework and QoS Flow mapping mechanisms for 6GR UP should support the follows:

- Differentiated QoS handling within multi-stream PDU Sessions for XR/token communication/multi-modal services, ensuring end-to-end low latency, high reliability, inter-stream synchronization, burstiness;

- Dynamic QoS configuration updating via RAN node.

    - IIoT and URLLC

The use cases on the real time digital twins and Industrial communication has been identified in 6G. In the manufacturing or construction, the multiple autonomous mobile robots (AMRs) work together using the Industrial communication and a digital twin. Both cases require the RAN located in manufacturing or construction to provide the time synchronization and highly reliable and low latency service. The TSN networking which provide the deterministic and ‌ultra-low latency networking‌ has been deployed and  integrated with the existing 5G network. To support the TSN in 6G could ensure ‌smooth migration and allow flexible deployment.

The TSG RAN has held initial discussion on requirement values of TRP (technical performance requirement) items and proposes the following table for consideration. In the below table, the UP latency for HRLLC is proposed as 1ms and the reliability is 99.999%.

Table 2.1.2-1 Proposed TPR and requirements for HRLLC

| Proposed TPR items | Proposed requirement values |
| --- | --- |
| User Plane Latency | IC: 4ms HRLLC: 1ms |
| Control Plane Latency | 20ms |
| Reliability | 1-10-5 with same IMT-2020 assumptions |

The TSG RAN in [2] has held initial discussion on requirement values of TRP (technical performance requirement) items and proposes the following table for consideration. In the below table, the UP latency for HRLLC is proposed as 1ms and the reliability is 99.999%.

Observation 2: The QoS requirement of IIOT services in 6G still needs to be satisfied on time synchronization, highly reliable and low latency.

    - Token communication

During the previous discussions on general aspects and user plane in RAN2, it was noted that token communication (TokCom)/mobile AI for AI/ML model applications (e.g., LLMs) could become a potential consideration in the study of 6G general and user plane design. These mobile AI applications not only create new traffic for mobile networks, but also bring unique transmission characteristics. Currently, mobile AI applications transmit data (including text, images, and videos) through the RAN in the transparent manner as traditional internet services. The RAN remains unaware of the AI semantic units—tokens—carried within these packets. However, as the fundamental units for AI model processing and information generation, tokens exhibit unique communication patterns that present new opportunities for optimization at the RAN.

Meanwhile, in RAN1#122bis, RAN1 also reached the initial agreements on the traffic model for 6GR AI/ML services during the previous meeting, covering parameters such as token size, token arrival rate, and token delay budget as following:

Agreement in RAN1#122bis For the study traffic model(s) for 6GR AI/ML services: - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870. Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services. Note: RAN1 is discussing the following options for the model: - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget. - Token is the minimum unit of data generated in the application layer. - How to associate Tokens to PHY layer packets. - How to reflect the variable importance of tokens. - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay. - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc. - Option-1c: reusing or extending the FTP-3/XR traffic model. - FFS other models/options need to be defined for other AI/ML services.

From the RAN2 perspective, several features of the TokCom service will also influence the design of the user plane. This section analyses how token communication features influence RAN2 user plane design.

Let's first clarify the concept of token communication. Token communication refers to a communication model that uses tokens as the basic unit for end-to-end data transmission. The typical service process is as follows. We noticed that tokens, as the basic unit of generative AI/ML information processing and generation, can be transformed from various modalities of data such as text, images, videos, etc. An example of the Service flow of TokCom in scenario of Human to agent conversation between NW side and UE side can be shown in Figure. X.

- At UE side, via tokenizer and Vocabulary/knowledge base (KB), the raw multimodality can be converted to several tokens, and the raw data can be in forms of video frames, audio, texts, etc.

- Then, RAN can transmit the tokens via over the air interface to the NW side and OTT server.

- After OTT server side processed/inferenced the tokens, it will generated response tokens and deliver the tokens back to the corresponding de-tokenizer at UE device

- After UE side obtains the response tokens, it can convert the tokens back to text, images, videos via the corresponding de-tokenizer at UE side.

Figure 2.1.3-1: Service flow of TokCom

In a specific AI service session with the same vocabulary, the size of the data unit (Token ID) formed after tokenization is fixed, which means that the traffic pattern of token communication depends primarily on the size of the original information and the design of the vocabulary.

Below are the typical model vocabulary and corresponding token ID binary length:

Table 2.1.3-1: Typical model vocabulary and token ID binary length

| Data | Tokenizer | Vocabulary Size | Bits/Token ID |
| --- | --- | --- | --- |
| Text | byte-level BPE[3] | 100,256 | 17 bits |
| Text | SentencePiece (BPE)[4] | 128,256 | 17 bits |
| Image/Video | dVAE[5] | 8192 | 13 bits |
| Image/Video | TiTok[6] | 4096 | 12 bits |

We can observe that the typical size of a Token is significantly smaller than the RAN PDU size. According to the standard model vocabulary, a Token ID typically occupies only 12-17 bits. A single PDU typically contains multiple tokens. When the RAN fails to recognize the token characteristic within a PDU, token communication becomes no more than another service flow for the air interface, and its unique transmission characteristics cannot be leveraged to achieve performance gains.

Observation 3: A single PDU typically contains multiple tokens. When the RAN fails to recognize the token characteristic within a PDU, token communication becomes no more than a other service flow for the air interface, and its unique transmission characteristics cannot be leveraged to achieve performance gains.

To enable RAN to support optimized Token communication, we must first study and make RAN aware of the following four characteristics of Token. These characteristics are the basis for the subsequent optimization mechanism design.

- Error tolerance:

Tokens are resilient to a range of transmission errors; i.e. certain percentage of successfully received tokens may be enough for the AI/ML processing to reach the required accuracy level as shown in Figure X. The post-processing capability of error tokens can also compensate for errors. This capability stems from AI/ML models' ability to incorporate contextual semantics during token processing to correct errors. This error recovery mechanism represents a novel form of AL-FEC.

Figure 2.1.3-2: Error Tolerance of Token

This feature enables RAN to implement differentiated HARQ and ARQ transmission mechanisms. For certain non-critical tokens, transmission reliability requirements can be relaxed, and retransmission can be discarded under specific conditions to reduce latency and signaling overhead.

- Token importance differentiation:

Different tokens carry varying importance. The contribution of tokens from different modalities (e.g., text versus image background) or those within the same modality (e.g., the arrival time requirement for the first token) to AI model performance and user experience (QoE) differs.

This requires the RAN to have awareness of differentiate PDUs carrying tokens of varying importance. As on the PDU Set concept introduced in Rel-18 XR, differentiated QoS metrics can be defined for PDU/PDU sets containing tokens of different importance. This enables optimization of discard metric and multiplexing. Additionally, the predictability of token importance helps the network more accurately predict BSRs, enabling more flexible MAC scheduling.

- Token traffic pattern:

Tokens may arrive in a streaming or burst manner, where each token can be represented as a bit sequence with length determined by the size of the vocabulary or codebook or knowledge base. Regarding the traffic characteristics analysis and KPIs of generative AI, SA1 has already presented relevant research findings in TR 22.870 as shown below.

Table 2.1.3-2 Required KPIs for GenAI applications in TR 22.870

| Traffic Type | Burst size | Max Allowed latency for a burst (note) | Service bit rate: user-experienced data rate | Delay (note) | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| Chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: max allowed latency for a burst: max latency for sending out the whole packets within a burst delay: transmission latency of a packet |  |  |  |  |  |

Figure 2.1.3-3: Traffic characteristic of typical GenAI applications in TR 22.870

Although the traffic analysis in TR 22.870 was not specifically focused on Token transmission over the air interface, as previously analyzed, the relatively fixed size of tokens suggests that their communication traffic patterns will mirror those of the underlying services.

Observation 4: Given the relatively fixed size of tokens, their traffic characteristics are expected to mirror those of the underlying service. The traffic and QoS in TR 22.870 may provides a foundation for RAN2 analysis.

Figure 2.1.3-4: Traffic characteristic of Streaming Manner GenAI applications

- Token context relevance:

In streaming, tokens are generated sequentially and transmitted in real time. If a token packet is lost, relying on traditional retransmission mechanisms would cause subsequent correctly received tokens to be unprocessed, forcing users to wait for retransmission and disrupting the smoothness of their experience.

This feature may require the RAN to implement a more intelligent packet discard mechanism. When retransmission cannot meet the latency budget, the RAN should determine whether to continue waiting for the lost packet or discard it and proceed with subsequent received tokens, allowing the application layer to recover using its contextual understanding.

Above all, the special characteristics of tokens generated by AI/ML models/used for AI/ML model processing including:

Table 2.1.3-3: Characteristic for TokCom

| Characteristics | Description | Impacts on User Plane |
| --- | --- | --- |
| Error tolerance | Tokens are resilient to a range of transmission errors; i.e. certain percentage of successfully received tokens may be enough for the AI/ML processing to reach the required accuracy level. The post-processing capability of error tokens can also compensate for errors | Differential retransmission |
| Token importance differentiation | There is differentiated importance for different tokens, i.e. the tokens for different modality data or different tokens for the same modality data may contribute differently to the AI/ML model processing and QoE of users. | Scheduling policy Discarding QoS Mapping |
| Token traffic pattern | Token traffic patterns can be categorized as either streaming or brust, with the tokens being fixed in size and extremely small. | Scheduling Policy Multiplexing |
| Token context relevance | Tokens are dependent on each other in sequence or context. Losing a Token may block the processing of subsequent Tokens. | Discarding Differential retransmission Submitting in order |

Proposal 2: The token communication has potential characteristics such as error tolerance, differential importance, special traffic pattern, and context relevance, which may involve technical optimization of RAN2 by awareness of these characteristics (e.g., Packet level Importance Indication and error tolerant indication).

  - Information for application/service-awareness

To enable the RAN to be aware of applications or services for corresponding optimization, QoE is introduced, which can provide MNO and application provider with direct insight into end-user experience, enabling detection of service issues, optimization of network and service management, and assurance of user-perceived quality. RAN2 should prioritize studying UP enhancement based on QoE and application/service-awareness.

Common characteristics may include whether the application or service runs in the foreground or background, whether it is currently active or idle, UL-DL correlation (i.e., whether uplink or downlink activity triggers corresponding traffic in the opposite direction), packet generation features (generation interval, packet size distribution, packet identifier), and time sensitivity. Where feasible, device status can also be considered, such as screen on/off, battery level, or Wi-Fi connectivity.

Service-specific characteristics are naturally application- or service-specific. Research on the unique characteristics is still limited, as this is a new concept for 3GPP. However, the objective QoE metrics specified by SA4 for DASH, MTSI, and VR services will serve as a good starting point.

Table 2.2-1: Common information for application/service-awareness

| Information | Characteristics Type | Description | Note |
| --- | --- | --- | --- |
| Interaction state | Common | Whether an application is actively interacting with the user, i.e., foreground or background |  |
| Process state | Common | Execution status in the operating system, e.g., active, suspended, idle, blocked |  |
| UL-DL correlation | Common | Whether uplink or downlink activity triggers corresponding traffic in the opposite direction. |  |
| Packet generation feature | Common | Parameter related to packet generation, e.g., generation interval, packet size distribution, packet identifier, latency requirements, etc. |  |
| Device related information | Common | Information related to device itself, e.g., screen ON/OFF, battery level, Wi-Fi connectivity. |  |

Table 2.2-2: Service-specificinformation for application/service-awareness

| Information | Characteristics Type | Description | Note |
| --- | --- | --- | --- |
| QoE metrics | Service-specific | Some objective QoE metrics, including RVQoE metrics specified by SA4 for MTSI and VR service, e.g., Buffer level and Playout delay for media start-up. |  |

Proposal 3: For service-awareness information for 6GR, RAN2 agrees that there are two types of information, i.e., the common information for all services, e.g., Interaction state (foreground/background), Packet generation feature, Process state (active, suspended, idle and blocked), etc., and service-specific information for a certain service or service set, e.g., objective QoE metric.

  - Application of application/service-awareness information

In 5G XR awareness, one of typical XR traffic is evaluated as GOP (Group of Picture) based model including I-frames and P-frames of video with different importance. The SA2 describes this model by defining the concept PDU set in TR 23.700, which is composed of one or more PDUs carrying the payload of one unit of information generated at the application level. For the GOP based model, one PDU set may be any I-frame or P-frame of video. Meanwhile SA2 provides relevant QoS requirements for PDU set: the semi-static information on PDU Set QoS parameters per QoS flow including PDU Set Error Rate (PSER), PDU Set Delay Budget (PSDB) and PDU Set Integrated Handling Indication (PSIHI) and dynamic information for DL including PDU Set information and identification, e.g. PDU Set Sequence Number or PDU Set Importance (PSI).

Base on the QoS profile/characteristic per PDU Set received from CN, the RAN could obtain some characteristics of the XR service. For example, the importance and delay budget of I-frame and P-frame of video could be recognized by the network based on the PSI and PSDB in PDU set QoS profile. In order to achieve differentiated PDU set handling, the RAN2 has introduced enhanced mechanisms on PDU discarding and delay-awareness scheduling. The PDCP layer discards packets considering both the discarding timer and its PSI in case of congestion for XR service and the MAC scheduling may prioritize the delay-awareness PDU transmission by selecting the additional LCH priority which reflects the delay information of packet or PDU set.

Observation 5: In 5G, based on the QoS profile/characteristic per PDU Set received from CN, the RAN could obtain some characteristics of the XR service. In order to achieve differentiated PDU set handling in XR, the RAN2 has introduced enhanced mechanisms on PDU discarding and delay-awareness scheduling.

In 6G, the immersive communication, AI token communication and mobile AI are recommended as critical services. How to treat these new services in RAN remains a challenge for us. The 5G QoS model is designed based on QoS Flows. A QoS Flow identifed by QoS Flow ID (QFI) is the finest granularity of QoS differentiation in the PDU Session. In RAN, it is allowed to categorize several QoS Flows belonging to the same PDU session with similar QoS profiles into the same DRB based on the network implementation. The gNB is responsible to select appropriate DRB/LCH as the packet forwarding treatment to guarantees the QoS requirement. The traditional QoS framework in 5G is assumed to be limited, since it merely processes the mapping rules in NAS and AS on QoS flow level, neither on the PDU set/data burst or packet level. For 6G, the discussion on how to define and use the QoS profile of a specific service e.g. PDU set/data burst/packet information in QoS mapping and LCH configuration in RAN could be revisited.

Some QoS architectures has been discussed in XR and it could be discussed further for the service-awareness. As similar to XR, the QoS framework needs to investigate that how to map the QoS flows or PDU Sets/data burst/packet onto DRB/LCH.Indeed the DRB mapping rules in RAN and Logical Channel Prioritization (LCP) procedure may be enhanced to satisfy the service-awareness QoS profiles. To provide differentiated QoS flow/PDU set/data burst/packet handling at RAN, the considering QoS architectures from 5G system as baseline are listed in below options:

- Option A: mapping one type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to one DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option B: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to its dedicated DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option C: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to different DRBs and LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

- Option D: mapping multiple type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to its dedicated DRB and LCH. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

Figure 2.3-1: Several options for PDU set mapping

Take the XR-awareness or AI-awareness service as an example to support the above alternative QoS architecture. In XR-awareness service, the PDU set 1 could be I-frame of video with higher importance compared to the PDU set 2, which is assumed as P-frame, or in AI-awareness service the PDU set1 or PDU set 2 could be tokens generated as intermediate result with different attention weight/importance. In these two cases, no matter one or multiple QFI flows assigned in CN, the gNB is expected to handle differentiated QoS processing with different DRBs/LCHs to execute the differentiate treatment on the I/P frames or tokens with different attention weight as shown in option A/B/C. For option D, it could be used in multi-modal service, the PDU set/data burst/packet with different QoS requirement could be map to the same DRB and execute the differentiate treatment in LCP, which is not allowed in the current QoS framework.

It should be noted that the differentiated service-awareness QoS handling in RAN is depended on the information on PDU set/data burst/packet provided by SA2. Currently SA2 investigates the topic on 6G QoS framework in parallel and may clarify more detailed assumption on the QoS architecture and handling. RAN2 needs to study the options for service-awareness QoS architecture with considering the SA2 decision on QoS framework enhancement

Proposal 4: RAN2 to study the options listed above for service-awareness QoS architecture with considering the SA2 decision on QoS framework enhancement:

- Option A: mapping one type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to one DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option B: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to its dedicated DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option C: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to different DRBs and LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

- Option D: mapping multiple type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to its dedicated DRB and LCH. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

Emerging 6G services, such as AI/ML, immersive communication and sensing, have the stringent requirements on QoS metrics (e.g., latency, packet loss rate) and the QoS requirements may be dynamically variable based on real-time states. The existing static QoS mechanism is inherently incapable of adapting to such dynamicity.

Take an AI/ML service as an example: a robot without local AI inference capabilities should transmit its real-time road data collected via its cameras and sensors, including information on the speeds of the preceding vehicles and pedestrians, and obstacle positions, to the network servers deployed with AI inference capabilities or computing power, where the data serves as input for AI inference. This enables the robot to obtain AI inference results from the network and determine its subsequent actions. In this process, the input data collected by the UE is first transmitted to the node and then forwarded to a suitable AI inference server; the inference output results, in turn, are transmitted back to the UE. UE needs to obtain the AI inference results within an extremely short time after sending the input data. This ensures the robot can promptly take appropriate actions, such as decelerating, stopping, or navigating around obstacles to avoid collisions. The fluctuations in transmission latency or AI inference latency will create a cumulative effect through the chain-like flow of data, ultimately compromising the validity and reliability of AI services.

Observation 6: Emerging 6G services have the stringent requirements on QoS metrics (e.g., latency, packet loss rate) and the QoS requirements may be dynamically variable based on real-time states. The existing static QoS mechanism is inherently incapable of adapting to such dynamicity.

Therefore, a dynamic QoS mechanism is needed to support in 6GR, where the RAN directly adjusts the QoS requirements of QoS flows/PDU sets based on real-time conditions (e.g., the latency fluctuations of AI inference in the above example) to ensure the timeliness and reliability of data transmission and processing.

Figure 2.3-2: An example of dynamic QoS configuration for temporary back-end overload

From our perspective, the RAN's dynamic adjustment of QoS requirements could include Delay Budget, Error Rate, and importance of QoS flows/PDU sets. The QoS flows/PDU sets configured with the new QoS profile will be accompanied by L2-related adjustments, such as remaining time thresholds, discard timers, retransmission mechanisms activation and deactivation and scheduling priority.

Observation 7: RAN's dynamic adjustment of QoS requirements could include Delay Budget, Error Rate, and importance of QoS flows/PDU sets. The QoS flows/PDU sets configured with the new QoS profile will be accompanied by L2-related adjustments, such as remaining time thresholds, discard timers, retransmission mechanisms activation and deactivation and scheduling priority.

Regarding the study of dynamic QoS, we propose to consider the following:

- Monitoring of key network indicators, e.g., real-time collection of core parameters including bandwidth utilization, latency, packet loss rate, and jitter;

- Identification of service types and requirements, e.g., service awareness;

- The design of the new processing mechanism, e.g., QoS adjustment trigger conditions, which aims to enhance the efficiency of dynamic QoS adjustment;

- Interaction between RAN and the core network to ensure consistent QoS policies;

Proposal 5: For the 6G new services, such as AI/ML, immersive communication and sensing, a dynamic QoS mechanism is needed to support, where the RAN directly adjusts the QoS requirements of QoS flows/PDU sets based on real-time conditions to ensure the timeliness and reliability of data transmission and processing. The following could be considered:

- Monitoring of key network indicators

- Identification of service types and requirements

- The design of the new processing flow

- Interaction between RAN and the core network

In NR, typical services e.g., MTSI, DASH, and VR have corresponding application-layer measurement and reporting mechanisms, i.e., QoE/QMC. QoE directly reflects user experience, but it is currently mainly used by operators or service providers for data analysis and optimization, rather than being directly utilized by the RAN.

Specifically, R17 introduced RAN-visible QoE and defined some metrics accordingly, e.g., buffer level and playout delay for media start-up as parameters visible to the RAN. Although QoE measurement reports do not include explicit UE identifiers, the gNB can implicitly determine which UE sent the RVQoE measurements, making it feasible to perform application/service-layer optimization based on UE-reported QoE data.

Figure 2.3-3: Example of RAN Visible QoE

Buffer level reflects the UE’s application-specific buffer status, allowing the gNB to adjust radio resource allocation, e.g., lowering the RB priority for applications with high buffer levels or vice versa. Playout delay for media start-up reflects the network’s responsiveness and scheduling capability. If the parameter indicates that QoE metric cannot be met, scheduling and resource allocation algorithms can be adjusted to improve user experience.

Although these RVQoE parameters were introduced, how the RAN should adjust radio resource strategies based on QoE, e.g., scheduling or discarding, has not been specified and is largely left to gNB implementation. Moreover, the UE AS layer currently has no capability to adjust based on RV-QoE.

Therefore, 6GR should consider applying relevant RVQoE metrics to AS layer functions. Based on our observations, leveraging these QoE metrics in at least scheduling, resource allocation, and discarding would be beneficial.

Proposal 6: As a starting point, RAN2 agrees that at least some metric from RVQoE, i.e., the buffer level and playout delay for media start-up can be regarded as service-awareness information and utilized for RAN operations, e.g., data differentiated handling and discarding.

Conclusion

Based on above, from the perspective of CMCC, the following is observed and proposed:

Observation 1: XR and multi-modal services in 6G exhibit highly dynamic, bursty, and asymmetric traffic with multiple parallel streams, short packet intervals, heterogeneous bandwidth and latency requirements, strict inter-stream synchronization, and high reliability needs.

Proposal 1: The QoS/QoE framework and QoS Flow mapping mechanisms for 6GR UP should support the follows:

- Differentiated QoS handling within multi-stream PDU Sessions for XR/token communication/multi-modal services, ensuring end-to-end low latency, high reliability, inter-stream synchronization, burstiness;

- Dynamic QoS configuration updating via RAN node.

Observation 2: The QoS requirement of IIOT services in 6G still needs to be satisfied on time synchronization, highly reliable and low latency.

Observation 3: A single PDU typically contains multiple tokens. When the RAN fails to recognize the token characteristic within a PDU, token communication becomes no more than a other service flow for the air interface, and its unique transmission characteristics cannot be leveraged to achieve performance gains.

Observation 4: Given the relatively fixed size of tokens, their traffic characteristics are expected to mirror those of the underlying service. The traffic and QoS in TR 22.870 may provides a foundation for RAN2 analysis.

Proposal 2: The token communication has potential characteristics such as error tolerance, differential importance, special traffic pattern, and context relevance, which may involve technical optimization of RAN2 by awareness of these characteristics (e.g., Packet level Importance Indication and error tolerant indication).

Proposal 3: For service-awareness information for 6GR, RAN2 agrees that there are two types of information, i.e., the common information for all services, e.g., Interaction state (foreground/background), Packet generation feature, Process state (active, suspended, idle and blocked), etc., and service-specific information for a certain service or service set, e.g., objective QoE metric.

Observation 5: In 5G, based on the QoS profile/characteristic per PDU Set received from CN, the RAN could obtain some characteristics of the XR service. In order to achieve differentiated PDU set handling in XR, the RAN2 has introduced enhanced mechanisms on PDU discarding and delay-awareness scheduling.

Proposal 4: RAN2 to study the options listed above for service-awareness QoS architecture with considering the SA2 decision on QoS framework enhancement:

- Option A: mapping one type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to one DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option B: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to its dedicated DRB and different LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB split or LCP selection procedure.

- Option C: mapping multiple type of PDU sets/data burst to one QoS flow and then RAN maps this QoS flow to different DRBs and LCHs with differentiated QoS handling. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

- Option D: mapping multiple type of PDU sets/data burst to its dedicated QoS flow and then RAN multiplexes QoS flows to its dedicated DRB and LCH. To support this architecture, the RAN should perform enhancement on DRB mapping rules.

Observation 6: Emerging 6G services have the stringent requirements on QoS metrics (e.g., latency, packet loss rate) and the QoS requirements may be dynamically variable based on real-time states. The existing static QoS mechanism is inherently incapable of adapting to such dynamicity.

Observation 7: RAN's dynamic adjustment of QoS requirements could include Delay Budget, Error Rate, and importance of QoS flows/PDU sets. The QoS flows/PDU sets configured with the new QoS profile will be accompanied by L2-related adjustments, such as remaining time thresholds, discard timers, retransmission mechanisms activation and deactivation and scheduling priority.

Proposal 5: For the 6G new services, such as AI/ML, immersive communication and sensing, a dynamic QoS mechanism is needed to support, where the RAN directly adjusts the QoS requirements of QoS flows/PDU sets based on real-time conditions to ensure the timeliness and reliability of data transmission and processing. The following could be considered:

- Monitoring of key network indicators

- Identification of service types and requirements

- The design of the new processing flow

- Interaction between RAN and the core network

Proposal 6: As a starting point, RAN2 agrees that at least some metric from RVQoE, i.e., the buffer level and playout delay for media start-up can be regarded as service-awareness information and utilized for RAN operations, e.g., data differentiated handling and discarding.

References

[1]	RP-251881 New SID: Study on 6G Radio

[2]	R2_131bis_ChairNotes_25-10-17_17-00_final

[3]	https://arxiv.org/pdf/1909.03341

[4]	https://arxiv.org/pdf/1508.07909

[5]	https://arxiv.org/abs/1805.07445

[6]	https://tiktokenizer.vercel.app/