3GPP TSG-RAN WG2 Meeting #132		R2-2508858

Dallas, USA, 17th – 21st Nov. 2025

Source:	TCL

Title:	Discussion on energy efficiency for 6GR

Agenda Item:	10.3.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN#109 meeting, the revised SID [1] for 6G radio was agreed, in which some 6G requirements for energy efficiency, system simplification, diverse device types, and latency are included. In the RAN2#131bis meeting [2], the agreement on energy efficiency was reached as follows.

Agreements on energy efficiency: - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we will discuss the device with multiple capabilities, network power saving, and UE power saving.

- Discussion

  - The device with multiple capabilities

In LTE and NR, different devices demonstrate distinct operating characteristics. eMBB devices are primarily optimized for high throughput and multimedia services, requiring sustained bandwidth utilization and frequent radio activity. As a result, eMBB devices generally exhibit higher power consumption, and their coverage performance is limited by link budget constraints and antenna configurations. By contrast, IoT devices are specifically designed with an emphasis on low complexity, low power consumption, and extended coverage to support massive, low-rate, and intermittent connectivity.

From a power efficiency perspective, IoT devices typically employ simplified RF architectures, optimized modulation and coding schemes, and long-cycle discontinuous reception (DRX/eDRX) or similar power-saving mechanisms. These allow IoT devices to significantly reduce energy consumption in both NON-CONNECTED and CONNECTED modes. From a coverage perspective, IoT devices are often configured with more relaxed throughput requirements but enhanced link budget targets. Mechanism such as transmission repetition allows for much deeper coverage.

Observation 1: In LTE and NR, IoT devices have lower power consumption and wider coverage compared to eMBB devices.

In 5G, a device can only be the eMBB device or IoT device in a cell. In 6G, diverse device types will be supported over the same air interface. For achieving the 6G energy efficiency requirement, etc, considering IoT devices have lower power consumption and wider coverage compared to eMBB devices, a device with multiple capabilities (e.g., eMBB and IoT) can be supported. The device can switch between different capabilities to on demand achieve high throughput and power saving.

Proposal 1: A device with multiple capabilities (e.g., eMBB and IoT) can be supported to on demand achieve power saving and high throughput.

  - Network power saving

RACH

Assuming PRACH is transmitted periodically by network in one cell. As load changes, the requirement of energy efficiency may be met through adaptation of RACH configuration, like the Rel-19 NES topic. New requirements based on basic capability or QoS may bring other on-demand transmission design. For example, one case with on-demand PRACH transmission is shown in Figure 1 below. For each PRACH, it’ll not be transmitted if downlink signaling indicates periodicity changing or if some RACH resources are absent for some reason. In addition, for each preamble (group) during one PRACH resource, it will also not be transmitted due to the requirement of QoS or device capability.

Figure 1: On-demand RACH transmission

Observation 2: RACH adaptation or on-demand RACH  transmission can meet some 6G requirements for energy efficiency, QoS, various device types, etc.

Proposal 2: Consider and study RACH adaptation or on-demand RACH transmission in 6G.

Cell DTX/DRX

A 6G base station will utilize discontinuous transmission techniques extensively. This means in the absence of active data or required signaling, the network can shut off its transmit chain or reduce its activity. For example, if no UEs need downlink data at a moment and all required control signaling has been handled, the network can enter a micro-sleep and stop all transmissions. Such discontinuous transmission techniques will coordinate with both on-demand UE-specific signals and on-demand common signals.

In the current 5G, Cell DTX and UE DRX are configured independently. It is difficult to align the active time of different mechanisms, especially when facing massive UEs or bulk services. To minimize both the network’s and UE’s energy wasting and minimize the latency of data transmission, the Cell DTX/DRX mechanism considering UE DRX and common signals should be studied.

Proposal 3: The Cell DTX/DRX mechanism considering UE DRX and common signals should be studied.

  - UE power saving

PDCCH monitoring

PDCCH monitoring reduction is vital for UE energy saving. In 5G, UEs often monitor PDCCH in every slot or multiple times per frame, which is power-expensive. Smarter PDCCH scheduling mechanisms are introduced, such as PDCCH skipping, SSSG switching, C-DRX, DCP, and LP-WUS. For PDCCH skipping and SSSG switching, if setting up the same parameters and considering the same scenario, no significant gain for power savings could be discovered for SSSG switching or PDCCH skipping schemes.

Observation 3: If setting up the same parameters and considering the same scenario, no significant difference for power saving gain could be discovered between SSSG switching and PDCCH skipping schemes.

Connected-mode DRX allows a UE in RRC_CONNECTED to periodically power down its receiver when no data is scheduled, waking up at specified intervals to check for incoming traffic. For DCP and LP-WUS, a brief indicator signal prior to the C-DRX on-duration is used to inform the UE whether it needs to wake up or not. If the gNB has no data for the UE, it sends a “no wake-up” indication so the UE can skip monitoring and stay in low-power mode. The only difference between DCP and LP-WUS is the receiver for the indicator. We think the NR power-saving features above can still be considered in 6G to achieve the UE power saving. But the duplicated functions brought by SSSG switching and PDCCH skipping, or DCP and LP-WUS, should be avoided.

Proposal 4: Study potential schemes to reduce unnecessary PDCCH monitoring and monitoring complexity in 6G, at least including SSSG switching or PDCCH skipping, C-DRX, DCP or LP-WUS, etc.

Measurement relaxation

UEs carry out continuous measurement activities for  RRM, RLM, and BFD, even when the device mobility is low and the link quality is strong. These continuous measurements increase UE power consumption and overhead. To address this, 5G introduces measurement relaxation mechanisms, allowing UEs to reduce or skip certain measurements when conditions are met, thereby improving energy efficiency and simplifying processing.

Looking toward 6G, the measurement problem becomes more complex because of expanded frequency ranges (including FR3), denser multi-carrier operation, and highly heterogeneous devices from massive IoT sensors to latency-sensitive XR terminals. Several technical issues can be anticipated.

- First, the overhead problem: 6G may require measurements across many bands, beams, and carriers, and repeating today’s frequent CSI/SSB measurements would be prohibitively power-hungry.

- Second, alignment challenges: current measurements are not always synchronized with DRX/DTX or cell sleep cycles, leading to situations where UEs wake to measure signals that are not relevant, undermining energy savings.

- Third, QoS and capability mismatch: devices with very different capabilities (e.g., ～3MHz UE vs. advanced handsets) may be forced to follow uniform measurement patterns, wasting energy or missing accuracy.

- Finally, cross-domain integration will be harder, as sensing, positioning, and communication functions converge, creating measurement demands that may conflict with energy-saving goals.

Observation 4：6G measurement for energy efficiency faces excessive overhead, poor alignment with DRX/DTX, device capability mismatches, and integration conflicts with sensing and positioning.

In our view, the 5G measurement relaxation can still be considered to achieve the UE power saving. Considering some 6G measurement problems above, the measurement relaxation should be enhanced to address these problems.

Proposal 5: Measurement relaxation can be considered and enhanced to address the issues proposed in Observation 4.

- Conclusion

In this contribution, we discuss energy efficiency for 6GR. Based on the discussion, we have the following observations and proposals.

The device with multiple capabilities

Observation 1: In LTE and NR, IoT devices have lower power consumption and wider coverage compared to eMBB devices.

Proposal 1: A device with multiple capabilities (e.g., eMBB and IoT) can be supported to on demand achieve power saving and high throughput.

Network power saving

Observation 2: RACH adaptation or on-demand RACH  transmission can meet some 6G requirements for energy efficiency, QoS, various device types, etc.

Proposal 2: Consider and study RACH adaptation or on-demand RACH transmission in 6G.

Proposal 3: The Cell DTX/DRX mechanism considering UE DRX and common signals should be studied.

UE power saving

Observation 3: If setting up the same parameters and considering the same scenario, no significant difference for power saving gain could be discovered between SSSG switching and PDCCH skipping schemes.

Observation 4：6G measurement for energy efficiency faces excessive overhead, poor alignment with DRX/DTX, device capability mismatches, and integration conflicts with sensing and positioning.

Proposal 4: Study potential schemes to reduce unnecessary PDCCH monitoring and monitoring complexity in 6G, at least including SSSG switching or PDCCH skipping, C-DRX, DCP or LP-WUS, etc.

Proposal 5: Measurement relaxation can be considered and enhanced to address the issues proposed in Observation 4.

- References

- RP-252912, Revised SID: Study on 6G Radio

- Draft_RAN2_131bis_MeetingReport_v1