3GPP TSG-RAN WG2 Meeting #132	R2-2508875

Dallas, USA, Nov. 17th - 21st 2025

Agenda item:	10.4

Source:	Samsung

Title:	Study on 6G Mobility Framework

Document for:	Discussion and Decision

Introduction

In RAN#108 plenary, a study item on “6G RAN” was approved [1]. Based on the SID, the study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements. This contribution provides a summary of Samsung’s views on the general mobility principles, unified mobility framework including RRM measurements, RLM/RLF and cell reselection.

Discussion

General 6G Mobility Principles

The transition from 4G LTE (Long Term Evolution) to 5G NR (New Radio) has revolutionized mobile network capabilities, significantly enhanced data throughput, latency, and reliability. Mobility technologies—encompassing seamless handover (HO) mechanisms, cell reselection, and tracking area management—play a pivotal role in maintaining uninterrupted connectivity for users, particularly in dynamic environments.

LTE established foundational mobility protocols, including intra- and inter-frequency handovers and idle-mode cell reselection, optimized for macro-cell deployments. In contrast, 5G NR introduced ultra-reliable low-latency handovers, and AI/ML-based predictive algorithms to anticipate UE’s measurement results on top of the L3-based handover principles introduced in LTE. These innovations minimize signalling overhead, reduce latency, and ensure service continuity across networks, enabling next-generation applications. Since at least the same level of mobility performance as NR is expected, for 6G, the similar mobility KPI should be considered. In this aspects, among many objectives of mobility enhancements, latency reduction, robustness, simple design to implement and resource efficiency would be the main considerations on 6G mobility.

Figure 1. 5G NR mobility schemes

In Figure 1, various mobility schemes introduced in NR are depicted. These NR mobility schemes have improved in many years, so that the mobility performances of wireless communication are enhanced a lot in terms of latency reduction, robustness and energy efficiency. NR mobility enhancements were mainly targeted to reduce the latency and improve the robustness during the HO.

In RAN2#131bis meeting, RAN2 agreed to determine the main requirements for 6G mobility design as below.

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In addition, more detail aspects to achieve above three main requirements need to be studied, we believe that the features introduced in NR could be the good starting point for each requirement and some new feature could be introduced if needed. We provide the candidate features with evaluation to achieve each requirement for 6G mobility design. Mostly NR LTM features and some evaluation was done and it is captured in TS 38.300 Annex G. The features introduced in LTM (e.g. early DL/UL synchronization, pre-processing of RRC decoding, optimized beam management, etc.) reduces HO interruption time significantly. Here, the mobility interruption time for LTM is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. The latency of the RACH-based and RACH-less LTM procedure is characterized by the terms illustrated in Figure G-2 and Figure G-3 in [2]. This LTM feature does not require big hardware upgrade so LTM is now implemented in many UE and NW vendors, so we believe the LTM will be used in real NR deployment soon.

The robustness of mobility aims at reducing the potential failures during UE mobility (i.e. missing the HO command from NW due to the bad channel condition). To achieve this purpose, conditional mobility mechanisms, e.g., conditional handover (CHO), conditional LTM (CLTM), conditional PSCell addition/change (CPAC), were introduced in NR. We also think this CHO and CLTM could be considered as the basic 6G mobility framework but CPAC study in 6G could be dependent on DC support in 6G. So, we summarize the expected mobility schemes in 6G as below.

- Minimize interruption time

  - Early DL/UL synchronization: UE could early DL/UL synchronize to the candidate cells while connected to the source cell, this procedure allows UE to RACH-less HO.

  - Early TCI state activation: UE could use the best beam right after the HO execution, so it reduces the beam management procedure at the target cell.

  - Pre-configuration of RRCReconfiguration: it reduces the RRC processing delay.

  - Pre-processing of ASN.1 decoding: it reduces the RRC processing delay.

- Minimize throughput degradation during mobility

  - Early CSI acquisition: UE measures the preconfigured CSI measurements before/during the HO procedure and report the CSI measurement result to the target cell in the first UL MAC PDU.

  - SCell activation during HO

- Robustness of mobility procedures

  - Conditional handover (L3 based conditional LTM)

  - Layer 1 based conditional LTM

- Energy efficiency

  - Efficient measurement resource design (RAN1 impact): RRC state dependent measurement configuration could reduce the UE monitoring power for measurement.

  - Measurement gap enhancements (RAN1/4 impact):: Optimized measurement gap pattern (i.e. non-uniform gap pattern) could be considered for power saving on the inter-RAT/inter-frequency measurement.

Figure 2. Mobility Latency for RACH-less LTM

We think above features could be applied not only to the L1-based mobility scheme but also to the L3-based legacy HO without additional evaluation i.e. no big issues applying these features to L3 legacy HO. So, the L3 based NW indicated HO performance is also enhanced by these features.

Observation 1: 5G NR introduced ultra-reliable low-latency handovers (e.g. LTM features) on top of the L3-based handover principles introduced in LTE.

Proposal 1: 6G mobility framework consider the following mobility schemes as basic feature to achieve the latency reduction with minimum throughput degradation, robustness enhancement and energy efficiency.

- Minimize interruption time:

  - Early DL/UL synchronization

  - Early TCI state activation

  - Pre-configuration of RRCReconfiguration

  - Pre-processing of ASN.1 decoding

- Minimize throughput degradation during mobility

  - Early CSI acquisition

  - SCell activation during HO

- Robustness of mobility procedures

  - Conditional handover (L3 based conditional LTM)

  - Layer 1 based conditional LTM

- Energy efficiency

  - Efficient measurement resource design (i.e. RRC state dependency)

  - Measurement gap enhancements (i.e. non-uniform gap pattern)

Unified L1/L3 Measurement Framework

According to NR measurement model in Figure 2, NR measurement model uses the beam measuring and the cell quality is determined by the detected beam measurements. We call this as Layer 3 RRM measurement model. That is, UE measures each detected beams of the serving cell and neighbor cells and these beam measurement results are averaged to derive the cell quality. For doing this, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from the beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). The main role of RRM measurement is to report the cell quality, while reporting the L3 filtered beam quality is an optional feature. Therefore, the measurement reports may contain the measurement results of the X best beams (L3 filtered) if the UE is configured to do so by the gNB. However, these X best beams are not exactly same as the beams which are used for calculating the cell quality based on the measurement model.

Figure 3. NR Measurement Model

In addition, NR LTM introduced the different measurement model for Layer 1 measurements with the Layer 3 ones, we call this as Layer 1 RRM measurement model. In this measurement model, the cell quality is not used for target cell selection and only the beam level measurements are used for mobility decision. This Layer 1 RRM measurement model follows the CSI measurement/report configurations so the measurement resources are configured explicitly to UE for this RRM purpose. From the NW perspective, this measurement resource information sharing should be done in the preparation phase in order that the source cell provides these measurement resource in L1 measurement/report configurations for LTM preparation to UE. Plus, this reference signal configuration might be duplicated for CSI measurement resource configuration and L1 measurement resource configuration in serving cell and candidate cells. For example, L1 RSes in CSI-MeasConfig (CSI) and LTM-CSI-ResourceConfig (RRM) could configure the same RSes for candidate cells but the usage is different. So, this redundant measurement configuration could be optimized to reduce the RRC signaling overhead.

In short, NR has three types of L1/L3 measurement configurations as below.

- Layer 3 measurement configuration for RRM

  - Measurement object provides the L3 measurement resources to be measured

  - Report configuration provides detail report configurations

  - measId links the measurement object and report configuration

- Layer 1 measurement configuration for LTM

  - Common LTM L1 measurement resource configuration for all candidate cells

  - LTM report configuration provides the detail configuration with associated LTM resource configurations

- Layer 1 measurement configuration for CSI measurement (either source cell or cell(s) for additionalPCI)

  - CSI measurement resources and report configuration for serving cell is provided.

  - For ICBM (inter-cell beam management) is configured, CSI measurement resources in cell(s) for additionalPCI could be configured

  - If ICBM is considered as mobility scheme then this feature should be considered for unified measurement framework as well.

Since these L1 based mobility schemes were not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel. Beam measurements would be used for both RRM and CSI purpose if 6G will introduce the L1-based mobility scheme, so the unifying any duplicated signaling and optimizing for beam and cell measurement configuration and reporting (e.g. unified measurement reporting triggering structure), in L1/L3 measurement framework is required in 6G.

Observation 2: Since L1-based mobility scheme (LTM) was not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel.

Proposal 2: Unified framework for L1/L3 measurement configuration (i.e. measurement resource and report), which is used for L3-based RRM, L1-based RRM and CSI acquisition purpose, is studied.

Unified mobility framework

NR mobility enhancements have been carried out over several releases, resulting in a very complex design with multiple mobility schemes to cover different scenarios. While these enhancements provide flexibility, they have also introduced redundancy in configuration, measurements, and reporting. In many cases, the network sends duplicate configuration parameters, and the UE reports duplicate measurements (e.g., for L3, LTM, etc.) to support different procedures. This leads to inefficient signaling and increased complexity in mobility management. In addition, NR LTM use the MAC CE (L2 signal) as a cell switch command but it is not secured (e.g. lacks integrity protection). This cell switch command makes UE to go another cell, so it creates the performance loss if this cell switch command has been tempered with. We assume the MAC CE is the good signaling for L1 based mobility because the L1 measurement would be handled in DU, then the NW implementation could be reduced if MAC CE is used after DU decide the LTM cell switch. Therefore, we can enhance MAC CE considering security perspective, i.e. make the HO indication more reliable.

Moreover, we think the key mobility features to achieve the latency reduction with minimum throughput degradation robustness enhancement and energy efficiency as we proposed in Proposal 1, should be supported all kinds of mobility schemes (e.g. L3 based legacy HO, (C)LTM, etc.) for general design of 6G mobility. We think most of features could be supported regardless of the mobility schemes, some enhancements for 6G unified mobility design is needed.

Plus, the early UL TA acquisition was introduced in LTM to derive the TA value of the target cell in advance (i.e. RACH-less LTM). The advantage of this feature is to reduce the interruption caused by the RACH procedure during the cell switch. To support this feature, the network side needs to reserve some dedicated RACH resource. In the current NR specification, the dedicated RACH resource for early TA acquisition can be shared among the UEs from the same DU. Thus, for each candidate cell, it has to reserve the separate RACH resource sets, each of which are prepared for UEs belonging to one DU, such per-DU resource reservation will consume more RACH resource. We believe this early TA acquisition could be extended to many additional cases in 6G. On the other hand, such extension may result in more RACH resource reservation for different purposes. Thus, we may consider the unified early TA acquisition scheme for resource efficient design.

In short, the unified simple design for 6G mobility by considering the following aspects are required.

- Avoid duplicated cell preparation and configuration: One cell may be prepared for multiple times for different purpose (e.g. (C)LTM, CHO).

- Avoid unnecessary measurements and duplicate measurement reporting: Both L1 and L3 measurements for the same cell can be used for mobility triggering.

- Secured L3 or L2 cell switch command with prepared target cell configuration.

- Ensure the efficient resource allocation, e.g., early TA resource.

- Apply all 6G mobility features to all mobility schemes (e.g. L3 based legacy HO, (C)LTM, etc.)

Figure 4. 6G mobility framework

Proposal 3: RAN2 study the unified design for 6G mobility supporting following aspects:

- Avoid duplicated cell preparation and configuration

- Avoid unnecessary measurements and duplicate measurement reporting

- Secured L3 or L2 cell switch command with prepared target cell configuration.

- Ensure the efficient resource allocation, e.g., early TA resource.

- Apply all 6G mobility features to all mobility schemes (e.g. L3 based legacy HO, (C)LTM, etc.)

AI/ML-based mobility

Finally, AI/ML-based mobility solutions have been investigated in NR and some features can facilitate the mobility performance improvement. One such feature is RRM measurement prediction, which enables the UE to anticipate future measurement results for serving/neighboring cells. This allows the UE to initiate early measurement reporting with the predicted values, effectively predicting measurement events. Based on this early report, NW can proactively prepare or trigger HO procedure earlier to avoid too late handover, compared to the legacy HO that relies solely on historical measurement results. From this aspect, the benefits of the AI/ML-based mobility features (e.g., HOF rate decrease) have been evaluated and validated under L3 HO framework in Release 19, as outlined in the AI/ML MOB SI.

To enhance the robustness of mobility, we believe that AI-based measurement/event prediction can be further extended to (C)LTM. For instance, the UE could report the predicted L1 measurements for neighbor cells using a new CSI report format. This would enable NW to select target or candidate cells that are likely to maintain stable channel quality even after (C)LTM. Additionally, for CHO/CLTM execution conditions, which currently rely solely on historical measurement results, incorporating predicted future measurement results could further improve robustness and reliability.

In Release 19 AI/ML MOB SI, RLF prediction was initially included in SID as one of the use cases for study but was eventually deprioritized due to time constraints. However, the value of RLF prediction remains significant, as this failure event directly and negatively impacts user experience. If an RLF is predicted, the UE could report it to NW to request an early HO or initiate a fast recovery procedure, thereby minimizing its adverse effects. Building on this motivation and approach, beam failure prediction can also be explored for 6G. Given that beam failure events are likely to occur more frequently and earlier than RLF, beam failure event prediction can represent an even more useful and important use case. For the feasibility of RLF and beam failure prediction, we believe these can be effectively implemented by leveraging the existing features such as RRM measurement prediction (from Rel-20 AI/ML MOB) and beam prediction (from Rel-19 AI/ML PHY), both of which have already been evaluated. Thus, RAN2 can study RLF and beam failure prediction for 6G.

Observation 3: AI/ML-based mobility solutions have been investigated in NR basic mobility scheme (e.g. L3 HO) and some features (e.g. RRM measurement prediction, RLF prediction) can facilitate the mobility performance improvement.

Proposal 4: RRM measurement/event prediction is considered as Day-1 feature for 6G mobility. RAN2 study the application of AI-based measurement/event prediction for CHO, (C)LTM, RLF, and beam failure.

RLM/RLF and Fast Recovery

As a radio channel monitoring and the radio link failure handling procedure, NR defined the Radio Link Monitoring (RLM) and Radio Link Failure (RLF) procedures. For RLM, UE in the active BWP monitors the configured reference signals and check this measured quality with the configured thresholds. This RLM is used for both beam failure detection (BFD) and RLF cases.

For example, the UE declares RLF when one of the following criteria are met in NR:

-	Expiry of a radio problem timer started after indication of radio problems from the physical layer; or

-	Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running; or

-	Random access procedure failure; or

-	RLC failure; or

-	Detection of consistent uplink LBT failures.

After RLF declared the RRC re-establishment timer T311 starts, the UE selects a suitable cell and then initiates RRC re-establishment. Otherwise, UE goes to RRC IDLE state. We think this NR RLF procedure is also needed in 6G as a baseline. For the enhancement of radio link recovery procedure, NR introduced the fast recovery procedure in CHO and LTM, the detail procedure is captured below.

-	in case of CHO, for RLF in the source cell:

-	selects a suitable cell and if the selected cell is a CHO candidate and if network configured the UE to try CHO after RLF then the UE attempts CHO execution once, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

-	in case of MCG LTM, for RLF in the source cell:

-	selects a suitable cell and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after RLF then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

We think the initial 6G mobility framework considers the pre-configurations of the candidate target cells as introduced in NR CHO and LTM as we proposed above. Then, this fast recovery procedure could be also applied for the unified radio link recovery scenario (i.e. RLF, HO failure).

Observation 4: RLM procedure in NR is commonly defined for both RLF and BFR, and the fast recovery procedures introduced in CHO and LTM enhanced the performance of the radio failure cases in terms of recovery time.

Proposal 5: RLM/RLF procedure in NR is used for the baseline and the unified fast recovery mechanism is applied for RLF and HO failure cases.

Cell Reselection

In NR, the IDLE/INACTIVE mode mobility is defined based on following cell reselection criteria which involves measurements of the serving and neighbour cells.

- Cell reselection is always based on cell defining sync signals

- Intra-frequency reselection is based on ranking of cells

- Inter-frequency/inter-RAT reselection is based on absolute priorities

- Cell reselection can be speed dependent

- Cell/Service specific prioritization

We think this cell reselection criteria which uses the frequency priorities and radio quality is stable in performance in LTE/NR so it could be reused in 6G cell reselection framework. In addition, NR introduced the slice-aware cell reselection separately in later release and we think RAN2 could consider this feature in 6G day1 based on the progress of other working group for 6G.

Proposal 6: NR cell reselection framework is the baseline of 6G cell reselection framework.

Conclusion

From lessons learned from NR, the following observations are justified:

Observation 1: 5G NR introduced ultra-reliable low-latency handovers (e.g. LTM features) on top of the L3-based handover principles introduced in LTE.

Observation 2: Since L1-based mobility scheme (LTM) was not considered in the beginning of NR, NR measurement models for RRM and CSI are designed independently, and also these models can work in parallel.

Observation 3: AI/ML-based mobility solutions have been investigated in NR basic mobility scheme (e.g. L3 HO) and some features (e.g. RRM measurement prediction, RLF prediction) can facilitate the mobility performance improvement.

Observation 4: RLM procedure in NR is commonly defined for both RLF and BFR, and the fast recovery procedures introduced in CHO and LTM enhanced the performance of the radio failure cases in terms of recovery time.

RAN2 is requested to discuss and agree the following proposals:

Proposal 1: 6G mobility framework consider the following mobility schemes as basic feature to achieve the latency reduction with minimum throughput degradation, robustness enhancement and energy efficiency.

- Minimize interruption time:

  - Early DL/UL synchronization

  - Early TCI state activation

  - Pre-configuration of RRCReconfiguration

  - Pre-processing of ASN.1 decoding

- Minimize throughput degradation during mobility

  - Early CSI acquisition

  - SCell activation during HO

- Robustness of mobility procedures

  - Conditional handover (L3 based conditional LTM)

  - Layer 1 based conditional LTM

- Energy efficiency

  - Efficient measurement resource design (i.e. RRC state dependency)

  - Measurement gap enhancements (i.e. non-uniform gap pattern)

Proposal 2: Unified framework for L1/L3 measurement configuration (i.e. measurement resource and report), which is used for L3-based RRM, L1-based RRM and CSI acquisition purpose, is studied.

Proposal 3: RAN2 study the unified design for 6G mobility supporting following aspects:

- Avoid duplicated cell preparation and configuration

- Avoid unnecessary measurements and duplicate measurement reporting

- Secured L3 or L2 cell switch command with prepared target cell configuration.

- Ensure the efficient resource allocation, e.g., early TA resource.

- Apply all 6G mobility features to all mobility schemes (e.g. L3 based legacy HO, (C)LTM, etc.)

Proposal 4: RRM measurement/event prediction is considered as Day-1 feature for 6G mobility. RAN2 study the application of AI-based measurement/event prediction for CHO, (C)LTM, RLF, and beam failure.

Proposal 5: RLM/RLF procedure in NR is used for the baseline and the unified fast recovery mechanism is applied for RLF and HO failure cases.

Proposal 6: NR cell reselection framework is the baseline of 6G cell reselection framework.

References

[1]	RP-251881		New SID: Study on 6G Radio	NTT DOCOMO.

[2]	TS 38.300		NR and NG-RAN Overall Description; Stage 2		Rel-18.