3GPP RAN WG2 Meeting #132	R2-2508901

Dallas, United States, November 17th – 21st 2025

Agenda Item:	8.1.2

Source:	InterDigital

Title:		Discussion on LCM-related RILs: [S052] [S054] [S050] [Z006] [E052]

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining issues related to implementation of the LCM procedure has been provided as part of the AIML ASN.1 review [2] [3]. This contribution discusses these remaining issues, specifically those related to UE sided model for BM use case.

# Remaining Issues marked ‘ToDo’

The following issues and descriptions have been extracted from the AIML issues list [2] [3] as part of the ASN.1 review, and listed as “ToDo” by the TS 38.331 specification Rapporteur.

## [S053/S054] AIML interaction with NTN and NR-U

[S053] and [S054] discuss the applicability of AI/ML to NTN and NR-U features respectively. Although optimizations to support each feature should not be introduced at this stage, the proposed changes impact only field descriptions (i.e., the inclusion of SRBX to the field descriptions of allowedHARQ-mode and channelAccessPriority) and may be considered if there is potential for benefit with minimal discussion.

NTN is characterised by large beam footprints ranging from 50km to 1000km+ in diameter, which are classified as earth-fixed, quasi-earth-fixed, or continuously moving across the earth. Because a single GSO satellite beam may serve an entire country, a UE may only ever be served (and measure) a single satellite beam. In this scenario the usefulness of AIML BM is limited. Similarly, positioning measurement in NGSO deployments may be taken from only a single TRP (i.e., satellite), but over multiple time instances (since due to the satellite movement a single TRP can act like multiple TRPs when taken at different time instances). This also is different from terrestrial positioning and may require further discussion on possible impacts to existing AIML framework.

NR-U is characterised by shared access, where a UE may transmit only if the channel is unoccupied. Occupancy is determined via the Listen-before-Talk (LBT) procedure, where the UE is required to monitor the channel for a period prior to transmission, with duration of listening period dependent on the transmission priority (based on Channel Access Priority Class (CAPC)). In situations where LBT fails and transmission is blocked/delayed, AIML seems beneficial to subsititute missing measurements, however the randomness of channel occupancy may impact AIML performance.

Although there are aspects of each feature which are different, the characteristics of NTN change vastly between deployment scenarios and would take far too much time to properly discuss how/whether each AIML use case would work with each deployment. The use of AIML in NR-U has clear benefits, but performance of the existing framework may be impacted by randomness of occupancy. Although optimization for this case will not be pursued (e.g., to account for randomness of LBT failure), there may still be potential benefits based on smart implementation. It is proposed that it not be explicitly disallowed in this release.

Proposal 1:	[S053] AIML for PHY is not supported in NTN for Rel-19.

Proposal 2:	[S054] AIML for PHY is not explicitly disallowed in NR-U. Add SIBx to the field description of channelAccessPriority.

## [A104] Simplification of procedural text on applicability reporting

[A104] highlights that the current procedural text for applicability reporting uses the following to describe inference configuration for beam management “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19”. As this is quite complex, [A104] notes that configurationForChannelPrediction is also only used for inference configuration for beam management, and would simplify the description if used instead.

The proposed change improves readability of the specification without technical impact and should be adopted. Although this change may be affected by ongoing revisions in [N021] and [H003], it should be listed as PropAgree for now with the provided TP as baseline. Any additional coordination can be completed in the CR review phase.

Proposal 3:	[A104] is PropAgree. Replace instances of “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19,” with “ including configurationForChannelPrediction” in procedural text for applicability reporting.

## [S050] UAI retransmission after HO/CHO/LTM cell switch

[S050] discusses UAI retransmission after mobility, where if the UE has sent UAI to the source cell within 1 second of handover or UE has sent UAI and a CHO/LTM cell switch has occured, the UE must resend UAI to the target cell. It is noted that based on current RRC specification the UE will report the new applicability after handover in RRCReconfigurationComplete so there is no need to retransmit UAI. Furthermore, there are existing triggers to transmit UAI to target cell (such as change of applicability) to notify the network of any subsequent change in applicability. It is therefore suggested that [S050] by listed as PropAgree.

Proposal 4:	[S050] is PropAgree. UE does not automatically retransmit UAI to target cell upon handover/CHO/LTM cell switch, and instead relies on RRCReconfigurationComplete and/or existing UAI triggers (i.e., change of applicability) to report applicability.

## [Z006] Field description of applicabilityReportList

[Z006] notes that according to the current field description for applicabilityReportList, there is an implication that the applicability report list in RRCReconfigurationComplete includes the parameters for prediction configuration, proposing to remove the related text:

applicabilityReportList The applicability reports related to prediction configurations and sets of parameters for prediction configurations.

This is however contrary to the following agreement from RAN2#130:

“RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.”

An alternative suggestion has been provided by the Rapporteur suggesting to add the text “related to” also for the sets of parameters for prediction configuration. This suggestion seems a reasonable clarification which is also inline with past agreement.

Proposal 5:	[Z006] Update the field decription of applicabilityReportList to “and related to sets of parameters for prediction configurations”.

## [E052] Unnecessary reportQuantity values from otherConfig

[E052] notes that not all the values for reportQuantity-r19 included in otherConfig are needed for applicability reporting (specifically those for used for UE-side data collection and monitoring). However, the current implementation is based on the following RAN1 agreement, which assumes that legacy parameters are reused:

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, - Note: this doesn’t imply the associated ID is always present

From RAN2 perspective, the legacy reportQuantity parameter contains unnecessary information for applicability determination and the proposal in [E052] is correct. However, by strictly following RAN1 agreement above the legacy parameter for reportQuantity is to be used including all subparameters. This is similar to the discussion in RAN2#131bis for [H010] which brought up a similar issue for reportConfigType (which was ultimately PropReject). Unless there is strong motivation in RAN2 (which doesn’t seem to be the case as both implementations work and overhead savings are minimal) there seems not enough motivation to overturn an explicit RAN1 agreement. It is therefore proposed [E052] is marked as “PropReject”.

Proposal 6:	[E052] is PropReject. Legacy IE for reportQuantity-r19 is used for applicability reporting as per RAN1 agreement.

# Conclusion

In this contribution the following proposals are made regarding issues identified during the AIML ASN.1 review:

Proposal 1:	[S053] AIML for PHY is not supported in NTN for Rel-19.

Proposal 2:	[S054] AIML for PHY is not explicitly disallowed in NR-U. Add SIBx to the field description of channelAccessPriority.

Proposal 3:	[A104] is PropAgree. Replace instances of “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19,” with “ including configurationForChannelPrediction” in procedural text for applicability reporting.

Proposal 4:	[S050] is PropAgree. UE does not automatically retransmit UAI to target cell upon handover/CHO/LTM cell switch, and instead relies on RRCReconfigurationComplete and/or existing UAI triggers (i.e., change of applicability) to report applicability.

Proposal 5:	[Z006] Update the field decription of applicabilityReportList to “and related to sets of parameters for prediction configurations”.

Proposal 6:	[E052] is PropReject. Legacy IE for reportQuantity-r19 is used for applicability reporting as per RAN1 agreement.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

Comments file – Rel-19 CR ASN.1 Review

Review file  – Rel-19 CR ASN.1 Review