3GPP TSG-RAN WG2 Meeting #132		R2-2508905

Dallas, USA, 17th - 21st Nov. 2025

Source:	TCL

Title:	Discussion on Data Transfer for 6G RAN

Agenda Item:	10.3.3.1

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, several consensuses on the requirements of data collection and the next research directions are approached, and the related agreements are provided in the following part.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

In the next chapter, the detailed analyses and views are provided.

- Discussion

In the existing 5G system, in order to support network optimization and AI-related enhancements, the standard has defined multiple solutions to support the management and transfer of different types of data (e.g., AI-related data, sensing data, QoE, SON/MDT, etc.). However, the fragmented design of these solutions in 5G significantly impacts scalability; i.e., each time a new data transfer requirement arises, a new solution needs to be designed.

Moreover, current 5G data collection schemes typically use either the control plane or the user plane for data transfer. The control-plane transfer method has limited throughput and latency performance, and data transfer can also affect the efficiency of signaling transmission. The user-plane transfer method is constrained by the transmission path — specifically, the data transfer path must include at least the UE to the UPF — which means the user-plane method cannot meet the requirements for flexible data delivery endpoints.

Observation 1: The existing 5G data transfer solutions are fragmented and weak in scalability. At the same time, current data transfer methods based on the control plane and user plane cannot meet the data transfer requirements of future 6G.

Therefore, 6G needs a unified data framework to cover the management, transfer, processing, and storage of existing data and meet the performance requirements; this data framework also needs to support the data requirements of future new services. Considering that some data transfer, processing, and storage is only related to the RAN (for example, when data are collected for beam management, channel prediction, channel compression and other AI use cases related to the air interface), 6G should consider whether to introduce a data management function on the RAN side. Furthermore, 6G needs to consider the relationship between the RAN data management function and the CN data management function — for example, whether they are parallel or hierarchical — and how to divide responsibilities and collaborate between them.

Proposal 1: 6G should consider whether to introduce a data management function on the RAN side, how the relationship between the RAN data management function and the CN data management function is (e.g., whether they are parallel or hierarchical), and how to divide responsibilities and collaborate between them.

Similar to traditional user plane data, the privacy and security protection of data, such as AI-related data, sensing data, QoE, SON/MDT, etc., is also critical. However, unlike user plane data which only requires the network to transparently forward it, these types of data may require user privacy protection before arriving at the final delivery node (i.e., data consumer). Additionally, these data may involve processing and storage at nodes beyond the final delivery node. Specifically, because the network — at the request of a data consumer — initiates a data collection process for the user, the collected data contains potential information that may exceed the scope of the user’s privacy consent given to that data consumer. Therefore, the user or the network needs to anonymize the collected data before delivery, in order to ensure the user’s privacy.

Observation 2: Collected data may require user privacy protection before arriving at the final delivery node, and may also involve processing, storage, and other operations at nodes beyond the final delivery node.

Considering that a user’s collected data also has privacy protection requirements with respect to the mobile network, 6G needs to support a mechanism for the user to indicate whether the collected data can be visible to the network, processed by the network, or stored by the network. For any collected data that is stored by the network, 6G also needs to support the user in indicating limiting conditions for that data, such as the scope of its use and the intended recipients.

Proposal 2: 6G needs to support allowing the user to indicate whether collected data can be made visible to the network, processed by the network, or stored by the network, and also support the user in specifying limiting conditions (e.g., usage scope and intended recipients) for stored data.

From current 5G data collection services, including the same service type and different service types, it can be observed that collected data is often repetitive in aspects such as data type, collection target, collection time, etc. This means that different data consumers from the same or different service types may have similar data needs—for example, they may all require channel information of users in a certain area. If the network carries out separate data collection procedures for different data consumers, then for the collecting side it could result in duplicate collection and duplicate transmission, which wastes computing and transmission resources.

Observation 3: In 5G system, decoupled data collection procedures for different data consumers may lead to additional computing and transmission resources consumption.

Therefore, in 6G, it is necessary to study how to improve data utilization in order to reduce the overhead of computing resources and air interface resources. Specifically, it is required to study how to identify the similarity of the data requests and how to merge the similar requests. Furthermore, considering that different data consumers might simultaneously need the same type of data or might need the same type of data in different time periods, 6G needs to study how to improve data utilization in these two scenarios as well.

Proposal 3: In 6G, it is necessary to study how to improve data utilization, thereby reducing the overhead of computing resources and air interface resources. Considering that different data consumers may simultaneously require the same type of data or require the same type of data in different time periods, 6G also needs to study how to improve data utilization in both scenarios.

In proposals from the last meeting, many companies suggested introducing a new data plane in 6G to achieve a unified data framework. Considering that multiple data transfer solutions have already been designed in 5G, in a future 6G system, if a new data plane is introduced, is it still necessary to retain control-plane (e.g., RRC message transfer) or user-plane data transfer methods as supplements? For example, for transferred data that are small in scale and not highly sensitive to latency, could the control-plane method still be used for transmission?

Proposal 4: In 6G, if a new data plane is introduced, it should be studied whether there is still a need to retain control-plane (e.g., RRC message transfer) or user-plane data transfer methods as supplements.

In proposals from the last meeting, many companies also mentioned adding a new radio bearer for data transfer. However, based on current data transfer requirements, many needs can be met through user-plane transfer. Therefore, as a precondition for introducing a new radio bearer, it is necessary to first determine the necessity of introducing it, and this necessity can be analyzed based on the transmission and processing requirements of different types of data.

Proposal 5: Before introducing a new radio bearer, its necessity should first be determined; this necessity can be analyzed based on the transmission and processing requirements of different types of data.

- Conclusion

In this contribution, we discuss the data transfer for 6G radio access technology. Based on the discussion, we have the following observations and proposals.

Observation 1: The existing 5G data transfer solutions are fragmented and weak in scalability. At the same time, current data transfer methods based on the control plane and user plane cannot meet the data transfer requirements of future 6G.

Proposal 1: 6G should consider whether to introduce a data management function on the RAN side, how the relationship between the RAN data management function and the CN data management function is (e.g., whether they are parallel or hierarchical), and how to divide responsibilities and collaborate between them.

Observation 2: Collected data may require user privacy protection before arriving at the final delivery node, and may also involve processing, storage, and other operations at nodes beyond the final delivery node.

Proposal 2: 6G needs to support allowing the user to indicate whether collected data can be made visible to the network, processed by the network, or stored by the network, and also support the user in specifying limiting conditions (e.g., usage scope and intended recipients) for stored data.

Observation 3: In 5G system, decoupled data collection procedures for different data consumers may lead to additional computing and transmission resources consumption.

Proposal 3: In 6G, it is necessary to study how to improve data utilization, thereby reducing the overhead of computing resources and air interface resources. Considering that different data consumers may simultaneously require the same type of data or require the same type of data in different time periods, 6G also needs to study how to improve data utilization in both scenarios.

Proposal 4: In 6G, if a new data plane is introduced, it should be studied whether there is still a need to retain control-plane (e.g., RRC message transfer) or user-plane data transfer methods as supplements.

Proposal 5: Before introducing a new radio bearer, its necessity should first be determined; this necessity can be analyzed based on the transmission and processing requirements of different types of data.