3GPP TSG RAN WG2 #132	R2-2508914

Dallas, USA, Nov 17th-21st, 2025

Agenda Item:	8.8.2

Source: 	Xiaomi

Title:  	Discussion on the RRC corrections for NR NTN

Document for:	Discussion and Decision

Introduction

In the contribution, we further discussed the RRC corrections for NR NTN and provide the corresponding suggestions.

Discussion

RIL C003

C003 indicates the need to clarify how the UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not fully aligned, and suggests adding a note as follows:

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

The core network provides the target service area to the UE via the service announcement and the AMF provides the intended service area to NG-RAN via the NG interface respectively in the MBS session establishment for broadcast procedure, but in the MBS session update for broadcast procedure, only the AMF may provide the updated ISA to NG-RAN. Therefore, a misalignment between the Target Service Area in the USD and the ISA in SIB27 may occur, and the NG-RAN always has the latest ISA from the core network.

Observation 1: A misalignment between the Target Service Area in the USD and the ISA in SIB27 may occur, and the NG-RAN always has the latest ISA from the core network.

Therefore, we think the UE should use the service area provided in the SIB27 when the misalignment between the Target Service Area in the USD and the ISA in SIB27 happened.

Proposal 1: We suggest adding a note to clarify that the UE ignores the service area information in USD if the Target Service Area in the USD and the ISA in SIB27 is not aligned.

Service continuity for MBS

In the last meeting, RAN2 agreed that the service continuity issue can be further discussed [1].

Proposal 1	Broadcast future neighbour cells providing the same MBS service(s) as the serving cell.

Proposal 2	A UE is informed of an area over with uniform MTCH configuration parameters for a certain MBS service regardless the serving cell or satellite.

Can be discussed in the next meeting

According to the contribution, it is assumed that by introducing a uniform MTCH configuration for a given MBS service within a defined area, the UE would not need to reacquire the MCCH when performing cell reselection or handover if the target cell is listed in the future neighbour cell list and offers the same MBS service as the serving cell.

However, we believe this assumption is not practical for the following reasons:

- Uniform MTCH configuration across different cells or satellites is not feasible in real deployments, as resource allocation for MBS services is subject to dynamic scheduling by the gNB. Enforcing uniformity would constrain the network’s ability to adapt to varying traffic loads, link conditions, and service-specific requirements, potentially leading to inefficient resource utilization.

- As a result, the UE must reacquire the MCCH upon cell reselection or handover, even if the target cell is known to provide the same MBS service. This is necessary to obtain the updated PDSCH resource configuration specific to the new cell.

Proposal 2: Broadcasting future neighbour cells providing the same MBS service(s) as the serving cell cannot eliminate the need for MCCH reacquisition, since uniform MTCH configuration across different cells/satellites is not feasible.

Conclusions

In this contribution, we discussed the service area misalignment between Sib27 and USD and the service continuity for MBS and provide the proposal as follows：

RIL C003

Observation 1: A misalignment between the Target Service Area in the USD and the ISA in SIB27 may occur, and the NG-RAN always has the latest ISA from the core network.

Proposal 1: We suggest adding a note to clarify that the UE ignores the service area information in USD if the Target Service Area in the USD and the ISA in SIB27 is not aligned.

Proposal 2: Broadcasting future neighbour cells providing the same MBS service(s) as the serving cell cannot eliminate the need for MCCH reacquisition, since uniform MTCH configuration across different cells/satellites is not feasible.

Service continuity for MBS

Proposal 2: Broadcasting future neighbour cells providing the same MBS service(s) as the serving cell cannot

eliminate the need for MCCH reacquisition, since uniform MTCH configuration across different cells/satellites is not feasible.

References

[1] R2-2507634, Service continuity in MBS NTN