3GPP TSG-RAN WG2 Meeting #132	R2-2508930

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	10.3.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on initial and system access and others of 6GR

Document for:	Discussion and Decision

1 Introduction

It was agreed in RAN2#131bis [1] to system information and paging with following design goals:

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplify for both UE and Network.

In this contribution, we will discuss the SSB, system information and paging enhancement.

2 Discussion

## 2.1 On-demand SSB

In 5G R19, considering the back-compatibility, on-demand SSB is only supported for RRC-connected UE on Scell in CA scenario. In 6G day-1, without the back-compatibility issues, scenario of on-demand SSB can be extended. Both single cell and multi cell on-demand SSB for idle/inactive mode UE can be studied.

Standalone OD-SSB for single cell scenario

For single cell scenario, there are two cases: case 1 is that no always-on sync signals is transmitted in the cell, and case 2 is that always-on SSB with large periodicity (e.g., 160ms) is transmitted in the cell. For case 1, UE needs on demand SSB for synchronization. For case 2, on-demand SSB can be used for following purposes:

- Detection of AO-SSB

  - If AO-SSB enables one-shot detection, the detection performance can meet requirement, so we think this scenario has low priority.

- Synchronization of SIB1 reception or paging reception or RACH process

  - In 5G R19, OD-SSB is introduced for SCell to let UE achieve synchronization with SCell as soon as possible. For idle/inactive mode, it may be also necessary for UE to achieve synchronization with serving cell as soon as possible. In other words, UE may need request assistance of OD-SSB with short periodicity for synchronization before SIB1 reception or paging reception or RACH processing by sending UL WUS. How to provide UE the UL WUS configuration needs to be discussed.

    - For SIB1 reception, UL WUS configuration can be pre-defined or configured by MIB.

    - For paging reception or PRACH process, UL WUS configuration can be configured by SIB.

Figure 1 Always on SSB with large periodicity in standalone cell

In this scenario, by leveraging OD-SSB, the network enables UE synchronization while reducing power consumption.

Proposal 1: Study standalone OD-SSB for single cell scenario in 6G day-1.

Cell A assisted OD-SSB&SIB1 for multi cell scenario

Similar as R19 OD-SIB1 operation, Cell A assisted OD-SSB for multi cells (cell A and NES cell) can also be studied in 6GR. As an example, it can be shown in following Figure 2. There are periodic AO-SSB in cell A and no AO-SSB/SIB1 in NES cell. For the purpose of UE to discover the NES cell, NES cell may transmit the reference signal, such as low power RS.

Figure 2 Cell A assisted OD-SSB&SIB1 for multi cell scenario

UE can receive cell A’s SSB and SIB, and acquire UL WUS configuration for NES cell. UE can measurement other lower power reference signal from NES cell and identify a NES cell that meet the conditions (e.g. the NES cell with an RSRP value not less than the threshold). If the UE need to access the target NES cell, then the UE can send UL WUS to the NES cell to trigger OD-SSB based on the UL WUS configuration of the NES cell.

In this scenario, the NES cells are not required to transmit AO-SSB and SIB1. This eliminates the need for certain wake periods, thereby increasing sleep opportunities and maximizing overall network energy efficiency.

Proposal 2:  Study Cell A assisted OD-SSB&SIB1 operation for multiple cell scenario in 6G day-1.

## 2.2 On-demand SIB1

In 5G, cell A assisted on-demand SIB1 is support for NES cell for cell reselection procedure as shown in figure 3 (a). UE camps on Cell A and receive UL WUS configuration from Cell A’s SIB. UE can send UL WUS to NES cell to trigger OD-SIB1 if the UE need to access the NES cell due to cell reselection. When receiving corresponding RAR of UL WUS, UE would monitor SIB1 in a time window. 5G has some limitations for multi cell scenario, which restrict power saving gain. Single cell scenario could be main scenario in 6G day-1. On-demand SIB1 for single cell can be studied in 6GR as shown in figure 3 (b). For single cell scenario, the main issue is how to obtain UL WUS configuration.

| (a) On-demand SIB1 procedure for multi cell in 5G | (b) On-demand SIB1 procedure for single cell |
| --- | --- |

Figure 3 On-demand SIB1 procedure for different scenarios

Proposal 3: Cell A assisted OD-SIB1 can be inherited to 6G.

Proposal 4: Study standalone on-demand SIB1 in 6G day-1.

## 2.3 Paging enhancement

In 5G first release design, PFs/POs are uniformly distributed in time-domain and the network has to frequently transmit paging signals across all time-domain positions, thus the network cannot enter the sleep mode for energy saving.

In Rel-19, to support the adaptation of POs in the time domain for energy saving, the N/Ns extension has been introduced to allow paging occasions to adapt in the time domain for energy saving. However, the energy saving gain is limit and this method cannot achieve optimal energy efficiency among all possible paging cycles. On the UE side, some UEs need to keep awake for a long time to monitor their own POs after synchronization, especially when the PFs/POs are distributed over long paging cycles. Thus, from 6G Day1, it is necessary to consider a more energy efficiency paging adaptation scheme.

Proposal 5: Study a more energy efficiency paging adaptation scheme.

3 Conclusion

In this contribution we discuss initial and system information and others with the following proposals:

Proposal 1: Study standalone OD-SSB for single cell scenario in 6G day-1.

Proposal 2:  Study Cell A assisted OD-SSB&SIB1 operation for multiple cell scenario in 6G day-1.

Proposal 3: Cell A assisted OD-SIB1 can be inherited to 6G.

Proposal 4: Study standalone on-demand SIB1 in 6G day-1.

Proposal 5: Study a more energy efficiency paging adaptation scheme.

- References

- RAN3 #131-bis Meeting Report, ETSI MCC, Prague, Czech Republic, 13 - 17 Oct 2025.