3GPP TSG-RAN WG2 #132	Tdoc R2-2508968

Dallas, USA, 2025-11-17 - 2025-11-21

Agenda Item:	10.2.1

Source:	KDDI Corporation

Title:	Discussion on IODT problems for 6G UE capability framework

Document for:	Discussion

1	Introduction

In RAN2 #131-bis, it was agreed that as part of the 6G UE capability framework study, companies can focus on practical IODT (Interoperability and Device Testing) problems observed in the field and discuss possible ways to address them[1]. This contribution presents a practical IODT problem that operators face when deploying new features in commercial networks.

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

2	Discussion

We are continuously promoting the commercial introduction of new gNB functions (both standardized and vendor-specific) related to Handover (HO) or Carrier Aggregation (CA) to improve network quality of experience. Some of these new functions require field trials and parameter tuning before full commercial deployment.

However, during field trials or after commercial introduction, we sometimes discover undesirable and unexpected behavior that occurs only on specific UE models and/or specific UE software versions. In the worst-case scenario, this can lead to the UE crashing and service loss.

In such cases, to avoid customer complaints, the operator must stop the rollout (i.e., roll back) the new function, even if the function provides quality improvements for all other UEs. The operator cannot re-deploy the new feature until the problematic UEs complete a FOTA (Firmware Over The Air) update.

FOTA rollout is very slow as it requires end-user consent and depends on user timing. During this FOTA delay, we cannot deploy the new function, and consequently, other users (without the issue) miss the opportunity for quality improvement.

Figure 1 illustrates this situation.

Based on this discussion, we provide the following observations and proposals.

Observation 1: In commercial environments, problems exist that only occur on specific UE models and specific UE software versions.

Observation 2: Due to these specific problems, other users may be deprived of opportunities for quality improvement.

These problems are significant in 5G and are expected to become more severe in 6G with the introduction of more features and diverse device types. Learning from 5G, 6G should address this issue from the early design phase.

Proposal 1: 6G should study enabling the network to identify the UE's model and/or software version.

Proposal 2: 6G should study enabling the network to control the applicability of a function (i.e., enable/disable) based on the UE's model and/or software version.

Figure 1: Illustration of IODT problem impact on quality improvement

3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1: In commercial environments, problems exist that only occur on specific UE models and specific UE software versions.

Observation 2: Due to these specific problems, other users may be deprived of opportunities for quality improvement.

Proposal 1: 6G should study enabling the network to identify the UE's model and/or software version.

Proposal 2: 6G should study enabling the network to control the applicability of a function (i.e., enable/disable) based on the UE's model and/or software version.

4	References

R2_131bis_ChairNotes_25-10-17_17-00_final