3GPP TSG-RAN WG2 Meeting #132	 R2-2508976

Dallas, USA, November 17-21, 2025

Source:	KDDI Corporation

Title:	Discussion on 6GR User plane.

Tdoc Type:	Discussion

Agenda Item:	10.3.1.1

Document for:  Discussion

Release:       Rel-20

1	Introduction

In RAN#108 meeting, following Radio Interface protocol architecture and procedures for 6GR objective was introduced [1]:

One of the detailed objectives of the study are:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This paper is a contribution regarding the 6G user plane.

2	Discussion

2.1 SDAP Layer

According to 3GPP TS 38.300, the SDAP layer is responsible for mapping QoS Flows to Data Radio Bearers (DRBs), assigning the QoS Flow Identifier (QFI) to each packet, and supporting features such as Reflective QoS. However, considering the limited commercial deployment of certain features like Reflective QoS in 5G NR, it is important to reassess the role of SDAP in the context of 6GR

For 6GR, it is reasonable to start from the 5G baseline and consider enhancements to meet new requirements such as low latency, high reliability, and flexible QoS control.

To simplify the protocol stack and improve processing efficiency, it is worth exploring the possibility of merging SDAP functions into PDCP. This could help reduce overhead and latency, which are critical in 6G systems.

Proposal 1: RAN2 study whether the necessary functions for SDAP in 6G continue to be handled by SDAP or be integrated into PDCP.

3	Conclusion

We make the following observations and proposals:

Proposal 1: RAN2 study whether the necessary functions for SDAP in 6G continue to be handled by SDAP or be integrated into PDCP.

3	　Reference

[1] RP-251881, NTT Docomo (Moderator), "Study on 6G Radio"