3GPP TSG-RAN WG2 Meeting #132                                                                      R2-2508986

Dallas, USA,  Nov. 17th - 21st , 2025

Agenda item:	10.3.3.2

Source:	China Telecom

Title:	Discussion on 6G energy efficiency

Document for:	Discussion

- Introduction

The RAN Plenary has approved a new Study Item on next-generation Radio Access Technology [1]. The 6GR Study Item aims to develop a non-backward-compatible RAT capable of supporting a wide range of use cases and requirements identified during the RAN requirements study. The study prioritizes user experience, energy efficiency, and system extensibility while also addressing practical considerations in spectrum utilization and network deployment. In RAN2#131bis meeting, the following agreements on energy efficiency were achieved:

Design goal

- Strive to develop solutions that consider network energy efficiency and UE power saving

- For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

Based on the above progress, the present document presents a preliminary analysis of UE and network energy saving in 6G system.

- Discussion

Despite the introduction of multiple energy-saving enhancement solutions in 5G system from Rel-15 to Rel-19, the largely independent design of UE and network mechanisms has resulted in limited system-wide coordination. Challenges such as inconsistent evolution across releases and rising network complexity further hinder a balanced approach to energy efficiency. Moving to 6G, it will be critical to understand how network energy saving affects UE power consumption, and to ensure that future network operations and UE power-saving features are co-designed to account for these interactions.

Observation 1: In 5G NR, energy saving mechanisms were designed separately for UE and network, which resulted in limited coordination between the two sides and consequently constrained overall energy efficiency.

Proposal 1: To joint UE-network energy saving must be a high priority in 6G SI phase to develop a holistic framework where network operations and UE power saving mechanisms are seamlessly integrated to maximize overall system efficiency.

2.1 UE Energy Saving

The RRC_INACTIVE state was introduced in 5G to enhance UE energy efficiency and reduce signaling overhead. In this state, the UE retains its AS context and supports mechanisms such as fast RRC resume, paging, and RNA updates, resulting in lower latency compared to the RRC_IDLE state. However, the RRC_INACTIVE state is rarely activated in commercial networks, offering valuable insights for future development. While 6G is considering optimization of INACTIVE state design, the 5G IDLE/INACTIVE related energy saving techniques can serve as a good starting point for the 6G study.

Power consumption for UE in IDLE/INACTIVE state is primarily attributed to paging monitoring and RRM measurements. To mitigate this, 5G introduced features such as RRM measurement relaxation and the use of TRS to reduce power consumption. In 5G, UEs in IDLE/INACTIVE state are allowed to relax RRM measurements of neighbouring cells under conditions of low mobility or when not located at the cell edge. Additional techniques with similar objectives can be combined with RRM measurements. For example, configuring TRS for UEs in IDLE/INACTIVE state enables further power savings by allowing the UE to skip certain SS/PBCH block measurements and enter deeper sleep modes.

In 6G system, RAN2 can take RRM measurement relaxation mechanism specified in 5G as a starting point, and further study new requirements and corresponding enhancements for RRM and mobility related functions, including potential relaxation of serving and/or neighbouring cell RRM measurements.

Proposal 2: RAN2 to take RRM measurement relaxation mechanism specified in 5G as a starting point and further study new requirements and corresponding enhancements for RRM and mobility related functions, including potential relaxation of serving and/or neighbouring cell RRM measurements.

In the RRC_CONNECTED state, UE power consumption stems primarily from PDCCH monitoring. To address this issue, the C-DRX mechanism allows the UE to periodically wake up to monitor the downlink control channel and receive data when available. This mechanism effectively lowers the average UE power consumption during data transmission. However, it has been observed that merely adjusting C-DRX configuration parameters often fails to achieve an optimal trade‑off between latency and power efficiency. The current C-DRX pattern remains relatively static; when a downlink scheduling opportunity occurs while the UE is in its inactive period, data reception must wait until the next active cycle, thereby increasing latency.

For 6G systems, introducing AI/ML‑assisted prediction for adaptive C-DRX operation could be highly beneficial. By learning and predicting UE traffic patterns, AI/ML models can dynamically and more accurately activate the UE, enabling more precise alignment with actual data arrivals. This approach would enhance overall energy efficiency without compromising latency performance.

Observation 2: In 5G, the C-DRX mechanism is semi-static and lacks adaptability to varying traffic patterns, limiting its efficiency in balancing latency and UE power consumption. To introduce AI/ML-assisted prediction mechanisms for adaptive C-DRX operation can achieve a better balance between latency performance and UE power consumption.

2.2 Network Energy Saving

5G has introduced several mechanisms to improve network energy efficiency, including On-Demand SSB, On-Demand SIB1, SSB-less operation, and C-DRX. These features allow the network to dynamically control broadcast transmissions and reduce power consumption. Specifically, 5G supports on-demand SSB transmission for secondary cells (SCells), as well as on-demand SIB1 transmission triggered by network commands or a Wake-Up Signal (WUS). These capabilities enhance energy efficiency by extending the default SSB periodicity or confining additional SSB transmissions to short active windows. However, the continued requirement for periodic CD-SSB transmission limits potential energy savings. In current 5G implementations, OD-SSB is only applicable to Cell A, which inherently restricts its energy-saving impact. PCells still must transmit SSBs in an "always-on" manner, which also affects UE synchronization performance. Similarly, OD-SIB1 is only supported on anchor or non-cell defining secondary cells, leading to scenarios where SIB1 is broadcast without any UE receiving it, resulting in unnecessary energy waste. For 6G, it would be beneficial to extend OD-SSB and OD-SIB1 applicability to additional cell types and deployment scenarios while maintaining backward compatibility with 5G. Dynamic and on-demand signalling mechanisms should be designed to support all cell types and enable efficient standalone operation, allowing broadcast transmissions to be flexibly controlled based on actual network activity. Furthermore, supporting OD-SSB and OD-SIB1 across all RRC states could help maximize energy efficiency throughout the network.

Proposal 3: RAN2 may study the extension of OD-SSB and OD-SIB1 mechanisms to a wider range of cell types and deployment scenarios. It is desirable to define dynamic and on-demand signaling frameworks applicable to all RRC states to enhance network energy efficiency and enable efficient standalone operation.

In the frequency domain, additional energy savings can be realized through carrier aggregation in conjunction with SSB-less SCells, thereby minimizing always-on transmissions. UE can perform time and frequency synchronization as well as automatic gain control using SSB signals received from the PCell or other SCells, while SSB-less SCell operation maintains synchronization and measurement capabilities with reduced signalling overhead. By allowing inactive SSB-less SCells to remain in a dormant state for extended periods, overall network energy consumption can be further reduced. To enhance efficiency, extending the default SSB periodicity—for example, from 20 ms to 160 ms—and restricting always-on signals to a limited subset of carriers could be considered. This design offers an improved balance between operational simplicity and energy conservation, particularly when implemented alongside on-demand SSB mechanisms in other carriers.

Proposal 4: To minimize the need for always-on signal configuration by supporting flexible carrier-level configurations that do not require continuous SSB transmission on all carriers. The interaction between such flexible configuration and on-demand SSB operation can be further evaluated to improve network energy efficiency.

5G NR introduced Cell DTX/DRX mechanisms, enabling base stations to dynamically suspend transmission and reception during inactive intervals and enter sleep mode, thereby reducing power consumption. When the network is in an inactive or sleep state, the UE can also suspend certain transmission and reception activities, contributing to improved UE energy efficiency. However, in 5G NR, Cell DTX/DRX has not been extended to RRC_IDLE or RRC_INACTIVE states. It would be valuable to evaluate whether supporting Cell DTX/DRX in these states could yield additional energy savings for both the network and UEs. Moreover, the current Cell DTX/DRX is primarily a network-side energy saving feature, with limited coordination with UE-side DRX operation. This lack of alignment, combined with heterogeneous UE capabilities, may lead to interoperability challenges. In addition, trade-offs between low latency and QoS have not been fully optimized. Building on existing mechanisms, it would be beneficial for RAN2 to study approaches that allow UEs to provide input or suggestions to the network for optimizing Cell DTX/DRX configurations, thereby enabling more intelligent and coordinated energy saving.

Observation 3: Coordination between UE-level C-DRX and Cell-level DTX/DRX has not been fully explored, and their joint operation could further enhance overall energy efficiency.

2.3	Unified and Coordinated energy saving framework in 6G

Based on the analysis of the aforementioned energy-saving techniques, it is crucial for 6G research to ensure proper coordination and interoperability between UE and network energy-saving mechanisms. The coexistence of multiple mechanisms targeting similar objectives—such as C-DRX, PEI, and LP-WUS—can result in functional redundancy and reduced efficiency. Therefore, avoiding overlapping functionalities and ensuring complementarity among these mechanisms will be essential to achieve consistent and effective energy-saving behaviour in 6G. In particular, the coordination between Cell DTX/DRX and UE DRX is highly valuable. Aligning C-DRX with Cell-level DTX/DRX can enable temporal synchronization of sleep cycles between the UE and the network, thereby facilitating joint energy savings. Furthermore, integrating C-DRX with lower-layer dynamic mechanisms—such as search space configuration or adaptive BWP switching—can improve operational flexibility. That said, such cross-layer coordination introduces additional design complexity and warrants further study within RAN2.

Proposal 5: To further study the coordination between Cell DTX/DRX and UE DRX in 6G, including mechanisms that enable the UE to provide optimization feedback to the network for improved Cell DTX/DRX configuration.

Additionally, many existing energy-saving mechanisms are designed to operate only in specific RRC states, which constrains their overall applicability. For 6G, the design of joint UE–network energy-saving mechanisms should aim to establish a unified and coordinated framework based on common principles applicable across all RRC states. The goal is to minimize unnecessary power-consuming transmission and reception activities, maintain sleep synchronization between the UE and the network, and maximize overall energy efficiency—without compromising user experience or network performance.

Finally, to ensure a smooth evolution from 5G to 6G, future 6G networks should maintain compatibility with existing 5G terminals. New features introduced in 6G are expected to be applicable to Release-19 and earlier UEs, while offering scalability across different device types to support diverse deployment scenarios and a wide range of device capabilities.

Proposal 6: To study a unified and coordinated energy saving framework for 6G that ensures interoperability between UE and network energy saving mechanisms, minimizes redundant functionalities, supports common operation across all RRC states, and maintains backward compatibility with Rel-19 and earlier UEs while enabling scalability across different UE types.

- Conclusions

Based on the discussion in the previous sections, we propose the following:

Observation 1: In 5G NR, energy saving mechanisms were designed separately for UE and network, which resulted in limited coordination between the two sides and consequently constrained overall energy efficiency.

Proposal 1: To joint UE-network energy saving must be a high priority in 6G SI phase to develop a holistic framework where network operations and UE power saving mechanisms are seamlessly integrated to maximize overall system efficiency.

Proposal 2: RAN2 to take RRM measurement relaxation mechanism specified in 5G as a starting point and further study new requirements and corresponding enhancements for RRM and mobility related functions, including potential relaxation of serving and/or neighbouring cell RRM measurements.

Observation 2: In 5G, the C-DRX mechanism is semi-static and lacks adaptability to varying traffic patterns, limiting its efficiency in balancing latency and UE power consumption. To introduce AI/ML-assisted prediction mechanisms for adaptive C-DRX operation can achieve a better balance between latency performance and UE power consumption.

Proposal 3: RAN2 may study the extension of OD-SSB and OD-SIB1 mechanisms to a wider range of cell types and deployment scenarios. It is desirable to define dynamic and on-demand signaling frameworks applicable to all RRC states to enhance network energy efficiency and enable efficient standalone operation.

Proposal 4: To minimize the need for always-on signal configuration by supporting flexible carrier-level configurations that do not require continuous SSB transmission on all carriers. The interaction between such flexible configuration and on-demand SSB operation can be further evaluated to improve network energy efficiency.

Observation 3: Coordination between UE-level C-DRX and Cell-level DTX/DRX has not been fully explored, and their joint operation could further enhance overall energy efficiency.

Proposal 5: To further study the coordination between Cell DTX/DRX and UE DRX in 6G, including mechanisms that enable the UE to provide optimization feedback to the network for improved Cell DTX/DRX configuration.

Proposal 6: To study a unified and coordinated energy saving framework for 6G that ensures interoperability between UE and network energy saving mechanisms, minimizes redundant functionalities, supports common operation across all RRC states, and maintains backward compatibility with Rel-19 and earlier UEs while enabling scalability across different UE types.

- References

- RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator), Prague, Czech Republic