3GPP TSG-RAN WG2 Meeting #132        											                          R2-2508991

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Remaining open issues for CB-Msg3-EDT

Document for:	Discussion and Decision

# Introduction

In RAN2#131bis, there are still open issues for CB-Msg3-EDT procedure. In this paper, we discuss the remaining issues and provide our suggestions.

# Discussion

## 2.1 MPDCCH narrowband

According to the incoming LS R2-2508014/R1-2508114 [1] from RAN1, the support of 2 MPDCCH narrowbands request additional RAN1 work.

## Support narrow band MPDCCH configuration for CB-Msg3-EDT RAN2 ask RAN1 to check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed. RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT: - From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

From our perspective, it would be good to avoid RAN1 impact at this moment. Although RAN1 has no concern to support two MPDCCH narrowbands, it requests further RAN2 discussion on how to ensure the UE is monitoring a unique narrowband. So, we prefer NOT to support this.

The ASN.1 could be simply modified as

CB-MSG3-MPDCCH-Config-r19 ::=		SEQUENCE {

mpdcch-Narrowband-r19			SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r19		SEQUENCE{

numberPRB-Pairs-r19				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r19		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r19		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-CSS-r19			ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

mpdcch-Offset-CSS-r19			ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

Proposal 1: RAN2 to define the MPDCCH narrowband configuration for CB-Msg4 monitoring as a single value, not a set (i.e. revert previous RAN2 agreement and define only one MPDCCH narrowband value).

## 2.2 CQI and AS RAI reporting in Msg3

In last meeting, RAN2 discussed whether AS RAI could be sent in CB-Msg3 for NB-IoT, but there is no conclusion.

- CQI and AS RAI reporting in Msg3 R2-2507242 Issues on early termination of CB-Msg3-EDT without RRC message Google discussion Rel-19 IoT_NTN_Ph3-Core Observation 1 NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement). Proposal 1 NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB. - QC thinks this is already possible. Google thinks this is only clear for connected mode - Nokia and Nordic think we could use the NAS procedure. - Come back in the next meeting to decide whether NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB

The triggering of AS RAI reporting is specified in 36.321 clause 5.4.8. From our reading, there is no limitation in the SPEC saying that NB-IoT cannot trigger AS RAI in Msg3 (or CB-Msg3). The AS RAI is triggered by upper layer and it could be included in Msg3 when rai-ActivationEnh is configured in SIB2-NB.

5.4.8	Access Stratum Release Assistance Indication

Access Stratum Release Assistance Indication is used to provide the serving eNB with information whether subsequent DL or UL transmission is expected. AS RAI uses the DCQR and AS RAI MAC Control Element. Upper layers trigger AS RAI.

For EDT and transmission using PUR, if AS RAI is triggered by upper layers but is not included in the resulting MAC PDU with the MAC SDU as a result of logical channel prioritization, AS RAI is cancelled, for other transmissions if AS RAI is not included in the resulting MAC PDU as a result of logical channel prioritization, AS RAI may be cancelled.

If rai-Activation is configured and a buffer size of zero bytes has been triggered for the BSR and no subsequent DL and UL data transmission is expected, and if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], it is up to UE to send BSR MAC control element or DCQR and AS RAI MAC control element.

Note that some companies think that the text in 36.321 clause 5.25 prevents NB-IoT UE from sending AS RAI. But this is not correct. 5.25 is for triggering of channel quality reporting, not AS RAI. Although the downlink channel quality report and AS RAI use the same MAC-CE, the triggering condition of these two reports are different.

5.25	 Transmission of Downlink Channel Quality Report

The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur:

-	DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR";

-	for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR".

Therefore, it should be clear that AS RAI could be sent in Msg3.

Proposal 2: RAN2 confirms that, when rai-ActivationEnh is configured in SIB2-NB, current SPEC allows the NB-IoT UE to transmit “DCQR and AS RAI MAC CE” in Msg3 or CB-Msg3 for AS RAI purpose. (No SPEC change is needed).

## 2.3 Backoff value

It was agreed in last meeting to define the back off parameter value in multiples of the CB-Msg3-transmission window periodicity. The value range is FFS.

Details of Backoff scheme for CB-Msg3-EDT

R2-2507642    MAC IoT NTN issues           Ericsson               discussion            Rel-19    IoT_NTN_Ph3-Core

Proposal 1           The back off index 0 to 15 indicate a back off parameter value of 3, 6, 9,… 45, 48 multiples of the CB-Msg3-transmission window periodicity.

-      QC thinks the existing way is also fine and we could stick to that. HW agrees

-      MTK supports this because it’s good to align the attempts to the transmission window boundaries

-      Nokia thinks we could go for option B1 in NEC paper

-      NEC prefers Ericsson proposal (or A2 in their paper)

-      Samsung is fine as long as the final Backoff value is interpreted in ms

-      Xiaomi supports this proposal

- The back off index 0 to 15 indicate a back off parameter value of [FFS] multiples of the CB-Msg3-transmission window periodicity.

R2-2507656    Corrections for CB-MSG3 EDT         MediaTek Inc.    CR          Rel-19    36.321    19.0.0    1599      1             F             IoT_NTN_Ph3-Core

- Can further check the values in table 7.2-x: Backoff Multification Factor for CB-Msg3-EDT

- Endorsed

The BI field has 4 bits, 16 values. We think it is more nature to a value as multiples of 2. In the begging, the difference between two adjacent indexes could just be 2 transmission windows. The difference could become 4 transmission windows to cover more range in later indexes. A maximum value 40 transmission window periodicity should be long enough. With this, we suggest to have the value range defined as in below table.

Table 1: Backoff Multiplication Factor for CB-Msg3-EDT.

| Index | Backoff multiplication factor |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 10 |
| 5 | 12 |
| 6 | 16 |
| 7 | 20 |
| 8 | 24 |
| 9 | 28 |
| 10 | 32 |
| 11 | 36 |
| 12 | 40 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Proposal 3: The back off index 0 to 12 indicates a back off parameter value of {0, 2, 4, 8, 10, 12, 16, 20, 24, 28, 32, 36, 40} multiples of the CB-Msg3-transmission window periodicity. The back off indexes 13 to 15 are reserved for future usage.

# Conclusion

Proposal 1: RAN2 to define the MPDCCH narrowband configuration for CB-Msg4 monitoring as a single value, not a set (i.e. revert previous RAN2 agreement and define only one MPDCCH narrowband value).

Proposal 2: RAN2 confirms that, when rai-ActivationEnh is configured in SIB2-NB, current SPEC allows the NB-IoT UE to transmit “DCQR and AS RAI MAC CE” in Msg3 or CB-Msg3 for AS RAI purpose. (No SPEC change is needed).

Proposal 3: The back off index 0 to 12 indicates a back off parameter value of {0, 2, 4, 8, 10, 12, 16, 20, 24, 28, 32, 36, 40} multiples of the CB-Msg3-transmission window periodicity. The back off indexes 13 to 15 are reserved for future usage.

# References

[1] R2-2508014/R1-2508114, “Reply LS on power ramping and RRC configuration for CB-Msg3-EDT”