3GPP TSG-RAN WG2 Meeting #132														R2-2509012

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:		10.3.3.1

Souce:	Samsung

Title:					Considerations for 6G data transfer

Document for:		Discussion

1. Introduction

In RAN#108, a study item on the next generation of non-backward compatible Radio Access Network (RAN), i.e., 6G Radio (6GR), was approved 6G SID [1].

<6G_SID [1]> ### 4.1 Objective of SI or Core part WI or Testing part WI <…> The detailed objectives of the study are: <…> - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Within the scope of data transfer, RAN2 had the first meeting on 6GR in RAN2#131bis and made the following agreements.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

Based on the SID and the last agreements, this contribution discusses on data transfer design to support various types of data (e.g., AI/ML and sensing) in 6GR.

2. Discussion

2.1 Service plane in 6G

In 5G, there have been various types of data for services (i.e., AI/ML, SON, MDT, QoE, positioning). AI/ML data for UE-side and NW-side model training has been studied and standardized in Rel-19 for different use-cases (i.e., beam management, CSI prediction, positioning), and data for AI/ML mobility is supposed to be discussed and specified in Rel-20. Meanwhile, RAN3 has specified the data transfer for AI/ML for NG-RAN. Besides, legacy SON, MDT, QoE and positioning features have been introduced and evolved during a few of 5G releases. For these 5G data, we would like to note some issues in 5G data transfer framework:

- Most of 5G data (i.e., AI/ML, SON, MDT, QoE and positioning) are collected via control plane which inherently has scalability issue with large size of data volume. Besides, CP based data transfer may interfere essential CP signaling (e.g., RRC connection management, mobility management, session management).

- Meanwhile, Positioning WI adopted UP solution as well for UE to LMF signaling. However, UP-based data has to pass through UPF before data collection entity, leading to inefficient data path. Moreover, it is transparent to gNB, so gNB cannot be involved for the data transfer and management (when needed). Besides, the data transfer over UP is terminated in application layers which are not in control of 3GPP.

- In 5G, a separate data transfer framework for each of those services has been standardized, whenever each feature was introduced, throughout different releases. This has led 5G to have diverged frameworks for data transfer and management.

Observation 1. 5G data transfer over CP-based framework is not suitable to support large volume of data and has the risk of degrading the existing essential CP signaling.

Observation 2. 5G data transfer over UP-based framework has to go through UPF unnecessarily and gNB cannot be involved for data transfer and management (when needed). Besides, the data transfer over UP is terminated in application layers which are not in control of 3GPP.

Observation 3. 5G data transfer ended up introducing diverse frameworks to support various data types.

In 5G, diverse frameworks in data gathering have introduced significant complexity and inefficient management of data. In 6G, more types of AI/ML data could emerge based on new use-cases to be specified. Besides, sensing data is to be studied under 6G SI, and other legacy data types (i.e., SON, MDT, QoE, Positioning) could be adopted and enhanced throughout 6G phase. We may need to introduce more data types for different 6G services in different releases. Therefore, it is crucial to design data transfer/management framework to encompass various types of data from beginning of 6G. Besides, unlikely to 5G CP-solution, it should be able to accommodate large volume data transmission and different QoS requirements for different services. Unlikely to 5G UP-solution, 6G framework should be able to support efficient data path and potential sNB (i.e., 6G RAN) involvement (if needed). Therefore, we propose a generic framework over a new data plane, called “Service Plane”. We assume service plane is dedicated to data transfer for at least the new services captured in SID (i.e., AI/ML and sensing) to avoid any impact on legacy CP or UP operation. RAN2 can check further whether radio features to support service plane could be used for other services (e.g., SON, MDT, QoE, Positioning, and new 6G services). Introduction of service plane could improve data transfer/management efficiency and eliminate the need for defining different data transfer mechanisms in subsequent releases.

Figure 1. Example of architecture for data transfer

Proposal 1. Aim to a generic framework for various types of 6G data (e.g., AI/ML, sensing).

Proposal 2. Study a new data plane, called “Service Plane” to support a generic framework for various 6G data (e.g., AI/ML, sensing) transfer.

Meanwhile, SA2 has the similar motivation, and so is studying whether/how to develop a generic mechanism for operator services or data framework to support the various services (e.g., AI/ML and sensing), as specified the new approved 6G SID in SA2 [2].

<6G SID in SA2 [2]> WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network: NOTE 1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services. <…> WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework. NOTE 6: The work split with SA3, SA5 and RAN WGs will require TSG coordination

We propose to focus on RAN2 impact by introduction of service plane. One of RAN2 impact we assume is whether/how a new radio bearer is used for transfer of service plane data. At the same time, we should avoid or minimize its impact to traffic on existing RB(s). Besides, we can also discuss on which network entities (e.g., gNB or CN) can have access and/or controllability to the transferred data. Correspondingly, there could be necessary interaction between UE to the network entities (e.g., gNB or CN). Also, RAN2 should study whether/how to support security for those data.

The next issue could be what use-case(s) or service(s) would be supported within this service plane. We assume aforementioned all use-cases (i.e., AI/ML, SON, MDT, QoE, positioning, and new 6G services) could be regarded as candidates using the service plane, in the perspective of RAN2. However, other WGs (e.g., RAN3, SA2, SA5) are also responsible for designing framework architecture and they are also studying their own use-cases and corresponding requirements. So, as for now, it is not easy for RAN2 to discuss whether/how use-case(s) related to other WGs could be supported within this new framework. Hence, we suggest RAN2 to focus on the use-cases confirmed by RAN1/RAN2 in 5G as baseline (i.e., AI/ML for air interface, AI/ML for mobility, sensing) in order to make discussion progress on design of service plane, especially for Uu impact.

Lastly, all these aforementioned aspects to be studied in RAN2 are highly dependent to progress in other WGs (e.g., SA2, RAN3, SA5). So, close and timely coordination with them is essential.

Proposal 3. Study the following RAN2 impact from the introduction of service plane, in close and timely coordination with SA2, RAN3, and SA5.

- Uu impact, including whether/how to introduce new RB(s) while minimizing impact to traffic on existing RB(s)

- How to support NW-side visibility/controllability for data transfer

- How to support UE-sNB and/or UE-CN interaction

- Whether/how to support security for 6G-supported data

- Whether/what other use-cases (e.g., SON, MDT, QoE, Positioning, or new 6G services) can be supported over service plane, assuming AI/ML for air interface, AI/ML for mobility, and sensing as baselined use-cases

2.2 AI/ML specific data transfer

In this section, we would like to discuss AI/ML data transfer separately. In 5G Rel-19, data transfer framework for NW-side model training (i.e., NW-side data collection) has been standardized. Beam management use-case was the only use-case for NW-side model. For NW-side data collection, UE configured by NW measures beam resources (i.e., CSI-RS or SSB) and logs measured data (i.e., beam ID and corresponding L1-RSRP). Then, UE reports the logged data via RRC UEInformationResponse message to NW. As discussed in Observation 1, CP solution including RRC reporting has drawbacks, we assume NW-side data collection could be enhanced over service plane.

Meanwhile, regarding data transfer framework for UE-side model training (i.e., UE-side data collection), three options (i.e., Option 1a, Option 2 and Option 3) have been studied in 5G Rel-19 and captured in TS 38.843 [3].

- Option 1a: No standard impact on transfer of data collected in UE. i.e., UE transfers data to UE server in a non-standardized way (i.e., transparent to 3GPP network)

- Option 2: Standardized data transfer from UE to UE server via CN

- Option 3: Standardized data transfer from UE to UE server via OAM

In Rel-19, only Option 1a is assumed. Additionally, Option 2 and 3 could be further studied and specified (if deemed necessary) as captured in 5G Rel-20 WID [3],

<5G Rel-20 WID [3]> ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: <…> Data collection [RAN2]: Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

In 6G, standardized UE-side data collection solution(s) (like Option 2 and 3 in 5G) via service plane could be studied and specified. Here, we would like to clarify Option 1a should not be precluded in 6G, as in 5G i.e., UE can transfer collected data to UE server in a non-standardized way (i.e., transparent to 3GPP network). This is needed for UE server to collect data from UEs without relying on NW’s involvement, and it is essential to avoid leakage of UE proprietary data. Besides, Option 1a could be useful as a complementary solution when a standardized solution is not supported by NW.

Proposal 4. Data transfer over service plane supports both NW-side data collection and standardized UE-side data collection for AI/ML model training.

Proposal 5. Option 1a in TR 38.843 is not precluded for UE-side data collection as a complementary/alternative data transfer solution for standardized UE-side data collection.

3. Conclusion

Based on our discussion above, the following observations and proposals are suggested:

Observation 1. 5G data transfer over CP-based framework is not suitable to support large volume of data and has the risk of degrading the existing essential CP signaling.

Observation 2. 5G data transfer over UP-based framework has to go through UPF unnecessarily and gNB cannot be involved for data transfer and management (when needed). Besides, the data transfer over UP is terminated in application layers which are not in control of 3GPP.

Observation 3. 5G data transfer ended up introducing diverse frameworks to support various data types.

Proposal 1. Aim to a generic framework for various types of 6G data (e.g., AI/ML, sensing).

Proposal 2. Study a new data plane, called “Service Plane” to support a generic framework for various 6G data (e.g., AI/ML, sensing) transfer.

Proposal 3. Study the following RAN2 impact from the introduction of service plane, in close and timely coordination with SA2, RAN3, and SA5.

- Uu impact, including whether/how to introduce new RB(s) while minimizing impact to traffic on existing RB(s)

- How to support NW-side visibility/controllability for data transfer

- How to support UE-sNB and/or UE-CN interaction

- Whether/how to support security for 6G-supported data

- Whether/what other use-cases (e.g., SON, MDT, QoE, Positioning, or new 6G services) can be supported over service plane, assuming AI/ML for air interface, AI/ML for mobility, and sensing as baselined use-cases

Proposal 4. Data transfer over service plane supports both NW-side data collection and standardized UE-side data collection for AI/ML model training.

Proposal 5. Option 1a in TR 38.843 is not precluded for UE-side data collection as a complementary/alternative data transfer solution for standardized UE-side data collection.

4. Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] SP-250806, Study on Architecture for 6G System, SA WG2

[3] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm