3GPP TSG-RAN WG2 #132                                        	    R2-2509014

Dallas, TX, USA, November 17-21, 2025

Agenda item:	   10.3.2.2

Source: 		Qualcomm Incorporated, MediaTek Inc.

Title: 	RRC configuration for flexible and adaptive UE behaviour

Document for:	Discussion

# 1	Introduction

The Study Item for 6G has objectives related to the integration of AI/ML to the 6G protocols as follows ([1]):

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures [RAN2, RAN1, RAN3, RAN4]

    - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Adoption of extensive AI/ML is considered as one of the most important and distinguishing factors of 6G compared to previous generations of cellular networks. In general, this also includes usage of any types of smart algorithms which can improve the performance. From UE perspective, this can entail the usage of internal information and processing capability to adapt its behaviour to the dynamic conditions of the radio access. With this understanding, we consider UE AI/ML as an umbrella term for all “smart” UE implementation, regardless of which type of algorithm the UE uses.

In this paper, we discuss some methods towards the goal of enabling the UE to use its internal resources and capabilities to improve the performance of radio access and propose areas for study at Radio Protocol levels.

# 2	Discussion

As mentioned above, 6G will aim to support AI/ML and smart algorithms across an extended set of functionalities and features. In NR, there have been ongoing activities on AI/ML since Rel-18 for certain features. However, with 6G, the goal will be to support such functionality on Day 1 and make it an essential part of the system. This does not imply that 6G will require AI/ML models for its basic functionality but rather it will employ smart algorithms in order to provide better performance and efficiency.

In NR, the efforts to introduce AI/ML were accomplished by analysing certain use cases (e.g. CSI compression, beam prediction) during a Study Item phase and by comparison of legacy and model-based methods via simulations. A significant amount of time was also spent on how data can be collected to train AI/ML models at the UE and NW side and how the models and inference can be enabled and disabled.

In discussions that happened in many 6G-related forums and meetings prior to the 3GPP Study Item, the term “AI-Native” was also used by many companies as an essential component of 6G. This term does not have a clear or specified definition yet and thus can be interpreted in various ways.

In our understanding, “AI-Native” means that the AI/ML functionality or similar smart algorithms should be integrated to 6G protocol layers in Day 1. This way, the evolution of AI/ML with the related technological advances will help the improvement of 6G.

Observation 1: The term “AI-Native” as part of 6G is not clearly defined; however, a reasonable interpretation is for “smart” UE algorithms to be an integral part of the 6G system and protocol stack.

An important distinction to be made is that “AI-Native” or “AI/ML” does not necessarily mean that the 6G system should use LLM or a specific AI model. The “model” itself is a black-box and how the NW nodes (UE and RAN/CN nodes) use computational algorithms from a set of inputs to an output is purely an internal implementation. What really matters is the actual output of the model or algorithm as the UE and RAN will act on this output.

With this background, we can now look at how cellular networks have employed or addressed the usage of algorithms and in particular on the relationship between the UEs and the NW.

The fundamental paradigm of 3GPP defined systems (up until NR) relies on NW having a large degree of control over the UE behaviour. This is achieved by NW configuring the UE with a set of parameters and values. The rationale of this philosophy has been that it provides a predictable UE behaviour. Furthermore, 3GPP also utilizes requirements and tests which ensures that the UE behaviour is as expected.

Observation 2: Cellular networks until now have relied on NW configuration of the UE with a single set of parameter values, aimed at a very predictable UE behaviour.

In recent years, this paradigm has started to shift in order to accommodate the usage of AI/ML, especially at the UE side. 3GPP has embarked upon introducing AI models which the UE can use to change its operating conditions dynamically (e.g. RRM, Mobility) in order to improve the UE and system performance. Since it is impossible for the NW to be aware of every AI/ML algorithm or model used by each UE, the control mechanism has also changed where the NW monitors the UE’s behaviour or performance and decides whether to let the UE to use AI/ML models.

Observation 3: With the introduction of AI/ML models in NR-Advanced, 3GPP is now adopting a performance monitoring-based control of the UE behaviour.

We think this trend and change of the paradigm is the right thing to do to benefit from the rapidly changing AI technologies while at the same time providing a good path to improve 6G going forward.

Here, we will focus on what can be done in terms of further utilizing the processing and reasoning capabilities at the UE. It is a well-known fact that the UEs now have a significant amount of processing capability which can improve not only the HLOS applications (as they are mostly used for nowadays) but also the cellular connectivity.

Observation 4: The processing capabilities of the UEs have increased significantly in the recent years and this can be utilized to improve the performance of cellular connectivity.

A second and related part is the collection of vast amounts of data at every level of the cellular connections. The UE vendors now have access to this data, which they can analyse to improve their internal algorithms.

Observation 5: The UEs are capable of collecting and analysing vast amounts of data for self-learning and improvement.

In 3GPP studies on AI/ML so far, one of the most controversial and partly unresolved issues are related to data collection and model management. Even if 3GPP ends up introducing standardized methods for these functionalities, it remains to be seen how much of these will be deployed. This can delay or even hinder the usage of AI/ML at the UE side if such implementations will require deployment of all the related NW nodes and functionalities to train and deploy AI/ML models.

We suggest that 3GPP should also study how UE internal algorithms can be used (whether based on AI/ML models or not) under network control and guidance without absolutely relying on a full model management framework. We believe that this can also expedite the deployment of such algorithms.

Proposal 1: RAN2 study how to enable “flexible” protocols at Control and User Plane levels which allow the UE to use internal algorithms for better performance but do not rely on the AI/ML model management framework.

One promising approach we consider is to adapt the configured RRC parameters to the changing environment by the UE. Over the years, we have observed that, in existing deployments, the NW configuration of the UE is usually static, which means that the NW usually does not change the value of a parameter once it is configured. In addition, the values are almost always same for all UEs, despite them having very different capabilities and conditions. And when problem observed, there is usually considerable delay to update RRC parameters.

Observation 7: Existing deployments almost always use the same set of parameter values for all UEs with little differentiation for the dynamic conditions and capabilities of the UEs, which can result in sub-optimal performance for both the UE and the system and poor user experience for indefinite period of time.

In contrast, the UE is aware of its conditions and surroundings constantly and can even incorporate data from sensors at the device. In addition, the UE can leverage the data collected in the past and fingerprinting of the areas and cells. This gives the UE the ability to determine the best choices for the parameter values. In the Annex, we show examples for these types of UE optimizations as an illustration for the proof of concept.

Observation 8: The UE can adapt the parameter values dynamically based on environmental, application, and radio conditions to improve the user experience.

We do not assume that the UEs can change a value to any arbitrary value it selects as this may disrupt the overall network. Therefore, there should be a mechanism where the NW determines the bounds of such changes. For example, the NW can configure a set of values which is allowed for a certain parameter.

Proposal 2: RAN2 study a mechanism which can allow the UE to adapt values of RRC configured parameters under network supervision.

We should note that performing a similar optimization for each UE at the NW side will be costly and not practically feasible. Each UE reporting all the data which can be considered for such optimization will incur a large overhead. Furthermoe, configuring the UE frequently to perform a change to every parameter value requires additional signalling as well as reconfiguration delay. The UEs can also use device specific algorithms which may not be available or suitable for NW side implementation.

It is well understood that an optimal choice for one particular UE may not be best for the overall network, even if the UE stays within the bounds set by the NW. Therefore, it is important that the NW closely monitors the UE and system performance and makes adjustments (e.g. change the set of values or disable it completely) as needed. This is in principle not that different than monitoring the performance of AI/ML models at the UE and enabling/disabling an AI/ML model.

Observation 8: As in the AI/ML framework, the NW should be able monitor the UE performance and enable/disable the usage of such algorithms (in Proposal 2) by the UE.

Such monitoring and how much of it is needed and what granularity and frequency will of course depend on the use case and the parameters involved.

Proposal 3: To enable NW monitoring of the UE performance when the UE uses “smart” algorithms to adapt parameter values, RAN2 study KPIs and other metrics along with the applicable use cases and necessity and level of such reporting.

# 3	Conclusion

In this contribution, we discussed AI-Native Protocols for 6G and propose the following:

Observation 1: The term “AI-Native” as part of 6G is not clearly defined; however, a reasonable interpretation is for “smart” UE algorithms to be an integral part of the 6G system and protocol stack.

Observation 2: Cellular networks until now have relied on NW configuration of the UE with a single set of parameter values, aimed at a very predictable UE behaviour.

Observation 3: With the introduction of AI/ML models in NR-Advanced, 3GPP is now adopting a performance monitoring-based control of the UE behaviour.

Observation 4: The processing capabilities of the UEs have increased significantly in the recent years and this can be utilized to improve the performance of cellular connectivity.

Observation 5: The UEs are capable of collecting and analysing vast amounts of data for self-learning and improvement.

Proposal 1: RAN2 study how to enable “flexible” protocols at Control and User Plane levels which allow the UE to use internal algorithms for better performance but do not rely on the AI/ML model management framework.

Observation 7: Existing deployments almost always use the same set of parameter values for all UEs with little differentiation for the dynamic conditions and capabilities of the UEs, which can result in sub-optimal performance for both the UE and the system and poor user experience for indefinite period of time.

Observation 8: The UE can adapt the parameter values dynamically based on environmental, application, and radio conditions to improve the user experience.

Proposal 2: RAN2 study a mechanism which can allow the UE to adapt values of RRC configured parameters under network supervision.

Observation 8: As in the AI/ML framework, the NW should be able monitor the UE performance and enable/disable the usage of such algorithms (in Proposal 2) by the UE.

Proposal 3: To enable NW monitoring of the UE performance when the UE uses “smart” algorithms to adapt parameter values, RAN2 study KPIs and other metrics along with the applicable use cases and necessity and level of such reporting.

# References

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

# Annex

### RLC t-Reassembly – Current configuration & optimization opportunity

Observation: In current NR deployments, the RLC reassembly timer is often configured at a level that is too high

Over 30 ms > 75% of the time (see table of field data)

Most HARQ Tx and reTx finish much earlier than the time at which t-Reassembly expires

Tradeoff: Improved latency vs. performance loss due to redundant RLC transmissions

t-Reassembly set too high: Higher E2E latency & memory requirements

t-Reassembly set too low: Wasted channel resources because RLC NACK is transmitted before HARQ attempts are complete which results in unnecessary RLC retransmissions

Opportunity: UE can predict the t-Reassembly timer to improve latency while managing the wasted channel resources

Configured values in network deployments

| t-Reassembly value | >= 50 ms | >= 50 ms | >= 50 ms | >= 50 ms | >= 50 ms |
| --- | --- | --- | --- | --- | --- |
| Field data | ~9% | ~43% | ~26% | ~12% | ~10% |

Simulation Model

HARQ trace obtained from a single UE field data and used as an input to the simulation

Sub6 TDD (DDDSU), N77, 20MHz BW, 30kHz SCS, 1 CC  (RSRP -105 to -100dBm)

PDSCH HARQ trace from OTA testbed (BLER ~= 11%, rBLER ~= 0.5%)

1 UE in neighbor cell with periodic bursty traffic to create DL interference on DUT  abrupt SNR drop & HARQ failures

Interference traffic pattern:  30-40ms long burst every 2s

10Mbps XR DL video traffic (3GPP truncated Gaussian model) w/ 2 eye buffer @60fps

Results

| t-Reassembly | 5ms | 15ms | 25ms | 35ms | 65ms |
| --- | --- | --- | --- | --- | --- |
| 95%ile latency (ms) | 15 | 18 | 25.5 | 35.5 | 67.5 |
| RLC ReTx Increase (%) | 64.5 | 18 | 2.27 | 1.2 | 2.14 |
| Avg Resource Usage Increase (%) | 0.5% | 0.15% | 0.04% | 0.01% | 0.02% |

Observations:

- Reducing t-Reassembly from 35ms to 15ms leads to

  - Reducing the 95%ile latency by almost 50% (17.5ms)

  - Increases the resource usage OTA by only 0.15%

- In AI-native the NW can configure and UE can select the best operating point within the NW configuration

### PDCP t-Reordering – Current configuration & optimization opportunity

Observation: In current NR deployments, the PDCP reordering timer is configured at a level that is too high

Over 200ms > 93% of the time (see table of field data)

Tradeoff: Improved latency, throughput and  QoE vs some Out-of-Order delivery to the transport protocol

t-Reordering set too high: Higher E2E latency & memory requirements (possibly need for packet dropping especially for lossy channels)

t-Reordering set too low: Loss of late packets due to discarding

Opportunity: UE can predict the t-Reordering timer to improve latency while allowing allow “limited” Out-of-Order delivery relying on recovery at the transport layer

Configured values in network deployments

| t-Reordering value | 200ms | 300ms | 500ms - 3s | > 3s | 0ms |
| --- | --- | --- | --- | --- | --- |
| Field data | ~69% | ~10% | ~5% | ~9% | ~2% |

Simulation Assumptions

Simulation run with field trace data collected at cell edge (i.e., challenging RF conditions)

t-Reassembly = 25ms

t-StatusProhibit = 5ms

Allowed Out-of-Order delivery when a PDU arrives after t-Reordering timer expiry

Results

| t-Reordering | 0ms | 20ms | 30ms | 60ms |
| --- | --- | --- | --- | --- |
| 95% latency (ms) | 12 | 16.5 | 19.5 | 22 |
| Reduction in latency(%) | 55% | 25% | 11% | ____ |
| Out-of-Order delivery | 6.38% | 1.21% | 0.57% | 0 |

Observations

- Reducing t-Reassembly from 60ms to 20ms leads to

  - Reducing the 95%ile latency by almost 25% (5.5ms)

  - Results in out-of-order delivery at the transport layer of 1.21%

- In AI-native the NW can configure and UE can select the best operating point within the NW configuration

  - Allows the operator to achieve a better latency vs out-of-order delivery trade-off for an improved UX