3GPP TSG-RAN WG2 Meeting #132                                   R2-2509087

Dallas, USA, Nov. 17th - 21st

Agenda item:	9.2.2

Source:	TCL

Title:	Discussion on Topology 2 for Ambient IoT

Document for:	Discussion and Decision

1	Introduction

In the RAN2#131bis meeting [1], agreements were reached on AIoT resource allocation in TP2 as summarized below.

Agreements 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this contribution, we will share our views on the following aspects related to topology 2, including resource allocation and UE reader selection.

2 Discussion

In coordination with SA2, the RAN plenary has also updated the objective in the revised WID, as highlighted in yellow, as follows.

2. Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - Specify RRC-based architecture, in coordination with SA2. - The following are not supported:

Therefore, RAN2 should only consider the RRC-based solution to enable topology 2 for A-IoT in Rel-20.

For A-IoT topology 2, only the RRC-based solution is considered in Rel-20.

In Rel-19, for TP2 in Ambient IoT, it was agreed to configure dedicated resources to the UE Reader via dedicated signalling, as captured in TR [2] below.

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling.

Moreover, two alternatives for the radio resources were also discussed in Rel-19 [2]:

- Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

In the latest RAN2 meeting, it was agreed that Alternative 2, i.e., the resource allocation can be triggered by a CN request. However, In our views, Alternative 1 also facilitates a more efficient and precise resource allocation by the BS, using for the UE reader performing the A-IoT operations. This is because the UE Reader has a more direct understanding of the A-IoT interface conditions. For example, even if the quality of Uu interface between the UE reader and the BS is good, poor A-IoT interface conditions between the UE reader and the A-IoT devices may still result in a higher inventory failure rate, leading to longer completion time of the overall inventory procedure. Therefore, Alternative 1 allows it to autonomously decide when to initiate a resource request and the size of the required resources based on its actual situations. As for Alternative 2, gNB can only roughly estimate the required resources based on the received assistant information carried by a service request from CN.

Hence, from our perspective, it is recommended that Alternative 1 should be studied during Rel-20.

For the A-IoT radio resource allocation from BS to UE reader, RAN2 should also study the Alternative 1, where the UE Reader requests A-IoT radio resources from the gNB.

For TP2, it is beneficial that some assistance information is provided to BS, which can help BS assign the appropriate resources, improve resource efficiency. In our understanding, the assistance information may include A-IoT service type, proximate number of A-IoT devices, A-IoT device type, etc. etc. For example, if the A-IoT service types (e.g., inventory, command) are provided to BS, BS may know it requires the operation duration period, which is good for the BS to allocate sufficient resources. In another instance, the SID [2] has also approved three types of A-IoT devices with different capabilities, some of them may require different frequency bands to send messages over A-IoT interface. For example, Device 2, which transmits the D2R message and receives the R2D message, may need different bands.

It is suggested that RAN2 study the potential assistance information provided to the BS, such as the number of A-IoT devices, A-IoT device type.

During the Rel-19 discussion, RRC based solution, NAS based solution and UP based solution were identified to support Topology 2 for conveying A-IoT upper layer information, as summarized below.

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

The new WID [1] specifies an exclusively RRC-based architecture, for which the corresponding protocol stack is depicted in Figure 1.

Figure 1: RRC based solution of Topology 2

In this solution, the A-IoT-related message is carried in RRC signalling between the A-IoT-enabled UE reader and the A-IoT-enabled gNB. Take an example for the UL data transmission, the UE reader receives the A-IoT data from A-IoT device and forwards the A-IoT data to the gNB via RRC signaling, then the gNB transfers the A-IoT data to the A-IoT CN functions via XXAP. It can be observed that the gNB has the capability to decode the RRC message and XXAP message.

Based on the above discussion, the following Figure 2 is given to describe the stage 2 procedure for A-IoT inventory procedure in Topology 2 for RRC based solution.

Figure 2: Potential signalling procedure for A-IoT Inventory in Topology 2 (RRC-based solution)

1.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB over XXAP protocol.

2.	The A-IoT enabled gNB forwards the Inventory request to the A-IoT enabled UE by RRC message.

3. A-IoT enabled UE sends a A-IoT radio resource allocation request message to A-IoT enabled gNB, the message may include the number of AIoT devices.

4. A-IoT enabled gNB sends a A-IoT radio resource allocation response message to A-IoT enabled gNB

5. The A-IoT enabled UE performs inventory procedure in the new A-IoT interface using the allocated radio resource.

6. The A-IoT enabled UE sends the inventory report to the A-IoT-enabled gNB, which is received from A-IoT devices.

7. The A-IoT-enabled gNB sends the inventory report to the A-IoT-enabled CN.

For RRC-based solution, consider the above potential A-IoT Inventory in Topology 2 in Rel-20.

In the SA2 discussion, it has agreed that the CN function has capability to select the UE reader based on area/location information. While from RAN2 perspective, it is assumed that, at least for the RRC-based solution, there may be radio related reasons for BS involvement in UE Reader selection.

For RRC-based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

Therefore, considering that the CN-based approach to UE reader selection may prove insufficient due to the CN's inability to ascertain in a timely manner whether a UE is currently suitable to function as a reader. Hence, we think that UE reader selection with BS involvement should be considered during the WI phase.

In Rel-20, UE reader selection with BS involvement should be studied.

3	Conclusion

Based on the discussion in the previous sections, we propose the following:

- For A-IoT topology 2, only the RRC-based solution is considered in Rel-20.

- For the A-IoT radio resource allocation from BS to UE reader, RAN2 should also study the Alternative 1, where the UE Reader requests A-IoT radio resources from the gNB.

- RAN2 should to study the potential assistance information provided to the BS, such as the number of A-IoT devices, A-IoT device type.

For RRC-based solution, consider the above potential A-IoT Inventory in Topology 2 in Rel-20.

In Rel-20, UE reader selection with BS involvement should be studied.

References

RAN2#131bis, Chairman notes.

TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR.