3GPP TSG-RAN WG2 Meeting #132	R2-2509123

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	RRC and Stage 2 issues for MIMO

Document for:  Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, significant progress has been made in previous meetings. In RAN2 #131-bis, all the proposed RIL issues have been addressed, and several agreements for spec modification were achieved. Though the NR_MIMO_Ph5 work item was declared complete in RAN2, some necessary clarifications on RRC parameter definitions are needed as well as on Stage 2 details, for a better alignment with the expected functionality of the new feature.

This contribution provides additional analysis regarding the asymmetric mTRP and UEIBM procedure.

# 2	Discussion

## 2.1	RRC issues

### 2.1.1	[H407] startingBitOfFormat2-3-r19

In RAN1 #118-bis, about the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support

- Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

The above agreements have been captured in the MIMO RRC CR, as shown below:

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]],

[[

startingBitOfFormat2-3-r19            INTEGER (1..45)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-19 simultaneously.

The parameter startingBitOfFormat2-3-r19 uses the ‘-r19’ suffix. A ‘-r19’ suffix is normally reserved for defining a new version expecting to replace the old version. However, the range of the new field overlaps with the range of the existing field and the meaning is the same, regardless which version is used, so such a definition is wasting bits for nothing.

The associated UE capability was defined in 38.306 as:

| extendedStartBitDCI-2-3-r19 Indicates whether the UE supports the extended value range of starting bit of DCI format 2_3. A UE supporting this feature shall also indicate support of tpc-SRS-RNTI. NOTE: The starting bit of value range extends to X=45 for operations in FR1 shared spectrum or FR2-2 and X = 43 otherwise. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

In TS 38.212 v19.1.0, DCI format 2_3 is specified as:

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

There is no definition of a new field for startingBitOFormat2-3, and the definition of the UE capability is not whether the UE supports a new field called startingBitOfFormat2-3-r19, it is whether the UE supports a range extension for the existing field.

According to TS 38.331 v19.0.0, for a Rel-19 UE not supporting extendedStartBitDCI-2-3-r19, it seems possible for the network to signal the field startingBitOfFormat2-3-r19 set to a value from 1 to 31, but the UE that does not implement this feature may not understand it, and either ignore a part of the UE configuration, or even trigger reestablishment.

Observation 1: According to 38.331 v19.0.0, the network could signal the new Rel-19 field startingBitOfFormat2-3-r19 with a value set from 1 to 31 to any UE regardless of its capability, but some UEs may not support it and ignore the field, ignore DCI 2_3, or even trigger reestablishment.

Proposal 1: To avoid interoperability issues, change startingBitOfFormat2-3-r19 to startingBitOfFormat2-3-r19, defined as INTEGER (32..45).

### 2.1.2	pathlossOffset

TCI-UL-State field descriptions

pathlossOffset Indicates the pathloss offset applied to the CI tate of an UL-only TRP (see TS 38.213 [13], clause 7). Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

According to the agreement, in the MIMO RRC CR, the field description for pathlossOffset under TCI-State and TCI-UL-State IEs has been generalized, clarifying that the field applies to TCI state of an UL-only TRP. However, the use of "UL-only TRP" is a little problematic. For one thing, the term "TRP" is being phased out in favor of more precise terminology (e.g., "cell," "beam," "panel") in many parts of the specification. Using "TRP" in new or updated parameter descriptions is inconsistent with this general direction and can lead to ambiguity. For another, The term "UL-only TRP" is not used in TS 38.300 and the corresponding Work Item description, where it is officially named as "UL TRP". In our understanding, one solution is just to remove the word "only", changing the current description from "UL-only TRP" to "UL TRP", another solution is completely remove the phrase "UL-only TRP". The corresponding field description of the second solution is shown in Annex A.

Proposal 2: In the field description of pathlossOffset, change the description from "UL-only TRP" to "UL TRP", or completely remove the phrase "UL-only TRP".

### 2.1.3	minimumPucch-PuschOffset

In the current RRC spec, the field description of minimumPucch-PuschOffset in CSI-ReportUE-Initiated is shown below. It states that “Value sym0 corresponds to 0, value sym1 corresponds to 1, and so on.” According to the legacy field description of “sym0” and “sym1”, the current description should be modified for alignment. Thus, the field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

CSI-ReportUE-Initiated field descriptions

minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting. Value sym0 corresponds to 0, value sym1 corresponds to 1 and so on.

SL-Unlicensed field descriptions

sl-StartingSymbolFirst Indicates the location of first starting symbol within a slot. Value sym0 corresponds to first symbol, value sym1 corresponds to the second symbol and so on. If the field is not configured, the UE shall use value sym0.

Proposal 3: Thus, the field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

The corresponding TP is provided in the Annex A.

### 2.1.4	[H405] CBSR List

The ASN.1 definition of CodebookConfig is rather a long and some of its new pieces include a lot of repetition:

CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))},

two-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (64))},

four-one-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (16))},

three-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (96))},

six-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (24))},

four-two-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (128))},

eight-one-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (32))},

four-three-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (192))},

six-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (192))},

twelve-one-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (48))},

four-four-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (256))},

eight-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (256))},

sixteen-one-r19        CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (64))}

}

CRI-TypeII-N1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (16))},

two-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (43))},

four-one-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (32))},

three-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (59))},

six-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (48))},

four-two-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (75))},

eight-one-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (64))},

four-three-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (107))},

six-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (107))},

twelve-one-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (96))},

four-four-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (139))},

eight-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (139))},

sixteen-one-r19        CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (128))}

}

For every outer CHOICE value, there is the same rather long pattern repeated, of another CHOICE between no-cbsr-r19 NULL, and a list of 1 to 4, or 1 to 8 BIT STRINGs of a size depending of the outer choice value.

RAN2 already uses a parameterized type called SetupRelease, defined as:

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

This allows to make ASN.1 more compact writing, like:

aperiodicTriggerStateList  SetupRelease { CSI-AperiodicTriggerStateList }        OPTIONAL, -- Need M

instead of:

aperiodicTriggerStateList  CHOICE {

release         NULL,

setup           CSI-AperiodicTriggerStateList

}                                                     OPTIONAL, -- Need M

The complete details of parameterized types are specified in ITU X.683 specification, that is normative for ASN.1 syntax. Using a parameterized type, it is possible to define:

CBSR-List-r19 { INTEGER:bit-string-size, INTEGER:bit-string-size } ::= CHOICE {

no-cbsr-r19 NULL,

cbsr-list-r19 SEQUENCE (SIZE (1..bit-string-size)) OF BIT STRING (SIZE (bit-string-size))

}

and change the above definitions to:

CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CBSR-List-r19 {8, 8},

two-two-r19            CBSR-List-r19 {8, 64},

four-one-r19           CBSR-List-r19 {8, 16},

three-two-r19          CBSR-List-r19 {8, 96},

six-one-r19            CBSR-List-r19 {8, 24},

four-two-r19           CBSR-List-r19 {8, 128},

eight-one-r19          CBSR-List-r19 {8, 32},

four-three-r19         CBSR-List-r19 {8, 192},

six-two-r19            CBSR-List-r19 {8, 192},

twelve-one-r19         CBSR-List-r19 {8, 48},

four-four-r19          CBSR-List-r19 {8, 256},

eight-two-r19          CBSR-List-r19 {8, 256},

sixteen-one-r19        CBSR-List-r19 {8, 64}

}

CRI-TypeII-N1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CBSR-List-R19 {4, 16},

two-two-r19            CBSR-List-R19 {4, 43},

four-one-r19           CBSR-List-R19 {4, 32},

three-two-r19          CBSR-List-R19 {4, 59},

six-one-r19            CBSR-List-R19 {4, 48},

four-two-r19           CBSR-List-R19 {4, 75},

eight-one-r19          CBSR-List-R19 {4, 64},

four-three-r19         CBSR-List-R19 {4, 107},

six-two-r19            CBSR-List-R19 {4, 107},

twelve-one-r19         CBSR-List-R19 {4, 96},

four-four-r19          CBSR-List-R19 {4, 139},

eight-two-r19          CBSR-List-R19 {4, 139},

sixteen-one-r19        CBSR-List-R19 {4, 128}

}

This makes ASN.1 easier to read, while not changing the contents of RRC PDUs.

Proposal 4: Consider using a parameterized type to make the definition of CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 and CRI-TypeII-N1-N2-CBSR-List-r19 more readable. The exact way can be determined after ASN.1 freeze, since it does not change ASN.1 encoding.

### 2.1.4	[H406] X1-X2-CBSR List

There is another example of complicated definition that could be simplified using a parameterized type:

TypeI-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (384)), thirtyTwo-r19 BIT STRING (SIZE (512)),

sixtyFour-r19 BIT STRING (SIZE (1024))},

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (192)), thirtyTwo-r19 BIT STRING (SIZE (256)),

sixtyFour-r19 BIT STRING (SIZE (512))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (96)), thirtyTwo-r19 BIT STRING (SIZE (128)),

sixtyFour-r19 BIT STRING (SIZE (256))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (96)), thirtyTwo-r19 BIT STRING (SIZE (128)),

sixtyFour-r19 BIT STRING (SIZE (256))},

four-two-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (48)), thirtyTwo-r19 BIT STRING (SIZE (64)),

sixtyFour-r19 BIT STRING (SIZE (128))},

four-four-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (24)), thirtyTwo-r19 BIT STRING (SIZE (32)),

sixtyFour-r19 BIT STRING (SIZE (64))}

}

TypeI-X1-X2-SoftScalingRank-r19 ::= CHOICE {

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (576)), thirtyTwo-r19 BIT STRING (SIZE (768)),

sixtyFour-r19 BIT STRING (SIZE (1536))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (288)), thirtyTwo-r19 BIT STRING (SIZE (384)),

sixtyFour-r19 BIT STRING (SIZE (768))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (288)), thirtyTwo-r19 BIT STRING (SIZE (384)),

sixtyFour-r19 BIT STRING (SIZE (768))},

four-two-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (144)), thirtyTwo-r19 BIT STRING (SIZE (192)),

sixtyFour-r19 BIT STRING (SIZE (384))},

four-four-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (72)), thirtyTwo-r19 BIT STRING (SIZE (96)),

sixtyFour-r19 BIT STRING (SIZE (192))},

eight-one-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (144)), thirtyTwo-r19 BIT STRING (SIZE (192)),

sixtyFour-r19 BIT STRING (SIZE (384))}

}

TypeII-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (24)), thirtyTwo-r19 BIT STRING (SIZE (32)),

sixtyFour-r19 BIT STRING (SIZE (64))},

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (12)), thirtyTwo-r19 BIT STRING (SIZE (16)),

sixtyFour-r19 BIT STRING (SIZE (32))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (6)), thirtyTwo-r19 BIT STRING (SIZE (8)),

sixtyFour-r19 BIT STRING (SIZE (16))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (6)), thirtyTwo-r19 BIT STRING (SIZE (8)),

sixtyFour-r19 BIT STRING (SIZE (16))},

four-two-r19   CHOICE {thirtyTwo-r19 BIT STRING (SIZE (4)), sixtyFour-r19 BIT STRING (SIZE (8))}

}

The following parameterized type could be defined:

PortNumBitString-r19 { INTEGER:twentyFourSize, INTEGER:thirtyTwoSize, INTEGER:sixtyFourSize } ::= CHOICE {

twentyFour-r19 BIT STRING (SIZE (twentyFourSize)),

thirtyTwo-r19  BIT STRING (SIZE (thirtyTwoSize)),

sixtyFour-r19  BIT STRING (SIZE (sixtyFourSize))

}

Then, the above definitions can be simplified to:

TypeI-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    PortNumBitString-r19 {384, 512, 1024},

two-one-r19    PortNumBitString-r19 {192, 256 (512},

two-two-r19    PortNumBitString-r19 {96, 128 (256},

four-one-r19   PortNumBitString-r19 {96, 128 256},

four-two-r19   PortNumBitString-r19 {48, 64 128},

four-four-r19  PortNumBitString-r19 {24, 32 64}

}

TypeI-X1-X2-SoftScalingRank-r19 ::= CHOICE {

two-one-r19    PortNumBitString-r19 {576)), 768 (1536},

two-two-r19    PortNumBitString-r19 {288)), 384 768},

four-one-r19   PortNumBitString-r19 {288)), 384 768},

four-two-r19   PortNumBitString-r19 {144)), 192 384},

four-four-r19  PortNumBitString-r19 {72)), 96 192},

eight-one-r19  PortNumBitString-r19 {144)), 192 384}

}

TypeII-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    PortNumBitString-r19 {24)), 32 64},

two-one-r19    PortNumBitString-r19 {12)), 16 32},

two-two-r19    PortNumBitString-r19 {6)), 8 16},

four-one-r19   PortNumBitString-r19 {6)), 8 16},

four-two-r19   PortNumBitString-r19 {4)), 8))}

}

Proposal 5: Consider using a parameterized type to make the definition of TypeI-X1-X2-CBSR-r19, TypeI-X1-X2-SoftScalingRank-r19, and TypeII-X1-X2-CBSR-r19 more readable. The exact way can be determined after ASN.1 freeze, since it does not change ASN.1 encoding.

## 2.2	Stage 2

Regarding the Stage 2 level details of TS 38.300, the Rel-19 asymmetric DL/UL mTRP feature is described in clause 6.12.1:

### 6.12.1	Asymmetric DL single-TRP and UL multi-TRP

For asymmetric DL single-TRP and UL multi-TRP operation, the UL TRP may reduce or even disable DL transmission. Pathloss offsets between two TRPs can be configured by RRC and dynamically updated by Pathloss Offset Update MAC CE, which defined in 3GPP TS 38.321[6]. Each pathloss offset is explicitly indicated for the corresponding UL/Joint TCI state for PUSCH, PUCCH, and SRS transmission toward the UL TRP. The pathloss offset can also be indicated by a PDCCH order for a PDCCH order triggered PRACH toward the UL TRP, thereby facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states can be supported for SRS separated from those used for PUSCH.

In our understanding, the current text in the specification, while technically correct, is ambiguous and lacks necessary precision for implementation. The core issue with the phrase "reduce or even disable DL transmission" is its inherent vagueness. In the context of network implementation, "reduce" is a relative term that requires a reference point. Does "reduce" mean transmitting only control channels? Does it imply a lower power? Without concrete examples, different equipment vendors and network operators could interpret these terms differently, leading to interoperability failures.

In RAN1 #119, it was agreed that

Agreement

The answer to the Question 1 in LS R1-2409353 is:

•	From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

This agreement explicitly defines a key use case for a "reduced" DL state. In this mode, the UL Transmission/Reception Point (TRP) maintains a minimal downlink presence solely for essential functions. Meanwhile, it would not transmit any user-plane or dedicated control-plane data, thus fulfilling the "asymmetric" nature of the operation where its primary role is uplink reception.

Therefore, we propose to add the two clear, unambiguous, and implementable examples “only SSB for DL transmission or totally without any DL” after the definition of asymmetric DL/UL mTRP.

Futhermore, regarding the two closed loop power control states added for SRS, we propose to clarify that one state applies to the DL TRP while the second state applies to an UL TRP, which is correspondent with the Work Item Description.

Based on the above analysis, we propose the Stage 2 clarifications in Annex B.

Proposal 6: Clarify the Rel-19 asymmetric DL/UL mTRP feature in TS 38.300 as Annex B.

# 3	Conclusion

From the analysis on Rel-19 MIMO study, we made the following proposals:

RRC issues

Proposal 1: To avoid interoperability issues, change startingBitOfFormat2-3-r19 to startingBitOfFormat2-3-r19, defined as INTEGER (32..45).

Proposal 2: In the field description of pathlossOffset, change the description from "UL-only TRP" to "UL TRP", or completely remove the phrase "UL-only TRP".

Proposal 3: The field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

ASN.1 readability

Proposal 4: Consider using a parameterized type to make the definition of CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 and CRI-TypeII-N1-N2-CBSR-List-r19 more readable. The exact way can be determined after ASN.1 freeze, since it does not change ASN.1 encoding.

Proposal 5: Consider using a parameterized type to make the definition of TypeI-X1-X2-CBSR-r19, TypeI-X1-X2-SoftScalingRank-r19, and TypeII-X1-X2-CBSR-r19 more readable. The exact way can be determined after ASN.1 freeze, since it does not change ASN.1 encoding.

Stage 2

Proposal 6: Clarify the Rel-19 asymmetric DL/UL mTRP feature in TS 38.300 as Annex B.

# Annex A: TP for TS 38.331

### 6.3.2	Radio resource control information elements

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI.

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]],

[[

startingBitOfFormat2-3-v1900            INTEGER (32..45)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212 [17] clause 7.3.1 and , TS 38.213 [13], clause 11.4.

startingBitOfFormat2-3, startingBitOfFormat2-3-v1900 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-v1900 simultaneously.

startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4).

#### –	TCI-State

TCI-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB for both QCL-Type1 and QCL-Type2. In case the cell is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field cell. Otherwise, it refers to a PCI value configured in a list additionalPCI-ToAddModList configured in the serving cell where the TCI-State is applied by the UE. When this field is present the cell for qcl-Type1 and qcl-Type2 is configured with same value, if present.

pathlossOffset Indicates the pathloss offset as specified in TS 38.213 [13], clause 7. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

pathlossReferenceRS-Id The ID of the reference signal (e.g. a CSI-RS or an SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the TCI State is applied by the UE.

qcl-Type1, qcl-Type2 QCL information for the TCI state as specified in TS 38.214 [19] clause 5.1.5.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

tci-StateId ID number of the TCI state.

ul-PowerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the dl-OrJointTCI-StateToAddModList is configured. For each serving cell, ul-powerControl is either configured in each TCI-State configured or referred to in dl-OrJointTCI-StateList in each DL BWP, or it is not configured in any TCI-State configured or referred to in dl-OrJointTCI-StateList in any DL BWP.

#### –	CSI-ReportConfig

CSI-ReportUE-Initiated field descriptions

currentBeamReport If configured, the UE includes measurements of the current beam in the UE initiated CSI reporting.

eventDetectionTimeWindow Indicates the time window length for triggering event determination (see TS 38.214 [19], clause 5.2). Value ms4 corresponds to 4 milliseconds, value ms5 corresponds to 5 milliseconds and so on.

eventInstanceCount Indicates the minimum number of event instances for one same new beam within a configured time window that the UE can initiate SI report (see TS 38.214 [19], clause 5.2.1.5.4.1).

eventTypeUE-Initiated Indicates the event type for UE initiated CSI reporting and associated fields as specified in clause 5.2.1.5.4 of TS 38.214 [19]. When this field is set to event1, event1Threshold can only be configured with values from 16 to 113. The parameter conditionFulfillmentIndicator is only configured if eventDetectionTimeWindow is configured and nrofReportedRS-UE-Initiated is not set to n1.

minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting. Value sym0 corresponds to first symbol, value sym1 corresponds to second symbol and so on.

# Annex B: TP for TS 38.300

### 6.12.1	Asymmetric DL single-TRP and UL multi-TRP

For asymmetric DL single-TRP and UL multi-TRP operation, the UL TRP may reduce or even disable DL transmission (i.e. only SSB for DL transmission or totally without any DL). Pathloss offsets between two TRPs can be configured by RRC and dynamically updated by Pathloss Offset Update MAC CE, which is defined in 3GPP TS 38.321[6]. Each pathloss offset is explicitly indicated for the corresponding UL/Joint TCI state for PUSCH, PUCCH, and SRS transmission toward the UL TRP. The pathloss offset can also be indicated by a PDCCH order for a PDCCH order triggered PRACH toward the UL TRP, thereby facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states, one for the DL TRP and the other for UL TRP, can be supported for SRS separated from those used for PUSCH.