3GPP TSG-RAN WG2 Meeting #132	R2-2509124

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	LTM corrections

Document for: Discussion and Decision

# 1	Introduction

This contribution discusses several issues on MAC and RRC for LTM.

# 2	MAC issues

Clause 5.32.5 is specified as follows:

### 5.35.2	Performing measurement

An RRC_CONNECTED UE obtains L1 beam level measurement results by measuring one or multiple RSs as configured by the network as specified in TS 38.214 [7] for the LTM candidate cell(s) with the candidate ID configured in ltm-CandidateReportConfigList for evaluation of reporting criteria or of execution condition. For each L1 beam level measurement result in RRC_CONNECTED, the UE applies the layer 1 filtering by implementation, before using the measured results for evauation of reporting criteria and measurement reporting or of execution condition. When the UE has two indicated TCI-states, the UE uses the best beam of serving cell is used for LTM event evaluation. It is up to the UE implementation how to choose the best beam. The MAC entity performs the evaluation of reporting criteria as specified in 5.35.3 or of execution condition as specified in 5.36.2 based on the L1 measurement results from lower layer. For the LTM candidate cell(s) with the candidate ID not configured in ltm-CandidateReportConfigList, the UE is not required to perform and the event evaluation in 5.35.3 on the RSs belonging to the candidate ID for the corresponding event.

For L1 beam level event triggered measurements report, the network can configure SS/PBCH block(s) or CSI-RS as event evaluation RS type, and L1-RSRP as trigger quantity. Reporting quantity is the same as the trigger quantity.

TS 38.214 only specifies measurement and reporting "For a report setting ltm-CSI-ReportConfig configured with ltm-ReportConfigType set to ‘periodic’ or ‘semiPersistentOnPUCCH’ or ‘semiPersistentOnPUSCH’ or ‘aperiodic’" i.e., it does not cover the case of L1 event-triggered measurements or L1 execution conditions.

Proposal 1: Remove the reference to TS 38.214 in TS 38.321 clause 5.32.2 because TS 38.214 does not cover measurements for L1 event-triggered reports or L1 execution conditions.

5.35.2 includes considerations on:

-	L1 filtering;

-	determination of the current beam when two TCI states are indicated;

- 	which cell to measure for event-triggered evaluation in 5.35.3;

-	performing evaluation of reporting criteria in 5.35.3 or of execution condition in 5.36.

However, 5.35.3.1 determine the "applicable RS", which are exactly the ones to be measured, but 5.35.2 repeats that, in a different language, and with English language mistakes. Moreover, in 5.32.2, it is not specified for which LTM-CSI-ReportConfig this is applicable, while this is specified in 5.35.3.1:

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

Proposal 2: Remove from 5.32.2 all considerations of which cell to measure (because this is redundant with 5.35.3.1) and add in 5.35.3.1 to derive L1 measurement results for applicable RS, following the indications in 5.32.2.

### 5.35.2	Performing measurement

For L1 measurements performed for the evaluation of reporting criteria as specified in 5.35.3 or of execution condition as specified in 5.36.2, the MAC entity shall:

-	 apply layer 1 filtering by implementation, before using the measured results for evaluation of reporting criteria and measurement reporting or of execution condition;

-	when the UE has two indicated TCI-states, use the best beam of the serving cell. It is up to the UE implementation how to choose the best beam;

And in 5.36.3 for conditional LTM evaluation:

#### 5.35.3.1	General

[...]

The MAC entity shall:

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>	if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet:

5>	consider the current beam of serving cell, i.e. the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

4>	else:

5>	consider the current beam of serving cell, i.e. the SSB QCLed with the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

3>	if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed in ltm-CandidateReportConfigList, to be applicable.

4>	else:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed, to be applicable.

2>	derive L1 measurement results for the applicable RS, as specified in clause 5.35.2;

On which cell/RS the UE shall perform measurements is entirely determined by the LTM-CSI-ReportConfig for

As mentioned above, for which RS to perform measurement for execution condition evaluation is not specified in 5.35.2, but 5.36.2 specifies the "applicable beams", instead of RS. Also, the wording is extremely obfuscated.

Proposal 3: Specify in 5.36.2 that the UE shall derive L1 measurement results for applicable RS as specified in 5.35.2 (to make the consideration on "best beam" apply to execution conditions).

Proposal 4: Change in 5.36.2 "beam" to "RS" and simplify the terribly obfuscated text.

### 5.36.2	L1 measurement based Conditional LTM triggering condition evaluation

Upon indication from upper layers to initiate the LTM cell switch conditions evaluation based on L1 measurements, the MAC entity shall:

1>	for each entry within the VarLTM-ExecutionConditionList with executionCondition set to l1-Conditions:

2>	consider the RS of the ltm-CandidateId that are in the LTM-CSI-ResourceConfig indicated by ltm-ResourcesForChannelMeasurement of the ltm-CSI-ReportConfigId in this entry to be applicable;

2>	derive L1 measurement results for the applicable RS, as specified in clause 5.35.2;

2>	if the entry condition for the event associated with the ltm-CSI-ReportConfigId in this entry is fulfilled for TTT for one or more applicable RS:

3>	consider the event associated with LTM-CSI-ReportConfigId to be fulfilled for the ltm-CandidateId associated with LTM-CSI-ReportConfigId;

3>	perform the CLTM execution procedure for the LTM candidate configuration associated with ltm-CandidateId according to the procedure specified in 5.36.3.

# 2.2	RRC issues from ASN.1 review

### 2.2.1	[H155] reportQuantity

In Rel-19, reportQuantity-r19 was added:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

-- Contents omitted

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900       OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18  OPTIONAL  -- Need R

]]

}

LTM-ReportContent-v1900  ::= SEQUENCE {

reportQuantity-r19         ENUMERATED {cri-RSRP, ssb-index-RSRP, cri-RI-PMI-CQI, value1},

...

}

The network must not send this field to a Rel-18 UE that supports reporting L1-RSRP for LTM.

If a Rel-18 UE that supports L1-RSRP measurements for LTM is upgraded to support a Rel-19 feature unrelated to LTM, this UE has no reason to implement any Rel-19 extension for LTM, so if the network always sends the reportConfig field to such a UE, the UE may ignore the field, or ignore the whole LTM-CSI-ReportConfig, or even trigger re-establishment.

Besides, even though LTM-ReportContent-v1900 is optional and currently has a single field, it can be extended in a later release, then further values of reportQuantity would be introduced, it should be possibly to omit reportQuantity in LTM-ReportContent-v1900.

Proposal 5a: To allow including new fields in LTM-ReportContent-v1900 in a later release and omit reportQuantity-r19, make reportQuantity optional.

Proposal 5b: To avoid issues with Rel-19 UEs that only support Rel-18 LTM and will not understand reportQuantity-r19, remove the value ssb-index-RSRP from reportQuantity-r19 and capture that absence means that SSB index and RSRP are to be reported.

### 2.2.2	[H160] List of CSI-IM resource sets

In each LTM-Candidate, there is:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                          LTM-CandidateId-r18,

-- Fields omitted

...,

[[

-- Fields omitted

ltm-CSI-ReportConfig-r19                     SetupRelease {LTM-CSI-ReportConfig-r18}

OPTIONAL,    -- Need M

ltm-CSI-IM-ResourceToAddModList-r19          SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources))

OF CSI-IM-Resource         OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToReleaseList-r19         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources))

OF CSI-IM-ResourceId       OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToAddModList-r19       SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets))

OF CSI-IM-ResourceSet      OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToReleaseList-r19      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets))

OF CSI-IM-ResourceSetId    OPTIONAL     -- Need N

]]

}

In ltm-CSI-ReportConfig-r19, there is:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

-- Fields omitted

...,

[[

-- Fields omitted

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18  OPTIONAL, -- Need R

-- Fields omitted

]]

}

ltm-ResourceForInterferenceMeasurements-r19 refers to an LTM-CSI-ResourceConfig defined in LTM-Config, and its contents are:

LTM-CSI-ResourceConfig-r18 ::=    SEQUENCE {

ltm-CSI-ResourceConfigId-r18      LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18           LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19    LTM-NZP-CSI-RS-ResourceSet-r19         OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19        LTM-CSI-IM-ResourceSet-r19             OPTIONAL, -- Need R

resourceType-r19                  ENUMERATED {periodic, semiPersistent}  OPTIONAL  -- Cond NoSSB

]]

}

In the above, ltm-CSI-IM-ResourceSet-r19 refers to one CSI-IM-ResourceSet in same LTM-Candidate like ltm-CSI-ReportConfig-r19.

According to TS 38.214:

-	only CSI reporting uses CSI-IM resource sets, and CSI reporting for LTM only uses ltm-CSI-ReportConfig

-	all UE actions rely on an LTM-CSI-ReportConfig, so a CSI-IM resource (set) that is not referenced by an LTM-CSI-ResourceConfig that is referenced to an LTM-CSI-ReportConfig will not trigger any UE action.

This means that:

-	the UE will ignore any CSI-IM resource set in one LTM-Candidate besides the one that is referred to by ltm-CSI-ReportConfig in that LTM-Candidate.

-	the UE will ignore any CSI-IM resource in one LTM-Candidate besides the ones that are referred to by the CSI-IM resource set referred to by the ltm-CSI-ReportConfig in that LTM-Candidate.

Observation 1: In one LTM-Candidate, the UE can only use a single CSI-IM resource set, and can only use CSI-IM resources indicated by that CSI-IM resource set.

According to the current structure, if the network wants to configure the LTM-CSI

Proposal 6: Discuss whether to

a)	capture that ltm-CSI-IM-ResourceToAddModList-r19 can only contain one element, that that this element refers to all elements configured via ltm-CSI-IM-ResourceSetToAddModList-r19; or

b)	remove the ToAddModList/ToReleaseList for CSI-IM resources and resource sets, and define them directly in LTM-CSI-ResourceConfig (like LTM-SSB-ResourceSet-r18).

### 2.2.3	[H159] Early CSI report configuration

LTM-CSI-ReportConfig is defined as:

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure gNB-scheduled measurement report on the cell in which the LTM-CSI-ReportConfig is included when the field ltm-ReportConfigType is configured as periodic, semi-persistentOnPUCCH, or semi-persistentOnPUSCH/aperiodic.

The IE LTM-CSI-ReportConfig is also used to configure events for the conditional execution of an LTM cell switch procedure.

The IE LTM-CSI-ReportConfig is also used to configure event-triggered measurement report when the field ltm-ReportConfigType is configured as eventTriggered, within which the criteria for triggering measurement report by MAC CE as in TS 38.321 [3] is specified.

Event LTM2:	Beam of SpCell becomes worse than absolute threshold;

Event LTM3:	Beam of candidate cell becomes amount of offset better than the beam of SpCell;

Event LTM4:	Beam of candidate cell becomes better than absolute threshold;

Event LTM5:	Beam of SpCell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

servingSpecificOffset-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL, -- Need R

ltm-CondebookConfig-r19                        LTM-CodebookConfig-r19                                     OPTIONAL, -- Need R

ltm-cqi-Table-r19                              CQI-Table                                                  OPTIONAL  -- Need R

]]

}

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, ssb-index-RSRP, cri-RI-PMI-CQI, value1},

...

}

In the above:

-	the IE description does not cover early CSI report;

-	the fields in red are only for early CSI report;

-	the field in blue is the only field that is common to early CSI report and to other types of L1 reports.

Then, if would be clearer to have a separate field of early CSI report.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                          LTM-CandidateId-r18,

-- Fields omitted

...,

[[

-- Fields omitted

ltm-CSI-ReportConfig-r19                     SetupRelease {EarlyCSI-ReportConfig-r19}

OPTIONAL,    -- Need M

-- Fields omitted

]]

}

EarlyCSI-ReportConfig-r19 ::=    SEQUENCE {

resourceForChannelMeasurement-r19          LTM-CSI-ResourceConfigId-r18,

resourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18     OPTIONAL,  -- Need R

codebookConfig-r19                         CHOICE {

twoToThirtyTwoPorts     CodebookConfig,

moreThanThirtyTwoPorts  CodebookConfig-r19,

...

}                                OPTIONAL, -- Need R,

reportQuantity-r19                         ENUMERATED { cri-PMI-CQI, ... }  OPTIONAL, -- Need R,

cqi-Table-r19                              CQI-Table                        OPTIONAL, -- Need R,

...

}

Then, LTM-CSI-ReportConfig can be simplified:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

servingSpecificOffset-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL -- Need R

]]

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ReportInterval-r19,

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity, spare1},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                       OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                       OPTIONAL, -- Cond LTM2

...

}

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, ssb-index-RSRP },

...

}

Proposal 7: Define EarlyCSI-ReportConfig in LTM-Candidate with the only 5 fields required for early CSI report, and remove the fields specific to early CSI report from LTM-CSI-ReportConfig (whose IE description does not cover early CSI report).

# 3	Conclusion

In this contribution, we have listed necessary corrections for MAC and RRC and provided TPs.

MAC corrections -see TPs in the discussion part

Proposal 1: Remove the reference to TS 38.214 in TS 38.321 clause 5.32.2 because TS 38.214 does not cover measurements for L1 event-triggered reports or L1 execution conditions.

Proposal 2: Remove from 5.32.2 all considerations of which cell to measure (because this is redundant with 5.35.3.1) and add in 5.35.3.1 to derive L1 measurement results for applicable RS, following the indications in 5.32.2.

Proposal 3: Specify in 5.36.2 that the UE shall derive L1 measurement results for applicable RS as specified in 5.35.2 (to make the consideration on "best beam" apply to execution conditions).

Proposal 4: Change in 5.36.2 "beam" to "RS" and simplify the terribly obfuscated text (see TP).

RRC corrections

Proposal 5a: To allow including new fields in LTM-ReportContent-v1900 in a later release and omit reportQuantity-r19, make reportQuantity optional.

Proposal 5b: To avoid issues with Rel-19 UEs that only support Rel-18 LTM and will not understand reportQuantity-r19, remove the value ssb-index-RSRP from reportQuantity-r19 and capture that absence means that SSB index and RSRP are to be reported.

Proposal 6: Discuss whether to

a)	capture that ltm-CSI-IM-ResourceToAddModList-r19 can only contain one element, that that this element refers to all elements configured via ltm-CSI-IM-ResourceSetToAddModList-r19; or

b)	remove the ToAddModList/ToReleaseList for CSI-IM resources and resource sets, and define them directly in LTM-CSI-ResourceConfig (like LTM-SSB-ResourceSet-r18).

Proposal 7: Define EarlyCSI-ReportConfig in LTM-Candidate with the only 5 fields required for early CSI report, and remove the fields specific to early CSI report from LTM-CSI-ReportConfig (whose IE description does not cover early CSI report).