3GPP TSG RAN WG2 #133               	                                                             R2-2600069

Gothenburg, Sweden, Feb. 09th – 13th, 2026

Source:	CATT

Title:	Discussion on System Information

Agenda Item:	10.3.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2#132 meeting we agreed:

Agreements on system information

- Assume on demand SI is supported and study further.   Study further on demand SIB1 from RAN2 perspective.

- Assume area specific SI is supported.  Further study what information can be common.   FFS area specific SIB1.

Study SI scheduling mechanism to enable more flexible scheduling and clustering/bundling of transmission(s) with other common signalling.  Study should also take into account UE and NW complexity.   Aim to have one mechanism if possible.

Before discussing SIB1 splitting, understand SIB1 design, content and size.  Then we can revisit how and if splitting is needed taking into account benefits .

This contribution will discuss further on System Information for 6GR.

- Discussion

  - Broadcast of Positioning Assistance Data

RAN2 assumes that regulatory requirements on emergency calls, positioning associated to emergency call requirement (e.g. ECID) and PWS are should be met from Day1 at last meeting.

- Agreements

- 2	RAN2 assumes that regulatory requirements on emergency calls, positioning associated to emergency call requirement (e.g. ECID) and PWS are should be met from Day1.   RAN2 chair should highlight to plenary in status report that input may be required for the requirements and discuss whether to update SI description.    Wait for RAN input for further positioning requirements.

There was some discussion in RAN plenary #110 on positioning in RP-253806 as below:

Proposal 11: Capture following test proposal in TR 38.914:

5.4.x        Positioning

The 6GR and 6G RAN shall support RAT-dependent and RAT-independent positioning techniques (at least including ECID and A-GNSS with any potentially needed enhancements, based on a simple framework) to satisfy the requirements in section 5.1.14 and regulatory location services.

Considering the alignment with ITU, it was finally decided at RAN plenary #110:

proposal 11 is postponed (will be used as starting point at RAN #111, as we want to check ITU meeting status after their Feb. meeting).

RAN Plenary #110 decided to postpone Proposal 11 regarding positioning techniques (including ECID and A-GNSS, etc.) to align with the ITU process. Nevertheless, from a RAN2 perspective, it is reasonable to expect that basic positioning technologies, including A-GNSS and ECID, will be supported from the initial phase (Day-1) of 6G.

Positioning assistance data have been included in positioning System Information Blocks (posSIBs) since NR Rel-17. Positioning SIB type includes GNSS Common Assistance Data, GNSS Generic Assistance Data, and so on. Broadcast of positioning assistance data can improve network efficiency if multiple UEs need the information. Meanwhile, it can reduce positioning latency by reducing additional signaling procedure to obtain assistance data, and UE power consumption can be reduced accordingly, especially for UE-based A-GNSS position method.

Considering that the 5G standardization process experienced prolonged discussions and challenges due to prematurely defining specifics such as scheduling and capacity for particular posSIBs, the 6G system information design should, at an early stage, fully account for the distinct characteristics of posSIBs. These include, for example:

- Diverse periodicity requirements.

- Potentially larger message sizes.

- Validity, update, and latency requirements.

- Potential multi-technology extensibility (e.g., for A-GNSS data, RAT-Dependent positioning data etc.).

Therefore, it is necessary for RAN2 to initiate a forward-looking framework study on the posSIB broadcast mechanism at the outset of system information architecture design. This will ensure the design is flexible, efficient, and future-proof, rather than being limited only to the specific positioning technologies or data types supported from Day-1.

Proposal 1: RAN2 study the design of a flexible broadcast mechanism for positioning assistance data (posSIBs) in 6GR, as part of the overall system information framework. The study shall focus on ensuring that the SI design can natively accommodate the distinct characteristics of posSIBs (e.g., periodicity, message size, validity, and update mechanisms) from the initial design phase.

  - On-demand SIB1

In 5G, SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control. In R19 NES WID,  OD-SIB1 procedure is introduced for network energy saving considering the big size of SIB1. For 6GR, the specific content in SIB1 is still under discussion, however, the OD-SIB1 procedure could also be applied at least from the view of energy saving objective of 6GR SI.

Review of R19 OD-SIB1

R19 NES firstly focused on the OD-SIB procedure for UE in RRC Idle/Inactive mode. Below case 1 for standalone scenario and other three cases for Cell-A assisted OD-SIB1 procedure were once discussed in RAN1/RAN2 R19 NES topic.

Case 1 (Standalone Scenario)                                             Case 2 (selected by R19 NES)

Case 3                                                                                Case 4

- Case 1: UE will transmit OD-SIB1 preamble to NES cell and receive on-demand SIB1 from NES cell, after obtaining the OD-SIB1 configuration from the NES Cell MIB or pre-configuration;

- Case 2: UE will transmit OD-SIB1 preamble to NES cell and receive on-demand SIB1 from NES cell, after obtaining the OD-SIB1 configuration from the Cell A;

- Case 3:  UE will transmit OD-SIB1 preamble to Cell A and receive on-demand SIB1 from Cell A, after obtaining the OD-SIB1 configuration from the Cell A;

- Case 4: UE will transmit OD-SIB1 preamble to Cell A and receive on-demand SIB1 from NES cell, after obtaining the OD-SIB1 configuration from the Cell A.

Considering the spare bits in MIB is limited and pre-configuration of OD-SIB preamble is also complex, case 1 for standalone scenario was not supported in R19 NES. The case 2 with Cell A transmitting the OD-SIB1 configuration and NES Cell receiving preamble and transmitting OD-SIB1 was selected as the only supported case for R19 NES by majority of view.

RAN2 also included the OD-SIB1 procedure for UE in RRC Connected mode occurring RLF, the OD-SIB1 obtaining for UE in RRC Connected mode excepting RLF was considered as network implementation, such as the network could transmit the SIB1 to UE by RRC dedicated message, no spec impact was introduced.

OD-SIB1 for 6GR

For 6GR OD-SIB1 procedure, if standalone scenario as in case 1 is introduced, the MIB needs to include the configuration information on OD-SIB1 procedure, or the information on OD-SIB1 procedure needs to be preconfigured to UE, which are in the scope of RAN1, RAN1 will determine whether to support standalone OD-SIB1 scenario. If RAN1 support the standalone scenario for OD-SIB1 procedure, UE will read NES Cell’s MIB or pre-configuration information and then trigger the preamble to NES for OD-SIB1, the potential impact to RAN2 aspect is also limited. Thus, we could wait RAN1 conclusion on whether/how to support the standalone scenario for OD-SIB1 before RAN2 discuss this scenario.

For other cases on Cell A assisted OD-SIB1 procedure, the impact is shown in the table:

Table 1 Impact of different cases on Cell A assisted OD-SIB1

| Case\Impact | NES cell | Cell A | Xn | Delay to on demand SIB1 |
| --- | --- | --- | --- | --- |
| 2 | WUS receiving and SIB1 transmission | WUS configuration in SIB; | Exchange of WUS configuration; | Normal |
| 3 | N/A | WUS configuration in SIB; WUS receiving and SIB1 transmission; | Exchange of SIB1 and updating; | Normal |
| 4 | SIB1 transmission | WUS configuration in SIB; WUS receiving over Uu; | SIB1 triggering receiving/transmission ; | Extra delay over Xn to trigger SIB1 transmission |

When UE camps on Cell A, compared with method to read the MIB of NES Cell for OD-SIB1 configuration at standalone scenario, the Cell A assisted OD-SIB1 procedure could make UE reduce the delay and complexity to obtain the OD-SIB1configuration, such as the preamble and RO resource for OD-SIB1 request. Hence, RAN2 needs to discuss whether Cell A assisted OD-SIB1 procedure will be considered as one of the candidate solution for OD-SIB1.

Furthermore, if Cell A assisted OD-SIB1 procedure is supported, based on above table, similarly to R19 NES discussion, the SIB1 exchange/updating over Xn interface in case 3 seems more complex and frequently then the OD-SIB1 configuration exchange/updating in case 2, and more energy consumption at Cell A is introduced by case 3. For case 4, the extra delay over Xn interface to trigger SIB1 transmission is not desired for OD-SIB1 procedure. Hence, the case 2 is better to be considered as the supported one for 6GR Cell A assisted OD-SIB1 procedure. Besides, for 6GR OD-SIB1, as above review to R19 NES, the UE in RRC Idle/Inactive/Connected mode should all be able to obtain OD-SIB1 by OD-SIB1 request procedure or by network implementation.. The legacy technique for case 2(i.e., case 2 in R19 NES WI) could be considered as a baseline for 6GR Cell A assisted OD-SIB1 procedure.

Proposal 2: For 6GR OD-SIB1, RAN2 discuss the following scenarios:

- Standalone scenario: RAN2 shall wait RAN1’s conclusion on whether/how to support OD-SIB1 in standalone scenario before proceeding with the OD-SIB1 procedure design for this scenario.

- Inter-cell assistance scenario: RAN2 study the scenario that Cell A assisted OD-SIB1 procedure for Cell B. The Case 2 in R19 NES WI can be the starting point, where Cell A transmits the OD-SIB1 configuration and Cell B receives preamble and transmits OD-SIB1.

  - On-demand SI

NR supports on-demand system information. Instead of always periodically transmitting SI, the network can broadcast on-demand SI upon request from UEs in RRC_IDLE/RRC_INACTIVE via either a Msg1-based or a Msg3-based method. As system information is provided on-demand, network energy saving can be achieved. In low-load scenarios, particularly during nighttime or in suburban areas, significantly network energy efficiency can be achieved with on-demand SI for UEs in RRC_IDLE/INACTIVE states.

With Msg1-based SI request, the network can early identify on-demand SI request. Latency can be reduced. However, the network needs to reserve RACH resources for it. Especially when the network configures resources for Msg1 based SI request for each SI that is not broadcasting, more RACH resources need to be reserved.

In NR, the network can broadcast on-demand SI or send in a dedicated manner upon request from UEs in RRC_CONNECTED as well. The supported on-demand SIBs in RRC_CONNECTED includes posSIBs, SIB22 for ATG access and so on.

Observation 1: On-demand SI can achieve energy efficiency. Compared with Msg3-based on-demand SI in IDLE/INACTIVE, Msg1-based method provides lower latency but occupies more RACH resources. And the supported on-demand SIBs in CONNECTED include posSIBs, SIB22 for ATG and so on.

Considering SI request is not critical for latency and RACH resources are limited, the Msg1-based on-demand SI is not preferred. Hence, we propose RAN2 focus on-demand SI in non-connected mode for 6GR only within Msg3-based method.

6GR support Air-to-Ground deployment scenario[1]. In addition, it has already captured that positioning is the ability to estimate the position of connected devices in TR 38.914[1]. Therefore, it is reasonable to consider on-demand SIBs in RRC_CONNNECTED for ATG and positioning in 6GR. RAN2 need to study on-demand SI in connected mode for 6GR as well.

Proposal 3: RAN2 study on-demand SI in non-connected mode for 6GR only within Msg3-based method, and on-demand SIBs in connected mode for 6GR.

  - SI scheduling mechanism

In NR, SI scheduling is based on SI window. Each SI message is associated with an SI window. All SI messages use the same SI window length. But network can configure a different SI periodicity for each SI message. Within one SI window only the corresponding SI message is transmitted. An SI message may be repeated with the same content a number of times within the SI window. UE can combine the received SI messages within the window. With SI window mechanism, flexible scheduling and implementation simplification can be achieved.

In NR, there are three SI scheduling lists. At the beginning of NR, the start position of SI window is determined implicitly, which is related to the order of the concerned SI in the SI scheduling information list in SIB1. The determination of SI window is simple. But the maximum number of SI-messages is limited by the recurrence of the SI-message with the shortest SI periodicity. SI-windows for SI-messages using larger SI periodicity remain empty in some time domain. As more SIBs are introduced in later 5G release, more and more SI messages are required. It is not efficient any longer with the implicit determination of SI window. In NR Rel-17, an explicit start position of SI window was introduced, which can use the empty SI windows as much as possible.

However, in NR SI windows of different SI messages do not overlap. That means the network cannot transmit all SIs in a short time, even if the load of the network is low.

Observation 2: NR SI window mechanism can provide flexible scheduling and implementation simplification. In addition, an explicit start position of SI window was introduced in NR Rel-17, which can achieve more flexible and efficient scheduling. But SI windows of different SI messages do not overlap in NR, with which the clustered SI message transmission cannot be achieved.

Considering the benefit of SI window, we propose to support SI window for SI scheduling in 6GR. Furthermore, to enable more flexible and efficient SI scheduling, an explicit start position of SI window can be considered. In addition, in order to achieve more sleeping opportunities of the network, overlapping SI window can be considered.

Proposal 4: RAN2 support SI window for SI scheduling in 6GR. Furthermore, RAN2 study:

- an explicit start position of SI window to enable more flexible and efficient scheduling of SI,

- how to support overlapping of SI windows to achieve more sleeping opportunities of network for energy efficiency.

  - SIB1 contents

In NR, only one SIB, i.e., SIB1, is specified for Remaining Minimum SI (RMSI). This can reduce the latency of initial access as all information needed for initial access is combined in SIB1. However, SIB1 contains quite a lot of information, e.g. information for camping, information required for initial access, access control parameters, the scheduling of other system information blocks and so on. And during the NR spec evolution, more and more parameters are extended in SIB1. The SIB1 becomes bloated, as shown below.

Table 2 SIB1 contents in NR

| Fields | Brief descriptions | Whether crucial for camping |
| --- | --- | --- |
| cellSelectionInfo | Parameters for cell selection | Yes |
| cellAccessRelatedInfo | Cell access related information, including PLMN identity information, and cell reserved information | Yes |
| connEstFailureControl | Parameters for connection establishment failure control | No |
| si-SchedulingInfo/posSI-SchedulingInfo-r16/si-SchedulingInfo-v1740 | Information needed for acquisition of SI messages | No |
| servingCellConfigCommon | Cell specific parameters for initial access, including common downlink parameters of a cell, common uplink parameters of a cell, and so on. | No |
| ims-EmergencySupport | Indicate whether the cell supports IMS emergency bearer services for UEs in limited service mode. | No |
| eCallOverIMS-Support | Indicate whether the cell supports eCall over IMS services. | No |
| ue-TimersAndConstants | Timer and constant values to be used by the UE. | No |
| uac-BarringInfo/uac-BarringInfo-v1630/uac-BarringInfo-v1700 | Access control parameters | No |
| useFullResumeID | Indicate which resume identifier and Resume request message should be used. | No |
| idleModeMeasurementsEUTRA-r16/idleModeMeasurementsNR-r16 | Indicate whether the UE shall perform idle/inactive measurements. | No |
| hsdn-Cell-r17 | Indicate this is a HSDN cell. | No |
| sdt-ConfigCommon-r17/mt-SDT-ConfigCommonSIB-r18/ sdt-BeamFailureRecoveryProhibitTimer-r18 | SDT related common parameters. | No |
| hyperSFN-r17/eDRX-AllowedIdle-r17/eDRX-AllowedInactive-r17 | eDRX related parameters. | No |
| musim-CapRestrictionAllowed-r18 | Indicate the UE is allowed to send the musim-CapRestrictionInd in RRCSetupComplete, RRCResumeComplete and RRCReestablishmentComplete messages. | No |
| featurePriorities-v1800 | Indicate priorities for features for RACH partition. | No |
| redCap-ConfigCommon-r17/ eRedCap-ConfigCommon-r18/ cellBarredNTN-r17/ ncr-Support-r18/cellBarredATG-r18/ cellBarredNES-r18/ cellBarredFixedVSAT-r18/… | Cell bar related information in SIB1. | Yes |
| n3c-Support | Indicate the support of N3C MP. | No |

In order to reduce network energy consumption, On-demand SIB1 was introduced in NR Rel-19. However, it is not energy efficient enough with bloated SIB1. For example:

- When the network updates the info of SIBx, although the UE only cares about the scheduling info of SIBx in SIB1, the network has to provide the whole SIB1 and the UE has to acquire the complete SIB1.

- When a UE requests other SI, the UE only cares about whether the concern SI is broadcasting or not broadcasting. However, the UE has to acquire the complete SIB1 to obtain the broadcast status of the concern SI.

As summarized in the above table, some information in SIB1 in NR are not needed for some UEs to camp on a cell only. For example: For some UEs that only camp on the cell, it is not crucial to obtain information for initial access, e.g., access control parameters, common downlink parameters of the serving cell, common uplink parameters of the serving cell, and so on. But the information for camping, e.g., cell selection related information, cell access related information, and cell bar information if available, is crucial and necessary for all UEs camped in one cell.

Observation 3: The information for camping, e.g., cell selection related information, cell access related information, and cell bar information if available, is crucial and necessary for all UEs camping on one cell. But other information, e.g., the information for initial access, is not crucial for all UEs camping on the cell.

According to the above observation, we propose sub-SIB1 or sub-RMSI in 6GR at least contains the information for camping, including cell selection related information, cell access related information, and cell bar information if available. If the size of sub-SIB1 or sub-RMSI in 6GR is small enough, it can be always periodically broadcast. For another sub-SIB1 or sub-RMSI in 6GR, which contains other information for initial access like in NR SIB1, it can be broadcast with on-demand.

Proposal 5: sub-SIB1 or sub-RMSI in 6GR at least contains the information for camping, including cell selection related information, cell access related information, and cell bar information if available.

  - SIB validity

Area specific SIB is supported in NR. But the feature was not implemented yet. In most cases, for a certain SIB there are some parameters that can be same within an area while there are still some cell specific parameters. For example: SIB4 in NR contains information relevant for inter-frequency re-selection. For parameters for an inter-frequency reselection, they can be same within an area, e.g., center frequencies of the SS block of the neighbour cells (dl-CarrierFreq), cell reselection priority of the inter-frequency (cellReselectionPriority/cellReselectionSubPriority) and so on .But there are some cell specific parameters, e.g. the list of exclude-listed inter-frequency neighbouring cells (interFreqExcludedCellList) and so on. It is hard to apply the concept of area specific SIB. In addition, the storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation. It is uncertain if area specific validity can be applied in UEs.

For area specific SIB in 6GR, the following possible options can be considered:

- Option 1: Reuse area specific concept of SI in NR.

- Option 2: Reorganization of SIBs. Parameters that can be same within an area will be included in one SIB while parameters that are cell specific information will be included in another SIB.

- Option 3: It is allowed to support partial parameters of one SIB are area specific validity.

With Option 2, we need to discuss which parameters can be same within an area and which parameters can is cell specific information case by case. And it is not allowed to include SIBs that can be same within an area and SIBs that can be cell specific in an SI. More SIs are required. Furthermore, when a new feature is introduced in System information in later release, at least two SIBs need to be defined, one SIB for parameters that can be same within an area and the other SIB for parameters that are cell specific information. Additional complexity is introduced.

With Option 3, if partial parameters of one SIB are area specific validity, a UE still needs to acquire the SIB of the camped cell to obtain cell specific validity information, even if it has stored area specific valid parameters of the SIB. Compared with option 1, it is doubtful on the power saving gain with option 3.

Table 3 Summary of possible options for area specific SIB

| Options | Summary |
| --- | --- |
| Option 1: Reuse area specific concept of SI in NR | It was not implemented yet. |
| Option 2: Change the content of SIBs. Parameters that can be same within an area are included in one SIB while parameters that can be cell specific are included in another SIB. | Additional complexity is introduced. For example: we need to discuss which parameters can be same within an area and which parameters can is cell specific information case by case. And when a new feature is introduced in System information in later release, at least two SIBs need to be defined. |
| Option 3: It is allowed to support partial parameters of one SIB are area specific validity. | Compared with option 1, it is doubtful on the power saving gain. |

Therefore, if area specific SI is still supported in 6GR, we propose to consider the mechanism in NR as a starting point.

Proposal 6: Consider the area specific SI mechanism in NR as a starting point.

  - NTN aspects

For NTN, to address the impacts of NTN specific characteristics, such as the long and varying propagation delay and wide coverage, this section discusses NTN specific mechanisms on top of the unified system information design with TN.

NTN specific assistance information and its acquisition mechanism

In addition to the assistance information broadcast in TN, it is necessary to broadcast some NTN specific information. Following the NR design, some NTN specific information is contained in SIB19 and is essential for NTN access. The NTN specific information includes:

- For access to the serving cell: ephemeris, common TA, k_mac, etc.

- For measurement: t-Service, reference location, assistance information for neighbor cell, etc.

For 6G, the exact assistance information required for NTN access requires RAN1’s input firstly; and the SIB(s) in which it will be placed can be finalized after the basic system information structure is determined. Further, considering the ephemeris and common TA are constantly changing, a valid time based SIB acquisition mechanism is supported in NR. For 6GR, this mechanism can be reused, if needed.

Beam specific SIB mechanism

Given that an NTN cell has wide coverage, different areas of it may require different configurations. In NR, some area-related mechanisms are supported:

- Location-based SMTC selection mechanism: up to 7 SMTC offsets can be configured per frequency, each corresponding to a reference location (related to a neighbor cell). The UE measures those SMTCs whose reference location proximate in distance;

- Intended service area (ISA) for MBS broadcast: GNSS-defined ISAs are configured for an MBS session; only the UE within the ISAs can receive the MBS broadcast service;

- Geo-fencing for PWS SIBs: warning area is configured within PWS SIBs; only the UE within the warning area indicates the warning message to the user [2];

These area-related mechanisms rely on broadcast location information in SIBs, inevitably bringing large signaling overhead and energy consumption. For 6G, the pain point can be alleviated by a beam specific SIB mechanism. An example is given in the figure below. With this approach:

- Each beam configures SMTCs only for its nearby neighbor cells, e.g., Beam#2 configure SMTCs only for Cell#2 and Cell#3, so the signaling for irrelevant SMTCs and corresponding location information is saved;

- For MBS broadcast and PWS: if the intended area matches with one or more NTN beams (e.g., Beam#2), the network provides configuration only in the corresponding beam(s), achieving geo-fencing naturally; even if it does not match, each beam broadcasts only the intended areas within its scope, so the signaling overhead is reduced significantly.

Figure 1. NTN beam deployment scenario

Observation 4: With beam-specific SIB(s), the location related information, e.g., for location-based SMTC selection and PWS geofencing, can be reduced or omitted. Thus, signalling overhead and energy consumption can be efficiently reduced using beam-specific SIB mechanism.

Proposal 7: To achieve more efficiently SI broadcasting and lower energy consumption for NTN, 6GR should study the mechanism of beam-specific SIB.

- Conclusion

According to the above analysis, it is proposed:

Broadcast of Positioning Assistance Data

Proposal 1: RAN2 study the design of a flexible broadcast mechanism for positioning assistance data (posSIBs) in 6GR, as part of the overall system information framework. The study shall focus on ensuring that the SI design can natively accommodate the distinct characteristics of posSIBs (e.g., periodicity, message size, validity, and update mechanisms) from the initial design phase.

On-demand SIB1

Proposal 2: For 6GR OD-SIB1, RAN2 discuss the following scenarios:

⁻ Standalone scenario: RAN2 shall wait RAN1’s conclusion on whether/how to support OD-SIB1 in standalone scenario before proceeding with the OD-SIB1 procedure design for this scenario.

⁻ Inter-cell assistance scenario: RAN2 study the scenario that Cell A assisted OD-SIB1 procedure for Cell B. The Case 2 in R19 NES WI can be the starting point, where Cell A transmits the OD-SIB1 configuration and Cell B receives preamble and transmits OD-SIB1.

On-demand SI

Observation 1: On-demand SI can achieve energy efficiency. Compared with Msg3-based on-demand SI in IDLE/INACTIVE, Msg1-based method provides lower latency but occupies more RACH resources. And the supported on-demand SIBs in CONNECTED include posSIBs, SIB22 for ATG and so on.

Proposal 3: RAN2 study on-demand SI in non-connected mode for 6GR only within Msg3-based method, and on-demand SIBs in connected mode for 6GR.

SI scheduling mechanism

Observation 2: NR SI window mechanism can provide flexible scheduling and implementation simplification. In addition, an explicit start position of SI window was introduced in NR Rel-17, which can achieve more flexible and efficient scheduling. But SI windows of different SI messages do not overlap in NR, with which the clustered SI message transmission cannot be achieved.

Proposal 4: RAN2 support SI window for SI scheduling in 6GR. Furthermore, RAN2 study:

- an explicit start position of SI window to enable more flexible and efficient scheduling of SI,

- how to support overlapping of SI windows to achieve more sleeping opportunities of network for energy efficiency.

SIB1 contents

Observation 3: The information for camping, e.g., cell selection related information, cell access related information, and cell bar information if available, is crucial and necessary for all UEs camping on one cell. But other information, e.g., the information for initial access, is not crucial for all UEs camping on the cell.

Proposal 5: sub-SIB1 or sub-RMSI in 6GR at least contains the information for camping, including cell selection related information, cell access related information, and cell bar information if available.

SIB validity

Proposal 6: Consider the area-specific SI mechanism in NR as a starting point.

NTN aspects

Observation 4: With beam-specific SIB(s), the location related information, e.g., for location-based SMTC selection and PWS geofencing, can be reduced or omitted. Thus, signalling overhead and energy consumption can be efficiently reduced using beam-specific SIB mechanism.

Proposal 7: To achieve more efficiently SI broadcasting and lower energy consumption for NTN, 6GR should study the mechanism of beam-specific SIB.

- References

- TR 38.914  Study on 6G Scenarios and Requirements (Rel-20), v0.2.0

- TS 23.041 Technical realization of Cell Broadcast Service (CBS)

- RP-253806 Moderator's summary for 6G SI: requirements of new and existing services  CMCC RAN#110