3GPP TSG-RAN WG2 Meeting NB-IOT ad-hoc	R2-164035

Nanjing, China, May 23 – 27, 2016

Agenda Item:	7.14.2.2

Source:	Ericsson

Title:	SI-message specific repetition levels for NB-IoT

Document for:	Discussion, Decision

# Introduction

Related to System Information scheduling it was agreed at RAN2#92 that [1]:

- The UE is not required to accumulate several SI messages in Parallel.

Further, it was agreed in RAN1#Ad-hoc2 that [2]:

  - One transport block of an SI message is transmitted over 8 consecutive valid DL subframes.

In the way-forward from RAN2#93bis agreed that [3]:

  - The TBS of NB-SIB1 and other SIBs are {208, FFS, FFS, 680}. We will reconsider the FFS-marked values when SI contents has been settled

  - The repetition is the same as for eMTC (number of repetitions, timer interval between repetitions).

Agreements for RAN2#93tris-Ad-hoc [3]:

  - System Information message is repeated once in {every 2nd, every 4th, every 8th, every 16th} radio frame within an SI window.

  - On TB size for SI messages, we stick with the currently agreed values (from R1).

  - The value range of si-Periodicity for NB-IoT is ENUMERATED {rf64, r128, rf256, rf512, rf1024, rf2048, rf4096}.

  - Up to 8 SI-messages can be configured for NB-IoT.

  - On the smaller TB sizes for SI messages:

    - RAN2 ask R1 to consider additional TB sizes according to R2-163293

    - IF RAN1 cannot converge on this RAN2 will use the previous RAN1 agreements.

    - We will have an email discussion to agree on the  LS to R1

In this contribution, the remaining aspects with regards to System Information scheduling for NB-IoT is discussed.

# Discussion

For SI-message scheduling, the remaining open issues are the inclusion of TBSs smaller than 208 bits and whether the repetition indication should be per SI-message or common to all SI-messages. Regarding the first issue, an LS has been sent to RAN1 and the potential inclusion of smaller TBSs is now up to RAN1. The second issue, i.e. SI-message repetition indication, is discussed in this contribution.

## System Information message repetition

For NB-IoT it has been agreed to define the SI repetitions in the same way as for eMTC: “The repetition is the same as for eMTC (number of repetitions, timer interval between repetitions)”, which was done with after some necessary adoption to NB-IoT, i.e. ‘System Information message is repeated once in {every 2nd, every 4th, every 8th, every 16th} radio frame within an SI window.’ However, this agreement does not mention whether the repetition is indicated per SI-message or if it is common to all SI-messages and this remains to be resolved for NB-IoT.

If the repetition is common to all SI-messages, configuration per SI message can only be done via si-Periodicity which is quite limiting as described further below.

### SI-message repetition needs

Table 1 below visualizes the MCS/TBS table for NB-IoT. The TBS values 208 and 680 during 8 subframes have so far been agreed for SI-messages. As different SI-messages have different contents/sizes, it can be understood that different TBSs will be chosen for the different SI-messages. As a consequence, these different SI-messages will have different MCSs. For example in the in-band scenario and disregarding non-valid subframes for the sake of simplicity, an SI-message that has TBS 208 has roughly the coding rate 0.13, whereas an SI-message that has TBS 680 has roughly the coding rate 0.4. It can then be understood that the SI-message with TBS 208 has considerably more redundancy embedded than the SI-message with TBS 680 and needs not be repeated as many times as the latter SI-message for reaching the same coverage level.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 | 328 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 | 504 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 | 776 | 872 |
| 6 | 328 88 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | 936 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 680 | 840 | 968 | 1096 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 | 1256 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 | 1416 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1224 | 1384 | 1544 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1384 | 1608 | 1800 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1608 | 1800 | 2024 | 2280 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 | 1800 | 2024 | 2280 | 2536 |

Table 1: MCS/TBS table for NPDSCH where valid entries are highlighted. The green-coloured entries have so far been decided for SI-messages.

Different SI-messages will have different MCSs and require different amount of repetition for reaching the same amount of coverage.

One might think that with a common repetition, still si-Periodicity can be used as a tool to configure the system information broadcast. However, combinations of a common repetition factor and different si-Periodicity leads either to excessive amount of repetition as outlined in Figure 1a) and unnecessarily waste of DL resources or leads to the requirement the UEs needs to collect SI-messages across si-Windows as seen in Figure 1b).  However, due to simplicity of the NB-IoT UEs and the limited amount of memory thereof and the agreements from RAN2#92, UEs will most probably not accumulate SI message across multiple SI-windows in parallel but rather treat SI-messages one-by-one sequentially across si-Windows. This leads to excessive amount of delay and battery consumption for acquiring SI-messages compared to having suitable repetition rate per SI-message. Further, the UE implementation becomes more complex as a result of having to consider various modification periods for different SIB/SIs during such acquisition. This is visualized in Figure 1 below.

Using si-Periodicity as the only tool for configuring SI-specific schedule leads to either non-optimal DL resource usage or complex UE implementation and delays in the system.

Figure 1: Schematic illustration of SI-message scheduling with SI-message common amount of repetition. Figure a) illustrates the case of repetitions in every 2nd radio frame and b) the case of repetitions in every 8th radio frame.

It shall be noted that being able to configure repetition per SI message is also in line with legacy LTE, where the network via PDCCH can dynamically schedule different amounts of transmissions per si-Window for different SI-messages.

In legacy LTE, the amount of repetition could be controlled per SI-message.

It is also important to note that compared to other features/functionality, the nature of the SI broadcast scheduling is such that what is decided in Rel-13 will have to be valid for all future releases if backwards compatibility is considered.

The nature of the SI broadcast scheduling is such that what is decided in Rel-13 will have to be valid for all future releases if backwards compatibility is considered.

The only drawback of allowing SI-specific number of repetitions is the cost of 2 SIB1-NB bits per SI-message for SI-messages after the first SI-message, e.g. 6 bits instead of 2 bits when 3 SI-messages are used.

Therefore we propose the following:

si-RepetitionPattern-r13 is indicated per SI-message in SIB1-NB.

## ASN.1 proposal

Including the repetition indication per SI-message would only increase the SIB1-nb size by two bits per SIB-message after the first one. This can be seen from the below text proposal:

SystemInformationBlockType1-NB message

-- ASN1START

SystemInformationBlockType1-NB ::=	SEQUENCE {

hyperSFN-MSB-r13					BIT STRING (SIZE (8)),

cellAccessRelatedInfo-r13			SEQUENCE {

plmn-IdentityList-r13				PLMN-IdentityList-NB-r13,

trackingAreaCode-r13				TrackingAreaCode,

cellIdentity-r13					CellIdentity,

cellBarred-r13						ENUMERATED {barred, notBarred},

intraFreqReselection-r13			ENUMERATED {allowed, notAllowed}

},

cellSelectionInfo-r13				SEQUENCE {

q-RxLevMin-r13						Q-RxLevMin-NB-r13,

q-QualMin-r13						Q-QualMin-NB-r13

},

p-Max-r13							P-Max-NB-r13					OPTIONAL,	-- Need OP

freqBandIndicator-r13				FreqBandIndicator-NB-r13,

freqBandInfo-r13					NS-PmaxList-NB-r13				OPTIONAL,	-- Need OR

multiBandInfoList-r13				MultiBandInfoList-NB-r13		OPTIONAL,	-- Need OR

downlinkBitmap-r13					DL-Bitmap-NB-r13,

eutraControlRegionSize-r13			ENUMERATED {n1, n2, n3}			OPTIONAL,	-- Cond inband

nrs-CRS-PowerOffset-r13				ENUMERATED {dB-6,      dB-4dot77, dB-3,

dB-1dot77, dB0,       dB1,

dB1dot23,  dB2,       dB3,

dB4,       dB4dot23,  dB5,

dB6,       dB7,       dB8,

dB9}		OPTIONAL,	-- Cond inband-samePCI

schedulingInfoList-r13				SchedulingInfoList-NB-r13,

si-WindowLength-r13					ENUMERATED {ms160,  ms320,  ms480,	ms640,

ms960,	ms1280, ms1600, spare1},

si-RadioFrameOffset-r13				INTEGER (1..15)		OPTIONAL,	-- Need OP

systemInfoValueTagList-r13			SystemInfoValueTagList-NB-r13	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

PLMN-IdentityList-NB-r13 ::=		SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-NB-r13

PLMN-IdentityInfo-NB-r13 ::=		SEQUENCE {

plmn-Identity						PLMN-Identity,

cellReservedForOperatorUse			ENUMERATED {reserved, notReserved},

attachWithoutPDN-Connectivity		ENUMERATED {true}	OPTIONAL	-- Need OP

}

SchedulingInfoList-NB-r13 ::= SEQUENCE (SIZE (1..maxSI-Message-NB-r13)) OF SchedulingInfo-NB-r13

SchedulingInfo-NB-r13::=		SEQUENCE {

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

sib-MappingInfo-r13				SIB-MappingInfo-NB-r13,

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF,

every8thRF,  every16thRF},

si-TBS-r13						ENUMERATED {b208, b-TBD1, b-TBD2, b680}

}

SystemInfoValueTagList-NB-r13 ::= 	SEQUENCE (SIZE (1.. maxSI-Message-NB-r13)) OF

SystemInfoValueTagSI-r13

SIB-MappingInfo-NB-r13 ::= 			SEQUENCE (SIZE (0..maxSIB-1-NB-r13)) OF SIB-Type-NB-r13

SIB-Type-NB-r13 ::=					ENUMERATED {

sibType3-NB-r13, sibType4-NB-r13, sibType5-NB-r13,

sibType14-NB-r13, sibType16-NB-r13, spare3, spare2, spare1}

-- ASN1STOP

# Conclusion

In this contribution we have discussed the access control for NB-IoT. In section 2 we made the following observations and proposal:

- Different SI-messages will have different MCSs and require different amount of repetition for reaching the same amount of coverage.

Using si-Periodicity as the only tool for configuring SI-specific schedule leads to either non-optimal DL resource usage or complex UE implementation and delays in the system.

In legacy LTE, the amount of repetition could be controlled per SI-message.

The nature of the SI broadcast scheduling is such that what is decided in Rel-13 will have to be valid for all future releases if backwards compatibility is considered.

Proposal 1	si-RepetitionPattern-r13 is indicated per SI-message in SIB1-NB.

# References

R2-161067, Draft Report of 3GPP TSG RAN WG2 meeting #92 v2.0, Anaheim, USA, November 16 – 20, 2015

R1-163379, Final Report of 3GPP TSG RAN WG1 #2AH_NB-IoT v1.0.0, Sophia Antipolis, France, 22nd – 24th March 2016.

R2-163301, Draft Report of 3GPP TSG RAN WG2 meeting #93bis, Dubrovnik, Croatia, 11th – 15th April 2016.

Draft Report of 3GPP TSG-RAN WG2 NB-IOT Ad-hoc Meeting#2, Sophia-Antipolis, France, May 3 – 4 2016.

R1-156018, Coverage analysis in inband operation, Ericsson, Malmö, Sweden, 5th – 9th October, 2015.

R1-163406, Final Report of 3GPP TSG RAN WG1 #84 v2.0.0, Malta, 15th – 19th February 2016.

208 bits / ( (100 REs * 2 bits) * 8 subframes)