3GPP TSG-RAN WG2 Meeting #94	R2-164138

Nanjing, China, May 23 – May 27, 2016

Agenda item:		9.4.3.2.5 (FS_NR_newRAT)

Source:	LG Electronics Inc.

Title: 	Fast Uplink Channel

Document for:	Discussion and Decision

1.	Introduction

One of the Key Performance Indicators (KPI) in 5G is “user plane latency”. In [1], various kinds of user plane latency are defined, e.g. latency for URLLC, eMBB, and infrequent small packets.

The latency target for eMBB is 4ms for both UL and DL. For URLLC, the latency target is more stringent, i.e. 0.5ms for both UL and DL. These latency targets are pursued when the UE and the eNB are not restricted by DRX.

The latency for infrequent small packets is defined as “the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most "battery efficient" state.” Considering in terms of LTE, the battery efficient state is seen as the UE in RRC Idle or in Long DRX state. In this state, it would take more time for the UE before transmitting UL packets. The [1] also suggests to minimize UL latency in this case.

The document proposes a new UL channel that can ensure lower latency even if the UE is in battery efficient state.

2.	Fast UL Channel

The UE in RRC Connected state may be put in Long DRX state to save UE battery. In this state, the UE is likely to be configured with sparse SR or even no SR in order for the eNB to save SR resource. If UL data occurs in this state, the UE has to perform (RA or SR) / BSR / UL TX to transmit UL data. The study in Rel-13 [2] shows that it takes about 12.5ms even with 1ms SR period. This latency is not suitable for the 5G system.

The UE in RRC Idle requires more time before transmitting UL packets. This is because the UE has to first perform RA procedure to make RRC connection, and then establish RB to transmit UL packets. It would take about 50ms more than UL latency in RRC Connected.

In order to minimize UL latency for UEs in both RRC Connected and RRC Idle, it is proposed to introduce a new type of radio channel, called Fast UL Channel (FUCH).

We think the FUCH has following characteristics:

-	FUCH is always available channel in a cell

-	FUCH can be used by UEs in any state

-	FUCH is a contention based channel

-	FUCH configuration is broadcast by System Information

-	Multiple FUCHs can be configured in a cell

If such a Fast UL Channel is introduced, the UE can send UL packets anytime it wants regardless of UE states. Though the FUCH is subject to contention, it is possible to minimize the contention by proper grouping of UEs that can use a specific FUCH.

As the user plane latency is one of important KPI in 5G, we propose to consider FUCH as a key component of 5G.

Proposal: RAN2 studies always available Fast UL Channel to minimize UL latency in any UE state.

3.	Proposal

In this document, we explained the need for Fast UL Channel in order to minimize UL latency regardless of UE state.

Proposal: RAN2 studies always available Fast UL Channel to minimize UL latency in any UE state.

4.	References

[1] TR38.913 Study on Scenarios and Requirements for Next Generation Access Technologies

[2] TR36.881 Study on latency reduction techniques for LTE