3GPP TSG RAN WG2 Meeting #94				  	          R2-163726

Nanjing, China, 23th – 27th May 2016

Agenda item: 9.2 Study on New Radio Access Technology: General Aspects

Source: National Taiwan University

Title: Multicast and Broadcast in NextGen RAN

Document for: Discussion and Decision

- Introduction

This contribution proposes extending multicast and broadcast capabilities by the introduction of new reception mode.

Flexibility and adaptation is a distinguishing feature in the 5G next generation system. TR 22.864 [1] includes 6 flexibility enablers for the 5G system to provide optimized support for different configuration and various means. For broadcast service, system flexibility includes the resource allocation and simultaneous unicast and broadcast.

TR 22.864 5.1 System flexibility

Flexibility enabler 4:  Flexible broadcast service

…

Today, 3GPP specifications do not allow a stand-alone deployment of a multicast/broadcast network.  Additionally, the current implementation limits the radio resources that can be allocated for multicast/broadcast service to 60%.  It also limits the coverage size of the radio base station to a relatively small value. In order to support a variety of linear video based services for the users, deployment of the next generation of multicast/broadcast capabilities will require significantly more flexibility than what is available in the current network in terms of configuration, resource allocation and network deployment to support broadcast/multicast services. The flexibility required encompasses allocation of resources, multicast/broadcast network design as well as simultaneous user access to unicast data and broadcast service (cp. [2], clause 5.70).

SA1 also specified the enhancements of the multicast and broadcast. Enhanced broadcast management system and ad-hoc broadcasting will be introduced in the NextGen system operation [2]. To capture the flexibility requirement of multicast and broadcast service, current multicast reception and configuration should be investigated.

2. Discussion on the enhancement of multicast and broadcast

MBMS reception in RRC_Idle and RRC_Connected mode are specified in TS25.346 [3]. Besides, TR 23.799 [4] also claims the broadcast multicast capabilities in 5G system. Group configuration and various services should be supported architecturally.

TR 23.799 5.13 Key Issue 13: Broadcast Multicast Capabilties

Specific solutions proposed under this key issue include those needed to architecturally support:

-	1:many and 1:all communication involving 5G capabilities specified by RAN;

-	Group handling (e.g. geographic scope of groups);

-	Group communication (e.g. setup performance, services to the group member, group member requests of the system, priority and pre-emption of group communication, services provided during an ongoing group communication, and user perception of group communication);

-	Functionality and performance needed to satisfy current and emerging 3GPP broadcast/multicast service requirements and application architecture requirements (e.g. MCPTT, MCVideo, MCData, CriC, and massive MTC);

-	Consideration of related NextGen architecture key issues (e.g. session management, QoS framework).

Therefore, we consider two modes for the multicast and broadcast service in NextGen RAN.

Mode 1: Support multicast and broadcast reception in both RRC_Connected and RRC_Idle mode

Mode 2: Support multicast and broadcast reception in RRC_Connected mode only

Mode 1 multicast and broadcast is similar to current LTE eMBMS. Mode 2 multicast and broadcast might include tighter feedback and adaptation. It will be easier to provide flexible broadcast transmission and adaptive group communications when UEs are in RRC_Connected mode. For example, massive MIMO capability could be further exploited to provide high-rate multicast and broadcast in NR. With feedback from RRC_Connected mode UEs, Mode 2 multicast and broadcast scheme could perform semi-dynamic beamforming to improve throughput [5].

To support the key capabilities in the NextGen RAN and the high-rate multicast utilizing massive MIMO, we propose 3 options for the multicast and broadcast.

Proposal :  Consider the potential design options for the NR multicast and broadcast services

Option 1: NextGen RAN supports only Mode 1 multicast and broadcast

Option 2: NextGen RAN supports only Mode 2 multicast and broadcast

Option 3: NextGen RAN supports two multicast and broadcast modes (Mode 1 and Mode 2).

3. Conclusions

In this contribution, we propose that 3 options may be considered for multicast reception.

Proposal :  Consider the potential design options for the NR multicast and broadcast services

Option 1: NextGen RAN supports only Mode 1 multicast and broadcast

Option 2: NextGen RAN supports only Mode 2 multicast and broadcast

Option 3: NextGen RAN supports two multicast and broadcast modes (Mode 1 and Mode 2).

4. Reference

- 3GPP TR 22.864 V1.0.0, “Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1,” 2016 Mar.

- 3GPP TR 22.891 V14.0.0, “Study on New Services and Markets Technology Enablers,” 2016 Mar.

- 3GPP TS 25.346 V13.0.0, “Introduction of the Multimedia Broadcast/Multicast Service (MBMS) in the Radio Access Network (RAN),” 2015 Dec.

- 3GPP TR 23.799 V0.4.0, “Study on Architecture for Next Generation System,” 2016 Apr.

- 3GPP R1-162182, “MIMO for 5G new radio interface: sub-6GHz,” 2016 Apr.