3GPP TSG-RAN WG2 Meeting #95	R2-164790

Gothenburg, Sweden, 22 - 26 August 2016	R2-163440

Agenda item:	9.4.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Network Convergence Sublayer for NR

WID/SID:	FS_ NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

In the context of New Radio, some improvement of Layer 2 for User Plane can be achieved. One of the area is the Network Convergence Sublayer that would replace the PDCP sublayer of LTE. In this contribution we present some basic requirements and principles that should apply to this sublayer. It was also agreed during TSG RAN WG2 meeting #94 that the "data radio bearer" (DRB) defines the Over-The-Air packet treatments in the RAN.

# 2	NCS Architecture and functions

## 2.1	QoS Architecture

The proposed QoS framework for NR is presented in [1].

In the present contribution we propose an architecture of NCS (Network Convergence Sublayer) that replaces PDCP sublayer of E-UTRA and allows such framework [2]. The proposed NCS layer would not only replace PDCP for the L2 handling but would also be in charge of some QoS tasks as described below. The focus in this contribution is on the User Plane data handling, the Control Plane part, signalling and configuration is not covered.

## 2.2	NCS Role

As described in [1], the assumption is that the CN will deliver to RAN a set of PDU Sessions. Within each PDU Session, different sub flows are present, each one with possible different QoS characteristics. The RAN receives PDUs from Core Network with FII (Flow Identification Indicator) attached to them. The FII marking gives an indication on how to treat the PDU from QoS point of view. Some further information could be added inside the Radio Access Network (for example by the Content requirement Awareness Functions (CAF)).

The different sub flows with different QoS characteristics in the PDU session needs to be split into different DRB so that they can be differentiated from QoS point of view. We propose that this task is performed by NCS.

Figure 1 shows the internal split of NCS sublayer. The first function of NCS will be to distribute the PDUs from PDU session into different DRB, according to the QoS information associated with the PDU. The second function will be to handle the DRB. The PDU are then delivered to lower layer (RCS) in QoS Queues (see [2]). As the NCS entity is in charge of a PDU Session, there could be several NCS entity in a gNB.

In UL, the UE will need to split the data flow from NAS into the DRBs based on information linked to the PDU or other schemes like reflective QoS. This task is already done in the UE in LTE with the use of TFT ([3]).

NOTE:	whether the UE needs to be aware of PDU sessions is currently FFS.

Figure 1: NCS internal architecture

Proposal 1: NCS sub layer can split the data flow from NAS into Data Radio Bearers, based on information associated with the PDUs.

Each Data Radio Bearer is then handled separately in a NCS sub-entity in a similar way as in LTE. The allocation of SN will be done at this level, so that different QoS can be handled within a PDU Session. Indeed, the SN allocation imply that all the PDU with the same numbering scheme will be delivered in order or at least within a SN window, preventing any QoS differentiation.

Proposal 2: Sequence number is allocated at DRB level.

Figure 2: Handling of DRB, network side

Figure 3: Handling of DRB, UE side

Figure 2 and Figure 3 show more details about the functions of NCS for DRB. It includes the functions that were performed for a Radio Bearer in LTE:

-	Ciphering;

-	Integrity protection;

-	Header compression;

-	Reordering.

Proposal 3: Ciphering, integrity protection, header compression, reordering is performed per DRB.

# 3	Higher layer multi-connectivity

LTE supports Dual Connectivity, it is natural that NR supports the corresponding feature it as well. We give a more detailed description of it in [4]. Note that the proposed architecture for NCS does not preclude any lower layer architecture (RCS, MAC etc.). In LTE, split bearer for dual connectivity is based on PDCP. We think that the same approach should be taken in NR.

Proposal 4: Higher layer multi-connectivity is based on NCS

Figure 5 illustrates the routing and / or duplication function that is added in NCS in order to support split bearers for Dual Connectivity.

Figure 5: NCS based Dual Connectivity

The routing and / or duplication function, located in DRB handling part of NCS sublayer. This function is responsible of routeing the NCS PDU of a split bearer to the correct RCS queue. It may also duplicate the packets if configured.

In the previous generation (LTE) Dual Connectivity was limited to AM mode ([5]). We think that there is no reason to restrict it for NR and thus, we have the following proposal:

Proposal 5: Higher layer multi-connectivity for NR shall support both UM and AM traffic.

Some contribution were proposed to support this for PDCP ([6]).

# 4	Conclusion

In this contribution, we have proposed a structure of NCS layer that would support some principles that will be introduced in NR.

Proposal 1: NCS sub layer can split the data flow from NAS into Data Radio Bearers, based on information associated with the PDUs.

Proposal 2: Sequence number is allocated at DRB level.

Proposal 3: Ciphering, integrity protection, header compression, reordering is performed per DRB.

Proposal 4: Higher layer multi-connectivity is based on NCS.

Proposal 5: Higher layer multi-connectivity for NR shall support both UM and AM traffic.

# References

- R2-164789 Radio bearer management principles, Nokia, Alcatel-Lucent Shanghai Bell

- R2-163439 UP Radio Protocols for NR, Nokia, Alcatel-Lucent Shanghai Bell

- 3GPP TS 24.301 Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3

- R2-163687 Multi-connectivity in standalone NR, Nokia, Alcatel-Lucent Shanghai Bell

- R2-164787 Upper layer aggregation for NR-NR multi-connectivity, Alcatel-Lucent Shanghai Bell

- R2-141854 Report of 3GPP TSG RAN WG2 meeting #85, held in Prague, Czech Republic