3GPP TSG-RAN WG2 Meeting #96	R2-168424

Reno, USA, 14 - 18 November 2016

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Considerations on DL and UL mobility in NR

Agenda Item:	9.3.1.1.1

Document for:	Discussion

# 1	Introduction

Following agreements were made during RAN1#86:

Agreements: - For RRM measurement in NR, at least DL measurement is supported with the consideration on - Both single-beam based operation and multi-beam based operation - FFS: Definition of RRM measurement for multi-beam based operation - FFS: DL signal for RRM measurement - FFS: When DL measurement is applied - Note that there is no conclusion that DL measurement is a complete solution for RRM measurement in NR for now

Following agreements have been achieved during RAN1#86bis:

Agreements on R1-1610975 WF on DL measurement for L3 mobility Samsung, LG Electronics, NTT DOCOMO, AT&T, Ericsson, Intel: - Note: In this WF, IDLE mode refers to a UE state similar to LTE IDLE state, whose exact definition is up to RAN2 - Note: In this WF, CONNECTED mode refers to a UE state similar to LTE CONNECTED state, whose exact definition is up to RAN2 - Note: In this WF, cell refers to NR cell which is tied to a same ID carried by NR-SS. - Detailed definition of NR cell FFS - NR supports cell-level mobility based on DL cell-level measurement (e.g. RSRP for each cell) in IDLE mode UE

and throughout RAN2#95bis the following agreements have been reached:

Agreement 1: Concerning RRC driven UL-based connected mode mobility: • For connected active state mobility, DL-based handover is supported, and UL based mobility can continue to be studied. • For connected inactive state, DL-based reselection is supported, and UL-based mobility can also be studied • Benefits of UL based mobility, compared to DL based mobility, should be studied with performance analysis.

Based on the general agreements from RAN1#86,RAN1#86bis and RA2#95bis, related to mobility it seems that there is a common understanding that at least DL mobility would be useful to have.

In this paper, we will discuss on benefits and drawbacks of having both DL mobility and UL mobility. In addition, we will discuss in which scenario(s) the mobility schemes would be most beneficial to have.

# 2	Mobility

Currently in LTE, both idle and connected mode mobility are based on UE DL measurements. For connected mode, measurement results are reported to the network, which in turn commands the UE to perform handover if needed. In Idle mode, the UE changes cell autonomously without any need for interaction with the network (in most cases). Based on the agreements made in RAN1#86, this would appear to be the baseline also for NR.

The key characteristics or KPIs of interest in NR, under the considered scenarios, are the reliability/availability (e.g. handover performance) and user experience (e.g. outage time). These KPIs are highly influenced by the cell change latency in idle mode and connected mode – which in general needs to be minimized. A long cell change time may affect the UE paging reception, and in connected mode long cell change time may impact the network’s ability to schedule the UEs [2].

The primary motivation for UL based mobility as proposed in [4], [5], [7] and [8] seems to be the more beneficial trade-off between UE power consumption and paging delay in idle or RRC-inactive states. Moreover, UL based mobility is seen mostly useful in scenarios where the number of UEs per TRP is small, as otherwise the overhead from UL reference signal transmissions becomes substantial.

In this paper, we will discuss separately both DL and UL based mobility in such a setting. We will also present system simulation results showing the paging performance with DL based mobility.

## 2.1	Scenario

In the TR on scenarios and requirements for next generation access technologies [1] the high-level assumptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD, BS / UE antenna elements, UE distribution / speed and service profile are proposed. The scenarios include, among others, high speed train and high speed rural scenarios. The details of the scenarios are listed in [1]. Both of these scenarios address high or very high mobility in macro cell deployment (i.e. with ISD of 1732m). The KPIs of interest for these scenarios include the reliability/availability (e.g. handover performance) and user experience (e.g. outage time) [2].

| Figure 1 High speed rural [3] | Figure 2 High speed train scenario for 4GHz deployment [1] |
| --- | --- |

In case of high speed rural [3] (see Figure 1) with UE speeds up to 120 km/h is a reasonable scenario where UL mobility could potentially become useful. We’ll investigate the performance and suitability of DL and UL based mobility in this type of scenario. We consider both the 120 km/h and an extreme 480 km/h UE speeds in the simulations to get an understanding of the absolute worst case performance.

The high speed train (HST) scenario (as shown in Figure 2), on the other hand, seems somewhat misdirected in this context. If all the ~1000 UEs in the fast-moving train have a direct connection to the cells deployed along the track, there is unnecessarily high signal overhead due to the frequent cell changes: Tracking area updates in the case of DL based mobility, UL reference signal transmissions and (potential) network responses to those in case of UL based mobility. Moreover, the penetration loss due to the UEs being inside the train would pose a further challenge. Thus, a more reasonable deployment would be based on a fixed deployment of a relay or RRH within the train. This would no longer have the power limitation for the UEs and therefore it would not need to save power with idle/inactive state connection to the network along the track. The UEs inside the train would then be stationary relative to this relay/RRH, what in turn means that DL based mobility would be sufficient. Due to these reasons, in this paper we will focus on the closer analysis of high speed rural scenario instead of the HST scenario.

If proper evaluation of mobility in the HST scenario is desired, then the agreed assumptions from [1] should be used considering also the relay deployment. Macro only deployment (as assumed in [3]) is not ideal in this case, so instead of introducing a complex feature for a rare scenario, it could be accepted that the UE paging delay may be slightly impacted (the simulation results presented in this paper show that performance impact is small).

## 2.2	DL based mobility

When UE is paged, the paging may be missed because either the signal quality is low, or because the UE is not listening to the cell(s) where it is being paged. Thus, it is important that the cell reselections as well as tracking area updates are done without unnecessary delay.

The delay is affected by the DRX or paging cycle used. With shorter cycle, the network has more frequent paging occasions so the expected paging delay is shorter. Moreover, with shorter DRX cycle, the UE makes more frequent (neighbour cell) measurements and thus the expected cell reselection delay is also shorter. This also makes the expected paging miss rate lower, but means higher UE power consumption due to more frequent measurements and monitoring for paging.

When the cell reselection happens late and the signal quality of the serving cell drops too low, the UE may miss the paging due to this low signal quality.

When the UE has reselected a cell that is in a different tracking area, but hasn’t done tracking area update yet, the paging may be missed as the network does not know it should page the UE in the new serving cell belonging to another tracking area. However, with typical longish paging cycles (hundreds of milliseconds), this typically would not be an issue, as there would be plenty of time to complete the TAU before the next DRX/paging cycle.

## 2.2.1	DL mobility results

We conducted system level simulations in order to study the effect of paging-miss and paging delay as a function of DRX/paging cycle and system load. The scenario we used is the ‘Rural with high speed UEs’ from [3], for two different speeds: 120 km/h and 480 km/h. We deployed 21 macro cells, with ISD 1732 m, carrier frequency 4 GHz and 20 MHz bandwidth.

In our analysis, we have varied the network load from low to high (i.e. interference level inside own network). This reflects also the PDCCH interference.

Paging decoding is considered successful if instantaneous PDCCH SINR >-7dB. More parameters are given in Annex.

| Figure 3 Cell reselections/UE/second (120km/h) | Figure 4 Cell reselections/UE/second (480km/h) |
| --- | --- |

From the cell reselections results we can observe that:

- 120 km/h: Regardless of DRX cycle and load, the number of cell reselection is quite low (between 1 cell reselection in 6 s for DRX cycle 375 ms down to 1 cell reselection in 20 s for DRX cycle 1.5 s)

- 480 km/h: Similar observation as for 120 km/h, but the number of cell reselections is clearly higher (around 1 cell re-selection in 5 s)

| Figure 5 Paging miss rate for 120 km/h | Figure 6 Paging miss rate for 480 km/h |
| --- | --- |

From the paging-miss rate results we can observe that:

- Increasing the DRX cycle, the paging miss rates remain rather low to pose any problems in DL

- 120 km/h: very low page-miss rates (at most 2.5%)

- 480 km/h: low paging-miss rate for DRX cycle 375 ms (up to 2%, when the interference in the network is quite high), while for DRX cycle 1.5 s the paging-miss rate is under 10% (for  high interference in own network)

| Figure 7 Average paging delay, for 120km/h | Figure 8 Average paging delay, for 480 km/h |
| --- | --- |

The average delay depends primarily on the DRX/paging cycle length (i.e. on average the delay is at least half the paging cycle), less on the paging miss rate. For this reason even getting zero paging miss rates with UL based mobility, we cannot really go for much longer paging cycles without significantly increasing the average paging delay.

Observation 1: DL based mobility provides good paging performance even for high speed UEs.

Even if somewhat smaller paging miss rates could be achieved with UL based mobility, the average paging delay is primarily determined by the DRX/paging cycle.

Observation 2: Lower paging miss rate doesn’t significantly improve paging delay which follows basically DRX cycle length.

## 2.3	UL based mobility

In UL based mobility, the UE could be paged faster after a cell reselection/cell change. This is because there would not be need to wait until the next DRX/paging cycle as the paging could be done from the new cell right after measuring UE’s reference signal transmission.

However, even though this can lead to smaller paging miss rate than DL mobility (where we saw that in high speed rural scenario the paging miss rates were below 3 % for 120 km/h), it does not mean that substantially longer paging cycle could be used with UL based mobility. As an example 0 % paging miss rate with 1500 ms paging cycle means on average 750 ms paging delay (from the point of traffic arrival to the point when UE can be paged), whereas 10 % paging miss rate with 750 ms paging cycle means on average roughly only 450 ms paging delay. So to get the same average paging delay, relatively short paging cycle needs to be used with UL based mobility as well. As the results in section 2.2.1 show the average paging delay is primarily determined by the DRX/paging cycle, not by the paging miss rate.

Besides not bringing compelling performance gains, the main problem with UL based mobility for idle or inactive UEs is that it essentially means that UE is periodically polling the network for data. This is not a desirable. The overhead from UL reference signal transmissions from the UEs and responses by the network is also a problem, quickly compounding if the number of UEs per TRP grows.

# 3	Conclusions

In this paper we discussed UL and DL based mobility. In particular focusing on UE in idle or inactive state. We investigated DL based mobility performance using system simulations and made the following observations.

Observation 1: DL based mobility provides good paging performance even for high speed UEs.

Observation 2: Lower paging miss rate doesn’t significantly improve average paging delay which follows DRX cycle length.

Based on the above discussion and analysis, we think that DL based mobility gives very good paging performance in the high speed rural scenario at 120 km/h and reasonably good performance even up to extreme UE speed of 480 km/h. Therefore, we make the following proposals.

Proposal 1: Rely primarily on DL based mobility

Proposal 2: Need for UL based mobility is FFS

# References

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP RAN TSG

- R1-1610295 “High speed mobility in NR”, Nokia, Alcatel-Lucent Shanghai Bell

- R1-168488 “Evaluation Scenarios and Assumptions for DL and UL Mobility”, Qualcomm Incorporated, Huawei, Intel

- R1-1610167 “Comparative Power Analysis between UL and DL-based Mobility”, Qualcomm Incorporated

- R1-1609437 ” Scenarios and initial evaluation for UL-based RRM measurement and mobility”, Huawei, HiSilicon

- R1-1610295 “High speed mobility in NR”, Nokia, Alcatel-Lucent Shanghai Bell

- R2-167066 NR uplink measurement based mobility in the inactive state, Qualcomm Incorporated, 3GPP TSG-RAN WG2 Meeting #95bis Kaohsiung, Taiwan, 10 - 14 October 2016

- R2-166987 Benefit of UL beacon tracking for "power saving state", Huawei, Telefonica, 3GPP TSG-RAN WG2 Meeting #95bis Kaohsiung, Taiwan, 10 - 14 October 2016

- TR 36.873 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 3D channel model for LTE”, 3GPP

# Annex Simulation parameters

Table 1 General parameters

| Parameter | Assumption |
| --- | --- |
| System bandwidth | 20 MHz |
| Carrier frequency | 4.0 GHz |
| Cell layout | Hexagonal grid, 7 sites, 21 cells per site, with wrap-around |
| Network synchronization | Synchronized |
| UE mobility | 120 km/h, 480 km/h |
| UE deployment | Uniform random starting location and direction |
| Macro inter-site distance | 1.732 km |
| BS antenna height | 35 m |
| DL antenna configuration | 2x2 LMMSE-IRC |
| Macro BS Tx power | 49 dBm |
| Traffic | Keep alive |
| UE maximum Tx power | 23 dBm |
| Channel model | 3D channel model [9] |
| Pathloss model - macro | 3D-UMa [9] |
| Slow Fading standard deviation | 6 dB |
| PDCCH decoding threshold | -7 dB |
| Cell reselection | RSRQ |
| Cell reselection hysteresis | 1 dB |
| Measurement filtering | 2 DRX cycles L1 sample |
| Measurement bandwidth | 6 RBs |