3GPP TSG-RAN2 Meeting #96	R2-167638

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.306 | CR | 0512 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of HS-SCCH DRX feature in enhanced Cell Fach |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FACH_DTXDRX-Core |  |  |  |  |  | Date: |  |  | 2016-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce UE procedures on reception of an HS-SCCH order during the HS-SCCH DRX operation in CELL FACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE capability for UE procedures on reception of an HS-SCCH order during the HS-SCCH DRX operation in CELL FACH added in 4.5.3 and 5.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for HS-SCCH DRX operation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 25.212, 25.214, 25.300, 25.308, 25.331 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.5.3	FDD Physical channel parameters in downlink

Maximum number of DPCH codes to be simultaneously received

Defines the number of codes the UE is capable of receiving in parallel. For DPCH in soft/softer handover, each DPCH is only calculated once in this capability. The capability does not include codes used for S-CCPCH.

Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH)

Defines the number of physical channel bits the UE is capable of receiving. For DPCH in soft/softer handover, each DPCH is only calculated once in this capability.

The number of DPCH channel bits indicates the capability of the UE when operating in non-compressed mode.

The parameter also indicates the capability of the UE to support compressed mode by spreading factor reduction as follows. The UE shall:

-	for parameter values up to and including 9600 bits:

-	support compressed mode by spreading factor reduction when operating at any value up to the reported capability.

-	for parameter values greater than 9600 bits:

-	support compressed mode by spreading factor reduction when operating at any value up to the greater of:

-	half the reported capability; or

-	9600bits.

NOTE:	Compressed mode by spreading factor reduction is not applicable when operating at spreading factor 4.

Support for SF 512 and 80 ms TTI for DPCH

Defines whether the UE supports spreading factor 512 and 80 ms TTI in downlink DPCH or not.

Support of HS-PDSCH

Defines whether the UE supports HS-PDSCH in CELL_DCH state or not. If the UE supports HS-PDSCH in CELL_DCH state then the UE shall support also F-DPCH.

Support of HS-SCCHless HS-DSCH

Defines whether the UE supports HS-PDSCH reception in CELL_DCH without prior reception of HS-SCCH. If the UE supports HS-SCCHless HS-DSCH then the UE shall support HS-PDSCH in CELL_DCH and E-DPDCH.

Support of HS-PDSCH in CELL_FACH

Defines whether the UE supports HS-PDSCH in CELL_FACH state or not. If the UE supports HS-PDSCH in CELL_FACH then the UE shall support also HS-PDSCH in CELL_DCH and MAC-ehs.

Support of HS-PDSCH in CELL_PCH and URA_PCH

Defines whether the UE supports HS-PDSCH in CELL_PCH and URA_PCH states or not. If the UE supports HS-PDSCH in CELL_PCH then the UE shall support also HS-PDSCH in CELL_FACH.

Support of Enhanced F-DPCH

Defines whether the UE supports enhanced F-DPCH operation. If the UE supports Enhanced F-DPCH then the UE shall also support HS-PDSCH in CELL_DCH and E-DPDCH.

Maximum number of HS-DSCH codes received

Defines the maximum number of HS-DSCH codes the UE is capable of receiving. When the UE supports either MIMO or dual cell operation, this parameter defines the maximum number of HS-DSCH codes that the UE is capable of receiving per transport block.

Total number of soft channel bits in HS-DSCH

Defines the maximum number of soft channel bits over all HARQ processes. When explicit signalling is used, UTRAN configures Process Memory Size for each HARQ process so that the following criterion must be fulfilled in the configuration:

Total number of soft channel bits in HS-DSCH  sum of Process Memory Size of all the HARQ processes.

Minimum inter-TTI interval in HS-DSCH

Defines the distance from the beginning of a TTI to the beginning of the next TTI that can be assigned to the UE.

Support of Target Cell Pre-Configuration

Defines if the UE supports simultaneous HS-DSCH reception from serving cell and decoding of an HS-SCCH sent from another cell in the active set. If the UE supports Target Cell Pre-Configuration then the UE shall also support Enhanced F-DPCH.

Support of Enhanced Serving Cell Change for Event 1C

Defines if the UE supports simultaneous HS-DSCH reception from serving cell and decoding of an HS-SCCH sent from another cell in the active set when an Event 1C measurement report requesting serving HS-DSCH cell change is triggered. If the UE supports Enhanced Serving Cell Change for Event 1C then the UE shall also support Target Cell Pre-Configuration.

Support of HS-DSCH DRX operation

Defines whether the UE supports HS-DSCH DRX operation in CELL_FACH state as defined in [13]. If the UE supports HS-DSCH DRX operation in CELL_FACH state then the UE shall also support HS-PDSCH in CELL_FACH.

Support for Two DRX schemes in URA_PCH and CELL_PCH

Defines whether UE supports Two DRX schemes in URA_PCH and CELL_PCH.

Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active

Defines whether the UE supports TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2.

Support for cell-specific Tx diversity configuration for dual-cell operation

Defines whether the UE supports cell specific Tx diversity configuration when configured for dual-cell operation.

Support of MIMO only with single-stream restriction

Defines whether the UE supports MIMO only with restriction to single stream operation. UE supporting this capability shall belong to any HS-DSCH physical layer category not supporting MIMO or the UE shall belong to category 17 or 18. If the UE supports MIMO only with single-stream restriction, the UE shall also support MAC-ehs.

Support of MIMO mode with four transmit antennas per band capability

Defines whether the UE supports MIMO mode with four transmit antennas in CELL_DCH. The capability is defined per frequency band.

Support of MIMO mode with four transmit antennas operation only with dual-stream restriction

Defines whether the UE supports MIMO mode with four transmit antennas only with restriction to dual stream operation. UE supporting this capability shall belong to category 28, 30, 32, 34 or 36. If a UE supporting this capability belongs to category 34 or 36, then only up to 4 carriers can be configured with this capability. The dual-stream restriction capability shall not be signalled when UE supports either category 37 or 38. If the UE supports MIMO mode with four transmit antennas only with dual-stream restriction, the UE shall also support MAC-ehs.

Support of dual band operation

Defines whether the UE supports dual cell operation in the band combinations indicated in the Radio Access Capability Band Combination List [12]. If the UE supports dual band operation, the UE shall also support dual cell operation on adjacent frequencies.

Support for dual cell with MIMO operation in different bands

Defines whether the UE supports dual cell with MIMO operation in different bands. If the UE supports dual cell with MIMO operation in different bands, the UE shall also support dual band operation.

Support for Multiflow operation

A UE capability to receive simultaneously two HS-DSCH transport channels per carrier frequency, where the HS-DSCH transport channels may belong to the same or different Node Bs. If a UE supports Multiflow operation, it shall also support Dual Cell HSDPA operation on adjacent frequencies and the HS-DPCCH power offset extension.

Support of NodeB triggered HS-DPCCH transmission

Defines whether the UE supports NodeB triggered HS-DPCCH transmission in CELL_FACH state. If the UE supports both HS-PDSCH in CELL_PCH and URA_PCH states and NodeB triggered HS-DPCCH transmission, then the UE shall also support NodeB triggered HS-DPCCH transmission in CELL_PCH state. If the UE supports NodeB triggered HS-DPCCH transmission, then the UE shall also support Common E-DCH.

Support of HS-DSCH DRX operation with second DRX cycle

Defines whether the UE supports HS-DSCH DRX operation with second DRX cycle in CELL_FACH state as defined in [13]. If the UE supports HS-DSCH DRX operation with second DRX cycle in CELL_FACH state then the UE shall also support HS-PDSCH in CELL_FACH, HS-DSCH DRX operation, and common E-DCH. If the UE supports UTRAN ANR and HS-DSCH DRX operation with second DRX cycle, then the UE shall also support measurement and logging in CELL_FACH state when second DRX cycle is used for Automatic Neighbour Relation (ANR) in UTRAN. If the UE supports logged measurements in Idle mode and PCH States and HS-DSCH DRX operation with second DRX cycle, then the UE shall also support logged measurements in CELL_FACH state when second DRX cycle is used in UTRAN.

Non-contiguous multi-cell

Defines whether the UE supports non-contiguous multi-cell operation on two, three or four cells with single gap in one band. If the UE supports non-contiguous multi-cell operation in a certain band, it shall also support dual cell operation on adjacent frequencies in that band.

Support of HS-DPCCH power offset extension

Defines whether the UE supports the values 9 and 10 of deltaACK, deltaNACK and deltaCQI power offset as specified in [12].

Support of STTD on DL Control Channels when Multiflow operation is active

Defines whether a Multiflow capable UE supports STTD on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when Multiflow operation is active, as specified in [12].

Non-contiguous multi-cell with MIMO

Defines whether the UE supports non-contiguous multi-cell operation on two, three or four cells with single gap in one band with MIMO. If the UE supports non-contiguous multi-cell with MIMO, it shall also support non-contiguous multi-cell.

Support of multi-cell configuration in inter-RAT handover

Defines whether the UE supports multi-cell configuration at inter-RAT handover to UTRAN. The UE shall support it for all multi-cell capabilities supported by the UE (i.e. multi-cell operation on two or more than two cells, multi-cell operation in the band combinations indicated in the Radio Access Capability Band Combination List [12], non-contiguous multi-cell operation on two, three or four cells with single gap in one band, dual cell E-DCH), except for multi-cell operation on more than four cells.

Support of DPCCH2

Defines whether the UE supports DPCCH2 transmission in CELL_DCH state. If the UE supports DPCCH2, it shall also support HS-PDSCH in CELL_DCH.

Support for DCH Enhancements

Defines whether the UE supports DCH Enhancements.

Basic capability indicates that the UE supports the following sub-features in DCH enhancements ([19]):

-	Basic mode of DL FET (Mode 0)

-	Pilot-free DL DPCH slot formats #17 and #18

-	Pseudo flexible rate matching

-	Uplink DPCCH slot format #5

-	Uplink DPDCH dynamic 10ms transmission

Full capability indicates that the UE supports the following sub-features in DCH enhancements ([19]):

-	Basic mode of DL FET (Mode 0)

-	Full mode of DL FET (Mode 1)

-	Pilot-free DL DPCH slot formats #17 and #18

-	 Pseudo flexible rate matching and transport channel concatenation in L1

-	Uplink DPCCH slot format #5 with DL FET ACK/NACK indication

-	Uplink DPDCH dynamic 10ms transmission

Simultaneous support for DCH Enhancements and Compressed Mode operation

Defines whether the UE supports simultaneous operation of DCH Enhancements and Compressed Mode. If the UE supports simultaneous operation of DCH Enhancements with Full capability and Compressed Mode, then the UE shall support simultaneous operation of DCH Enhancements with Basic capability and Compressed Mode.

Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission

Defines whether the UE supports simultaneous operation of DCH Enhancements and DPCCH Discontinuous Transmission. If the UE supports simultaneous operation of DCH Enhancements with Full capability and DPCCH Discontinuous Transmission, then the UE shall support simultaneous operation of DCH Enhancements with Basic capability and DPCCH Discontinuous Transmission.

DRX enhancements

Defines whether the UE supports DRX enhancements, as defined in subclause 6C.3 in [25], or not.

HS-DPCCH overhead reduction

Defines whether the UE supports HS-DPCCH overhead reduction for multi-RAB with DCH or not.

Support of F-TPICH feedback from the Multiflow assisting cell

Defines whether the UE supports reception of the F-TPICH feedback from the Multiflow assisting serving HS-DSCH cell. If the UE supports this feature, it shall also support uplink closed loop transmit diversity and Multiflow operation.

Support of power control algorithm 3

Defines whether the UE supports power control algorithm 3.

Support of blind HARQ retransmissions for HSDPA

Defines whether the UE supports blind HARQ retransmissions for HSDPA.

Support of HS-SCCH DRX operation

Defines whether the UE supports HS-SCCH DRX operation in CELL_FACH state. If the UE supports HS-SCCH DRX operation in CELL_FACH state then the UE shall also support HS-DSCH DRX operation in CELL_FACH.

**************************************** TEXT OMITTED ***************************************

## 5.1	Value ranges

Table 5.1: UE radio access capability parameter value ranges

|  |  | UE radio access capability parameter | Value range |
| --- | --- | --- | --- |
| PDCP parameters |  | Support for RFC 2507 | Yes/No |
|  |  | Support for RFC 3095 | Yes/No |
|  |  | Support for RFC 3095 context relocation | Yes/No |
|  |  | Support for loss-less SRNS relocation | Yes/No |
|  |  | Support for loss-less DL RLC PDU size change | Yes/No |
|  |  | Maximum header compression context space | 1024, 2048, 4096, 8192, 16384, 32768, 65536, 131072 bytes |
|  |  | Maximum number of ROHC context sessions | 2, 4, 8, 12, 16, 24, 32, 48, 64, 128, 256, 512, 1024, 16384 |
|  |  | Support for Reverse Decompression | Not supported, 1..65535 |
|  |  | Support for CS voice over HSPA | Yes/No |
| RLC, MAC-hs, MAC-ehs and MAC-i/is parameters |  | Total RLC AM, MAC-hs and MAC-ehs buffer size | 2, 10, 50, 100, 150, 200, 300, 400, 500, 750, 1000, 1150, 1250, 1800, 2000, 2300, 2550, 3400, 3500, 4400, 4500, 5000 kBytes |
|  |  | Maximum number of AM entities | 3, 4, 5, 6, 8, 16, 30 |
|  |  | Maximum RLC AM window size | 2047, 4095 |
|  |  | Support for MAC-ehs | Yes/No |
|  |  | Support for two logical channels | Yes/No |
|  |  | Support of MAC-i/is | Yes/No |
|  |  | Support of MAC-ehs window size extension | Yes/No |
|  |  | Support of UM RLC re-establishment via reconfiguration | Yes/No |
| PHY parameters | Transport channel parameters in downlink | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum number of simultaneous transport channels | 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo decoding | Yes/No |
|  | Transport channel parameters in uplink | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 2, 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH of DCH type (TDD only) | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks transmitted within TTIs that start at the same time | 2, 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo encoding | Yes/No |
|  | FDD Physical channel parameters in downlink | Maximum number of DPCHcodes to be simultaneously received | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH) | 600, 1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800 |
|  |  | Support for SF 512 and 80 ms TTI for DPCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_DCH | Yes/No |
|  |  | Support of HS-SCCHless HS-DSCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_PCH and URA_PCH | Yes/No |
|  |  | Support of Enhanced F-DPCH | Yes/No |
|  |  | Support of Target Cell Pre-Configuration | Yes/No |
|  |  | Support of Enhanced Serving Cell Change for Event 1C | Yes/No |
|  |  | Support of HS-DSCH DRX operation | Yes/No |
|  |  | Support of Node B triggered HS-DPCCH transmission | Yes/No |
|  |  | Support of HS-DSCH DRX operation with second DRX cycle | Yes/No |
|  |  | Support for Two DRX schemes in URA_PCH and CELL_PCH | Yes/No |
|  |  | Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | Yes/No |
|  |  | Support for cell-specific Tx diversity configuration for dual-cell operation | Yes/No |
|  |  | Support of MIMO only with single-stream restriction | Yes/No |
|  |  | Support for dual cell with MIMO operation in different bands | Yes/No |
|  |  | Support of MIMO mode with four transmit antennas operation only with dual-stream restriction | Yes/No |
|  |  | Support of dual band operation (Radio Access Capability Band Combination List) | 1..16 |
|  |  | >Band Combination | 1..256 |
|  |  | >Supported Carrier Combination List |  |
|  |  | >>Carrier Combination (1,2) | Yes/No |
|  |  | >>Carrier Combination (2,1) | Yes/No |
|  |  | >>Carrier Combination (1,3) | Yes/No |
|  |  | >>Carrier Combination (3,1) | Yes/No |
|  |  | >>Carrier Combination (2,2) | Yes/No |
|  |  | >>Carrier Combination (1,4) | Yes/No |
|  |  | >>Carrier Combination (4,1) | Yes/No |
|  |  | >>Carrier Combination (1,5) | Yes/No |
|  |  | >>Carrier Combination (5,1) | Yes/No |
|  |  | >>Carrier Combination (1,6) | Yes/No |
|  |  | >>Carrier Combination (6,1) | Yes/No |
|  |  | >>Carrier Combination (1,7) | Yes/No |
|  |  | >>Carrier Combination (7,1) | Yes/No |
|  |  | >>Carrier Combination (2,3) | Yes/No |
|  |  | >>Carrier Combination (3,2) | Yes/No |
|  |  | >>Carrier Combination (2,4) | Yes/No |
|  |  | >>Carrier Combination (4,2) | Yes/No |
|  |  | >>Carrier Combination (2,5) | Yes/No |
|  |  | >>Carrier Combination (5,2) | Yes/No |
|  |  | >>Carrier Combination (2,6) | Yes/No |
|  |  | >>Carrier Combination (6,2) | Yes/No |
|  |  | >>Carrier Combination (3,3) | Yes/No |
|  |  | >>Carrier Combination (3,4) | Yes/No |
|  |  | >>Carrier Combination (4,3) | Yes/No |
|  |  | >>Carrier Combination (4,4) | Yes/No |
|  |  | >>Carrier Combination (3,5) | Yes/No |
|  |  | >>Carrier Combination (5,3) | Yes/No |
|  |  | Support for Multiflow | Yes/No (per frequency band) |
|  |  | >Support for Multiflow and MIMO | No/Single stream/Dual stream NOTE: If a UE supports single-stream MIMO transmission with Multiflow, then single-stream MIMO is supported in all the frequency bands where Multiflow operation is supported. The dual-stream MIMO transmission with Multiflow support is per frequency band. |
|  |  | >Support for Multiflow with non-contiguous carrier allocation | 5, 10, any gap size (per frequency band) |
|  |  | Support for Multiflow in different bands | Yes/No (per frequency band combination) |
|  |  | Support for Multiflow with MIMO operation in different bands | No/Single stream/Dual stream NOTE1: If a UE does not support "Multiflow with MIMO operation in different bands", then no MIMO is supported in any of the band combinations where Multiflow is supported. NOTE2: If a UE supports single-stream "Multiflow with MIMO operation in different bands", then single-stream MIMO is supported in all the band combinations where Multiflow is supported. NOTE3: If a UE supports dual-stream "Multiflow with MIMO operation in different bands", then dual-stream MIMO transmission is supported in all the band combinations, in which the UE supports Multiflow and dual-stream MIMO in both bands of the corresponding band combination. Otherwise the UE supports single-stream MIMO in that band combination. |
|  |  | Support of MIMO mode with four transmit antennas operation | Yes/No (per frequency band) |
|  |  | Non-contiguous multi-cell | 1..3 |
|  |  | >Aggregated cells | 2, 3, 4 |
|  |  | >Gap size | 5, 10, any gap size |
|  |  | >Non-contiguous multi-cell Combination (2,2) | Yes/No |
|  |  | >Non-contiguous multi-cell Combination (3,1) (1,3) | Yes/No |
|  |  | Support of HS-DPCCH power offset extension | Yes/No |
|  |  | Support of STTD on DL Control Channels when Multiflow operation is active | Yes/No |
|  |  | Non-contiguous multi-cell with MIMO | Yes/No |
|  |  | Support of multi-cell configuration in inter-RAT handover | Yes/No |
|  |  | Support of DPCCH2 | Yes/No |
|  |  | Support for DCH Enhancements | Basic/Full |
|  |  | Simultaneous support for DCH Enhancements and Compressed Mode operation | Yes/No |
|  |  | Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission | Yes/No |
|  |  | DRX enhancements | Yes/No |
|  |  | HS-DPCCH overhead reduction | Yes/No |
|  |  | Support of F-TPICH feedback from the Multiflow assisting cell | Yes/No |
|  |  | Support of power control algorithm 3 | Yes/No |
|  |  | Support of blind HARQ retransmissions for HSDPA | Yes/No |
|  |  | Support of HS-SCCH DRX operation | Yes/No |
|  | FDD Physical channel parameters in uplink | Maximum number of DPDCH bits transmitted per 10 ms | 600, 1200, 2400, 4800, 9600, 19200, 28800, 38400, 48000, 57600 |
|  |  | Support of E-DPDCH | Yes/No |
|  |  | Support of Discontinuous Transmission in CELL_DCH | Yes/No |
|  |  | Support of Slot Format #4 | Yes/No |
|  |  | Support for E-DPCCH power interpolation formula | Yes/No |
|  |  | Support for E-DPCCH power boosting | Yes/No |
|  |  | Support of common E-DCH | Yes/No |
|  |  | Support of Common E-RGCH based interference control | Yes/No |
|  |  | Support of Fallback to R99 PRACH | Yes/No |
|  |  | Support of Concurrent deployment | Yes/No |
|  |  | Support of TTI alignment and Per HARQ process | Yes/No |
|  |  | Support of uplink open loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of uplink closed loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of Uplink MIMO | Yes/No (per frequency band) |
|  |  | Support of Serving E-DCH cell decoupling | Yes/No |
|  |  | Support of Radio Links without DPCH/F-DPCH | Yes/No |
|  |  | Support of Cell Reselection Indication Reporting | Yes/No |
|  |  | Access Groups based access control | Yes/No |
|  |  | Enhanced TTI switching | Yes/No |
|  |  | Implicit Grant handling | Yes/No |
|  |  | DTX enhancements | Yes/No |
|  |  | Support of Dual Cell E-DCH transmission with DPDCH | Yes/No |
|  |  | Support for Dual Band Dual Cell E-DCH operation | Yes/No |
|  | TDD 3.84 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..224 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  | TDD 3.84 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 7.68 Mcps physical channel parameters in downlink TDD 7.68 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..448 |
|  |  | Minimum SF | 32, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..32 |
|  | TDD 7.68 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 32, 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in downlink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per subframe | 1, 2, 3, …, 96 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  |  | Support 8PSK | Yes/No |
|  |  | UE specific capability Information LCR TDD | Enumerated (NF, TriRxUniTx, TriRxTriTx, HexRxUniTx, HexRxTriTx, HexRxHexTx, TwoRxUniTxDiscontiguous, TwoRxTwoTxDiscontiguous, TwoRxUniTxContiguous, TwoRxTwoTxContiguous) NOTE: If three frequencies are supported, the three frequencies shall be configured within 5 MHz; if six frequencies are supported, the six frequencies shall be configured within 10MHz; TwoRxUniTxDiscontiguous and TwoRxTwoTxDiscontiguous mean that the UE is capable of supporting two non-adjacent carriers; TwoRxUniTxContiguous and TwoRxTwoTxContiguous mean that the UE is only capable of supporting two adjacent carriers; if two non-adjacent carriers are supported, the two carriers can be in the same band or in two different bands. |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of SPS | Yes/No |
|  |  | Support of HS-SCCH/E-AGCH Discontinuous Reception | Yes/No |
|  |  | Support of SF Mode For HS-PDSCH dual stream | Yes/No |
|  |  | Support of Enhanced TS0 | Yes/No |
|  |  | Support of Non-rectangular Resource Allocation | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in uplink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of 8PSK | Yes/No |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  |  | Support of E-DCH in CELL_FACH | Yes/No |
| RF parameters | FDD RF parameters | UE power class | 3, 4 NOTE: Only power classes 3 and 4 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [4]. UEs that support band XIX shall also support band VI |
|  |  | Tx/Rx frequency separation | Defined in [4] for the respective supported radio frequency band |
|  |  | Support of Multiple Frequency Band Indicators | Yes/No |
| RF parameters | TDD 3.84 Mcps RF parameters | UE power class | 2, 3 NOTE: Only power classes 2 and 3 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
|  | TDD 1.28 Mcps RF parameters | UE power class | 2, 3 |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
| Multi-mode related parameters |  | Support of UTRA FDD | Yes/No |
|  |  | Support of UTRA TDD 3.84 Mcps | Yes/No |
|  |  | Support of UTRA TDD 1.28 Mcps | Yes/No |
| Multi-RAT related parameters |  | Support of GSM | Yes/No (per GSM frequency band) |
|  |  | Support of multi-carrier | Yes/No |
|  |  | Support of UTRAN to GERAN Network Assisted Cell Change | Yes/No |
|  |  | Support of Handover to GAN | Yes/No |
|  |  | Support of Inter-RAT PS Handover | Yes/No |
|  |  | Support of PS Handover to GAN | Yes/No |
|  |  | Support of E-UTRA FDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA FDD | Yes/No |
|  |  | Support of E-UTRA FDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Support of E-UTRA TDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA TDD | Yes/No |
|  |  | Support of E-UTRA Multiple Frequency Band Indicators | Yes/No |
|  |  | Support of E-UTRA TDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Support of RAN-assisted WLAN interworking based on RAN rules | Yes/No |
|  |  | Support of RAN-assisted WLAN interworking based on ANDSF policies | Yes/No |
|  |  | Support of extended E-UTRA frequency priority | Yes/No |
| Security parameters |  | Support of ciphering algorithm UEA0 | Yes |
|  |  | Support of ciphering algorithm UEA1 | Yes |
|  |  | Support of ciphering algorithm UEA2 | Yes |
|  |  | Support of integrity protection algorithm UIA1 | Yes |
|  |  | Support of integrity protection algorithm UIA2 | Yes |
| UE positioning related parameters |  | Standalone location method(s) supported | Yes/No |
|  |  | UE based OTDOA supported | Yes/No |
|  |  | Network assisted GPS support | Network based / UE based / Both/ None |
|  |  | Network Assisted GANSS support List: | per GANSS |
|  |  | >GANSS ID | Galileo / SBAS / Modernized GPS / QZSS / GLONASS/ BDS |
|  |  | >SBAS IDs | WAAS / EGNOS / MSAS / GAGAN |
|  |  | >GANSS mode | Network based / UE based / Both/ None |
|  |  | >GANSS Signal ID | 0..7 |
|  |  | >GANSS Signal IDs | Yes/No (per GANSS signal) |
|  |  | >Support for GANSS timing of cell frames measurement | Yes/No |
|  |  | >Support for GANSS Carrier-Phase Measurement | Yes/No |
|  |  | >Support for non-native assistance choices | Yes/No |
|  |  | Support for GPS timing of cell frames measurement | Yes/No |
|  |  | Support for IPDL | Yes/No |
|  |  | Support for Rx-Tx time difference type 2 measurement | Yes/No |
|  |  | Support for UE Positioning assisted GPS measurement validity in CELL_PCH and URA_PCH RRC states | Yes |
|  |  | Support for SFN-SFN observed time difference type 2 measurement | Yes/No |
|  |  | Additional Positioning method support List | Per Additional Positioning method |
|  |  | >AddPos ID | Barometric Pressure, WLAN, Bluetooth, MBS |
|  |  | >AddPos mode | Standalone / UE assisted / Both |
| Measurement related capabilities |  | Need for downlink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Need for uplink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Support for absolute priority based cell re-selection in UTRAN | Yes/No |
|  |  | Support for System Information Block type 11bis | Yes |
|  |  | Adjacent Frequency measurements without compressed mode | Yes/No |
|  |  | Inter-band Frequency measurements without compressed mode | Yes/No |
|  |  | Enhanced inter-frequency measurements without compressed mode | Yes/No |
|  |  | Extended measurements ID support | Yes |
|  |  | Frequency specific compressed mode | Yes/No |
|  |  | Frequency specific compressed mode for non-contiguous operation | Yes/No |
|  |  | Inter-frequency detected set measurements | Yes/No |
|  |  | Inter-frequency measurements on configured carriers without compressed mode | Yes/No |
|  |  | Cells excluded from detected set measurements | Yes/No |
|  |  | Wideband RSRQ FDD measurements | Yes/No |
|  |  | Wideband RSRQ TDD measurements | Yes/No |
|  |  | Event 2g reporting on a configured secondary downlink frequency | Yes/No |
|  |  | Enhanced UPH reporting | Yes/No |
|  |  | RRC measurement events 6h and 6i for UPH reporting | Yes/No |
|  |  | Increased UE carrier monitoring UTRA | Yes/No |
|  |  | Increased UE carrier monitoring E-UTRA | Yes/No |
|  |  | Extended RSRQ lower value range | Yes/No |
|  |  | RSRQ measurement on all symbols | Yes/No |
| General capabilities |  | Access Stratum release indicator | R99, REL-4, REL-5, REL-6, REL-7, REL-8, REL-9, REL-10, REL-11, REL-12, REL-13 |
|  |  | Device type | Benefits from NW-based battery consumption optimisation / Does not benefit from NW-based battery consumption optimisation |
|  |  | Support of DSAC and PPAC update in CELL_DCH | Yes/No |
|  |  | Support of retrievable configurations | Yes/No |
|  |  | Support of URA_PCH with seamless transition | Yes/No |
|  |  | Support of improved synchronized RRC procedures | Yes/No |
|  |  | Support of enhanced state transition | Yes/No |
|  |  | Support of simultaneous setup and release of RABs and RBs | Yes/No |
| DL capabilities with simultaneous HS-DSCH |  | DL capability with simultaneous HS-DSCH configuration | 32 kbps, 64 kbps, 128 kbps, 384 kbps |
| UL capabilities with simultaneous E-DCH |  | UL capabilities with simultaneous E-DCH | 64 kbps |
| UE based network performance measurements parameters |  | Support of logged measurements in Idle mode and PCH States | Yes/No |
|  |  | Support of UTRAN ANR | Yes/No |
| Home Node B Inbound Mobility Related Parameters | CSG Proximity Indication capabilities | Support of intra-frequency proximity indication | Yes/No |
|  |  | Support of inter-frequency proximity indication | Yes/No |
|  |  | Support of E-UTRA proximity indication | Yes/No |
|  | Neighbour Cell SI Acquisition capabilities | Support of intra-frequency SI acquisition for HO | Yes/No |
|  |  | Support of inter-frequency SI acquisition for HO | Yes/No |
|  |  | Support of E-UTRA SI acquisition for HO | Yes/No |
| IMS Voice Parameters |  | Voice over UTRA PS HS Support | Yes/No |
|  |  | SRVCC Support from UTRA to UTRA | Yes/No |
|  |  | SRVCC Support from UTRA to GERAN | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN FDD | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN TDD | Yes/No |