3GPP TSG-RAN WG2 #97	Tdoc R2-1700834

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.1.2

Source:	Ericsson

Title:	Further aspects of data duplication in PDCP layer

Document for:	Discussion, Decision

# Introduction

Transmission reliability and latency enhancement are two key aspects for URLLC, defined as Ultra-Reliable and Low-Latency Communications in TR 22.862 and TR 38.913.

In RAN2#96, it wss agreed as follows

- Multi-connectivity (e.g. with packet duplication, link selection) should be studied for achieving the reliability requirements for URLLC.

In RAN2 NR adhoc meeting, it was agreed as follows

1: Packet duplication is supported for user plane and control plane in NR-PDCP (This agreement does not preclude discussion of other mechanisms to improve mobility robustness) FFS whether packet duplication should also be supported for LTE-NR dual connectivity 2 The PDCP function in the transmitter supports packet duplication and the PDCP function in the receiver supports duplicate packet removal.

This paper is to address some further aspects of data duplication in PDCP layer.

# Discussion

## Intra-frequency vs. inter-frequency

In 3GPP Rel-12 study item on LTE small cell enhancement (SCE) – higher layer aspects [1], there are multiple scenarios addressed, including both intra-frequency and inter-frequency cases. Finally only inter-frequency scenario was addressed in Rel-12 WI on dual connectivity (DC), in order to implement inter-node radio resource aggregation for improving per-user throughput.

Rel-12 DC architecture is limited to throughput maximization and thus is limited to inter-frequency scenario.

Now in order to address the reliability requirement for URLLC / eV2x in NR, PDCP duplication for both user plane and control plane is agreed. Therefore, considering that it was proved that diversity gain can be achieved in the intra-frequency scenario, it is important to extend the DC architecture to intra-frequency scenario as well.

Extend DC achitecture to intra-frequency scenario to explore diversity gain.

From RAN2 point of view there should not be any major issues with respect to e.g. RRC configuration, however this new functionality puts new requirements on e.g. UE measurements and possibly MeNB/SeNB coordination. This implies studies are needed in more working groups than only RAN2.

## Triggers for PDCP packet duplication (de)activation

As analysed in [2], although packet duplication is efficient to provide diversity gain, the obvious drawback is the low resource efficiency, and thus it would be optimal to combine packet duplication and link selection. In order to implement it, the key component is the trigger design, which decides when / how the duplication or selection should be (de)activated and switch between each other.

In general, there could be various types of trigger that can be used, which can be divided into two categories, i.e., channel status and A/N feedback.

  - Channel status: it can be the measured at layer-1 like CSI report or layer-3 like RRM / RLM measurement results. The advantage of using channel status report as trigger is that it can be decoupled from data transmission, and thus can proactively trigger (de)activation of duplication or selection;

  - A/N feedback: it can be obtained at HARQ or ARQ. One drawback compared to a), is an additional latency of the previously failed data transmission(s). But this trigger type is useful as a complementary part of a), considering channel status only is not enough to act as the trigger to balance reliability and resource efficiency.

For DL, it is more for network implementation to decide on which trigger type(s) to use. Besides, in order to avoid the aditional latency due to trigger forwarding via backhaul link, it is beneficial to ensure the used trigger is sent to MeNB instead of SeNB.

For UL, it can be

- Either also the network implementation to decide on which trigger type(s) to use, and then use explicit DL signalling to indicate UE of the PDCP duplication (de)activation decision;

- Or rely on UE to decide on PDCP duplication (de)activation, for which a network configured rule can be used to ensure satisfying performance. And to define the rule, there is a need to regulate the applied trigger type(s).

RAN2 to investigate triggering schemes to activate/deactivate packet duplication and/or link selection.

## Aspects of PDCP duplication

As stated in section 2.2, one drawback of PDCP duplicate transmission is low resource efficiency, in more details:

- The identical payload is transmitted on two parallel links;

- Additionally, retransmissions on one link (HARQ) while the packet has been successfully transmitted on the other link further reduces the resource efficiency.

The latter aspect above is especially harmful since that it not only causes the lower spectrum efficiency, but also may cause delay of data due to queuing. To alleviate this, the transmitter should be able to abort HARQ retransmissions of data successfully received on the other link.

RAN2 to consider solutions to improve PDCP duplication mechanisms in the face of per link re-transmissions.

# Conclusion

In section 2 , we made the following observations:

Observation 1	Rel-12 DC architecture is limited to throughput maximization and thus is limited to inter-frequency scenario.

Based on the discussion in section 2 we propose the following:

Proposal 1	Extend DC achitecture to intra-frequency scenario to explore diversity gain.

Proposal 2	RAN2 to investigate triggering schemes to activate/deactivate packet duplication and/or link selection.

Proposal 3	RAN2 to consider solutions to improve PDCP duplication mechanisms in the face of per link re-transmissions.

# References

TR 36.842, Study on Small Cell enhancements for E-UTRA and E-UTRAN -  Higher layer aspects, V12.0.0, 2013-12.

R2-1700835, URLLC aspects for grant-free transmission in NR, Ericsson, RAN2#97.