3GPP TSG-RAN WG2 #97	Tdoc R2-1700893

Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.2.6

Source:	Ericsson

Title:	Further considerations on RAN and CN paging in INACTIVE

Document for:	Discussion, Decision

# Introduction

Following agreements related to RAN and CN-initiated paging/notification of an UE in INACTIVE state is captured in the meeting report from the RAN WG2 NR adhoc meeting in January.

Agreements:

1	A UE in INACTIVE is reachable via RAN-initiated notification and CN-initiated Paging. RAN and CN paging occasions overlap and same paging/notification mechanism used.

2	A RAN node can configure a UE in INACTIVE with a RAN configured paging DRX cycle (which could be UE specific configuration).

In addition, following agreements related to paging and DRX for an UE in IDLE and INACTIVE was also captured in the meeting report.

Agreements

1: UE in RRC_IDLE and RRC_INACTVE state monitors paging/notification every DRX cycle.

2: UE monitors one paging occasion per DRX cycle. Paging occasion is the time interval over which a paging message is transmitted by gNB.

3: The length of DRX cycle is configurable. A default DRX cycle length is provided in system information. Additionally, a UE specific DRX cycle length can also be provided to UE in dedicated signalling.

4: The number of paging occasions in the DRX cycle is configurable and provided in system information.

5: If multiple paging occasions are configured by network in the DRX cycle then UEs can be distributed to these paging occasions based on UE ID.

This contribution discusses some further aspects on RAN and CN-initiated paging while the UE is in INACTIVE state.

# Discussion

## RAN Paging and DRX cycle

As agreed at the RAN2 NR adhoc meeting, a UE-specific DRX cycle can be provided to the UE in case the default DRX cycle broadcasted in the serving cell does not meet the latency and/or power saving requirements of the UE. Based on this agreement it is assumed that the UE negotiates the idle mode DRX cycle in one or more NAS procedures during the attach and TAU (or equivalent procedures), i.e. similar to how the DRX negotiation process works in legacy LTE.

RAN2 also agreed to base the paging/notification process in RAN and in the CN on the same mechanisms (“RAN and CN paging occasions overlap and same paging/notification mechanism used”). Furthermore, for robustness reasons the UE in RRC_INACTIVE need to monitor and act on a RAN initiated page as well as on a CN initiated page (“A UE in INACTIVE is reachable via RAN-initiated notification and CN-initiated Paging”). Based on these agreements it is obvious that the RAN must have access to the same UE related information as is used in the Paging Occasion calculation process in the CN.

For this reason, the negotiated idle mode DRX cycle (in case the UE has not chosen the default DRX cycle) and the UE identity for Paging Occasion calculation (IMSI mod X or equivalent) should be sent to the RAN at the establishment of the CN-RAN connection (e.g. as part of the UE context setup information) to enable the RAN to calculate the correct Paging Occasion within the DRX cycle.

To allow RAN initiated paging/notification, the negotiated idle mode DRX cycle and the UE identity for Paging Occasion calculation need to be known in RAN at e.g. the initial setup of the UE context.

- As the length of the DRX cycle is a trade-off between paging latency and power saving, UEs with low latency requirements and limited power supply may request for very long DRX cycles comparable to the eDRX value range in LTE. Therefore, it should be discussed whether this type of UE requirements shall be respected by the network or not, i.e. if a UE that has negotiated a longer idle mode DRX cycle, e.g. in the range above 10 seconds, shall be a candidate for the INACTIVE state.

- While different services have different requirements on UE reachability, a default DRX cycle or a negotiated DRX cycle is likely not appropriate for all services used by the UE over time. For example, for delay sensitive services the response time from the UE in INACTIVE may need to be shortened compared to the paging response time valid for idle state. There could also be other reasons for the RAN to change the DRX cycle, e.g. if the buffered downlink signalling/data in the gNB reaches a critical limit.

- For these reasons, it should be possible for the gNB to configure the UE with a shorter DRX cycle compared to what is used by the UE in idle mode, so that the expected UE response time (thus the paging monitoring cycle) is changed to a level which is acceptable for the network, the UE and the service(s) used.

- The RAN configured DRX cycle is sent to the UE in dedicated signalling, preferably in the same message that will request the UE into RRC_INACTIVE state.

- Since the UE in RRC_INACTIVE also need to be reachable from a CN initiated page, the RAN configured DRX cycle must be time aligned with the idle mode DRX cycle used by the UE while in RRC_IDLE. In other words, the RAN configured DRX cycle can be shorter than the idle mode DRX cycle as long as the CN paging monitoring cycle always coincide with one of the RAN paging occasions.

The UE can be configured with a shorter DRX cycle in RRC_INACTIVE state compared to what is used in IDLE state as long as the CN paging monitoring cycle always coincide with one of the RAN paging occasions.

- It seems reasonable to assume that the RAN configured DRX cycle is only valid as long as the UE remains in RRC_INACTIVE state or until it is further modified by the RAN (this could be the same or another gNB).

- When the UE enters IDLE state it will revert to the idle mode DRX cycle previously used in that cell (i.e. if the UE remains in the same cell in which it entered CONNECTED/INACTIVE state) or apply to the DRX cycle valid for the new cell (alternatively negotiate an UE-specific DRX cycle).

The RAN configured DRX cycle is only valid while the UE remains in RRC_INACTIVE state.

## UE identity in Paging messages

In legacy LTE, a NAS identifier (typically S-TMSI) is used to address the UE in the paging procedure. With the Rel-14 Light connection WI and the introduction of RAN initiated paging, it has been agreed to use a RAN allocated UE identity (resumeIdentity) in the RRC Paging message when the paging is initiated in the RAN. One of the reasons behind this agreement is the potential security issue if the NAS identity (S-TMSI) is exposed on the radio interface without the CN being in control of this.

In NR a similar security issue by exposing the NAS identity on the radio interface is likely to appear which is also emphasized by SA3 in their LS reply (R2-167438) on S-TMSI in RAN paging to RAN2#96:

“SA3 would like to remind that frequent updates of the S-TMSI are necessary for privacy reasons. Not frequently updated temporary identifiers is an existing privacy threat that allows an attacker to track the subscriber using "non-changing" GUTI/S-TMSI, see [1]. SA3 also want to inform RAN2 that, in NextGen, SA3 is discussing solutions to refresh the S-TMSI frequently in order to enhance subscriber's privacy.”

Furthermore, as indicated by SA3 in their LS, using a NAS identifier at RAN initiated paging will probably also lead to additional signalling between the RAN and the CN since mechanisms for frequent updates of the NAS identity will be needed. It is also likely that the update mechanism will be more complex since recovery procedures will be needed if re-allocation of the NAS identity fails.

For the reasons above, the UE should be addressed with a RAN allocated UE identity at RAN initiated paging.

A UE in RRC_INACTIVE is normally only paged from the RAN, however for robustness purposes a UE in RRC_INACTIVE also need to be reached by a CN initiated page (as according to one of the agreements from the RAN2 NR Adhoc meeting). To resolve from a state inconsistency situation in which the UE is in RRC_INACTIVE while the network considers the UE to be in IDLE (e.g. if the UE was temporarily out of coverage at the time the release message was sent), the UE in RRC_INACTIVE need to respond on a CN initiated page containing its NAS identifier.

The UE is addressed with a RAN allocated UE identity at RAN initiated paging and with a NAS identity at CN initiated paging.

## UE response to paging

As agreed at the RAN2 NR adhoc meeting, a UE need to monitor and respond to a RAN initiated paging as well as to a CN initiated paging while in RRC_INACTIVE. For the reasons as discussed in section 2.2, a RAN initiated paging message includes a RAN allocated UE identity whereas a CN initiated paging message is sent with a CN allocated (NAS) identity.

At reception of the RRC Paging message while in RRC_INACTIVE the UE could however behave the same regardless if the paging procedure is triggered in the RAN or in the CN, i.e. independently of the UE identifier included in the message. That is, the UE takes advantage of the stored AS context and attempts to resume the RRC connection by sending an RRC Connection Resume Request message (or equivalent) to the gNB, identifying itself with the RAN allocated UE identity sent to the UE once it was transited to RRC_INACTIVE.

When the AS context is successfully retrieved in the network, the UE will be transited to RRC_CONNECTED as part of the Resume procedure, see Figure 1 below.

Figure 1: Paging with successful resume procedure

If the AS context for some reason cannot be retrieved in the network, thus the resumption of the RRC connection fails, a fall-back procedure was proposed in [1] in which the gNB triggers an RRC Connection Establishment procedure as a result of the failed resumption.
The signalling flow for this scenario is shown in Figure 2 below.

Figure 2: Paging with fall-back to RRC connection establishment procedure

From the figure above it is clear that even if the resumption attempt fails it will not add any additional roundtrip costs between the UE and the network compared to a normal RRC Connection Establishment procedure triggered from RRC_IDLE (the UE will in that case send a RRC Connection Request message instead of the RRC Connection Resume Request message).

The fact that the UE will keep the AS context until informed by the network (in Figure 2 at reception of the RRC Connection Setup message) could also be considered as a more secure solution compared to a solution where the AS context is simply released at reception of a paging message.

It shall be observed that the Paging procedure in Figure 1 and Figure 2 can also be initiated from the CN if e.g.  the UE was temporarily out of coverage at the time the release message was sent.

At reception of the Paging message while in RRC_INACTIVE, the UE shall always attempt to resume its RRC connection.

# Conclusion

This paper has discussed some further aspects on RAN and CN-initiated paging while the UE is in INACTIVE state.

Based on the discussion in section 2 we propose the following:

Proposal 1	To allow RAN initiated paging/notification, the negotiated idle mode DRX cycle and the UE identity for Paging Occasion calculation need to be known in RAN at e.g. the initial setup of the UE context.

Proposal 2	The UE can be configured with a shorter DRX cycle in RRC_INACTIVE state compared to what is used in IDLE state as long as the CN paging monitoring cycle always coincide with one of the RAN paging occasions.

Proposal 3	The RAN configured DRX cycle is only valid while the UE remains in RRC_INACTIVE state.

Proposal 4	The UE is addressed with a RAN allocated UE identity at RAN initiated paging and with a NAS identity at CN initiated paging.

Proposal 5	At reception of the Paging message while in RRC_INACTIVE, the UE shall always attempt to resume its RRC connection.

# References

R2-1700535, RRC state machine and RRC_INACTIVE characteristics, Ericsson, RAN2 NR AH, Spokane USA, 17th – 19th Jan 2017