3GPP TSG-RAN WG2 Meeting #97						R2-1701088

Athens, Greece, 13 - 17 February 2017

Title:	[DRAFT] Reply LS on bearer modelling and QoS for UE-to-NW relaying

Response to:	LS (S2-170398) “Reply LS on UE-to-NW relaying” from SA2

Release:	Rel-15

Work Item:	SA2: FS_REAR

RAN2: FS_feD2D_IoT_relay_wearable

Source:	Nokia [to be RAN2]

To:	SA2

Cc:	-

Contact Person:

#### Name:	Dawid Koziol

Tel. Number:

E-mail Address:	dawid.koziol@nokia.com

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA2 for their LS concerning UE-to-Network relaying and for providing clarifications requested by RAN2.

In the reply LS SA2 asks also two follow-up questions for further clarification from RAN2:

- Can RAN2 confirm that the existing limit of number of DRBs on Uu (8 DRBs, as according to TS 36.331) is to be maintained?

- If the above assumption A) is confirmed, would QoS differentiation be supported when different eRemote-UEs are served by an eRelay-UE by sharing DRBs over Uu?

RAN2 would like to give the following feedback:

- With the currently studied layer-2 relaying design there will be two types of Data Radio Bearers:

- “Relayed DRBs” – these will be bearers established logically between remote UE and eNB containing only PDCP layer configuration and mapped to remote UEs EPS bearer in 1:1 manner

- “Relaying DRBs” – these will be bearers established between relay UE and eNB containing configuration of RLC and MAC layers

It is important to note that it is possible to multiplex multiple relayed DRBs onto a single relaying DRB and the limitation of 8 DRBs pertains to relaying DRBs. Therefore, it is possible for the relay UE to relay more than 8 EPS bearers belonging to one or more remote UEs.

- QoS differentiation on Uu interface will be supported by mapping EPS bearers with different QoS characteristics to different Data Radio Bearers on Uu interface (different relaying DRBs).

2. Actions:

To SA2

ACTION: 	RAN2 respectfully asks SA2 to take the above information into account in their further work within related Work Item.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting #97bis	3rd – 7th April 2017	Spokane, US

TSG-RAN WG2 Meeting #98	15th – 19th May 2017	Hangzhou, China