3GPP TSG-RAN WG2 #97bis	 R2-1702529

Spokane, USA, April 3rd- 7th, 2017

Agenda Item:	10.2.4.2

Source: 	Xiaomi

Title:  	Visibility of Slicing to UE

Document for:	Discussion and Decision

Introduction

During RAN2 #97 meeting, RAN2 discussed whether network slicing is visible to UE. But the discussion was not fruitful, no conclusion was made. We think it is difficult to discuss directly whether UE need to be aware of the network slicing or not. A better way forward is to go through the details first. After details are clear to us, whether UE needs to see the network slicing is also clear.

In this contribution, we focus on three detailed aspects: PRACH partitioning, access and overload control, traffic transmission, mobility.

Discussion

PRACH partitioning

In LTE, PRACH resources (time, frequency, preamble) are common configured for all UEs in idle mode, and dedicated configured for connected UEs for the case of SCell configuration and handover. In NR, network slicing is introduced, then we need to decide whether different PRACH resources need to be configured to different network slices. We think the intention of introducing network slicing is to confine the impact of services of one slice within its own scope. As we know, random access is one major factor that leads to serious impact between different services, especially considering IOT services. If RAN to support network slicing, it has to support partitioning of random access resources for different slices. To do this, RAN needs to broadcast specific PRACH-config/RACH-config for some network slices. Then the question is whether RAN broadcast the PRACH config with network slice ID directly or some other tags can be used to represent different slices/slice types. For example, RAN2 is considering using a unified access category for access barring, UE performs access barring check taking only access category into count (agnostic to applications, services, call types, devices, signalling, may also agnostic to network slicing) [1]. Such an "access category" may be used for PRACH partitioning if it contains network slicing related information, the feasibility of which, however, still needs CT1 to evaluate. From our point of view, both network slicing ID and access category is technically feasible. The former requires UE to understand the concept of network slicing, but the latter does not.

Observation 1: For PRACH partitioning, whether network slicing is visible to UE depends on whether network slice ID or access category is used for partitioning.

Proposal 1: For network slicing, PRACH partitioning is supported. And in this case UE is unaware of network slicing if access category containing network slicing information is used for partitioning.

Access and overload control

There are multiple access and overload control mechanisms introduced in LTE, i.e. access barring, RACH backoff, RRC connection release/reject.

Access barring

Access barring is used in LTE to prevent certain UEs from accessing the network to solve the load condition. Access barring mechanisms introduced in LTE include ACB, EAB, SSAC, ACDC, UDT, to address different applications, services, call types, devices, signallings, etc.

Are different access barring parameters needed for different network slices? We think the answer is definitely yes because different network slices may have different load situations which requires different barring parameters. For example, there are two network slices A and B which belong to different business customers in a cell. Both of them have eMBB and mMTC services. The difference is that there is much less mMTC users in A than in B. Then it would be better if a bigger access barring factor is configured for A than for B.

As mentioned in the previous section, RAN2 was considering using "access category" specific access barring parameters for the support of one unified access barring mechanism. However, it is unclear whether such an access category is feasible to network slicing. If access category contains network slicing information, UE will be not aware of the network slicing.

Observation 2: For access barring, network slicing may not be visible to UE if access category includes network slicing information.

Proposal 2: Access barring should take different network slices into consideration. And in this case UE is unaware of network slicing if access category contains network slicing information.

RACH backoff

In LTE, backoff indicator is included in RAR. If NR follows this method and adopt PRACH partitioning for network slicing, network can know which network slice triggers the random access according to the PRACH resources the UE uses. Then the network can set the backoff indicator accordingly. There is no need for slice specific backoff indicator.

Observation 3: For RACH backoff, network slicing may not be visible to UE if PRACH partitioning is supported for network slicing.

RRC connection reject

RRC connection reject can be used for rejecting connection request for different type of services based on the establishment cause value contained in RRC connection request message. Establishment cause value includes emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess, mo-VoiceCall. Which type of services will be rejected depends on the load condition. In some cases, maybe only delayTolerantAccess will be rejected. In some other cases, even mo-data can be rejected. For network slicing, since different network slices are isolated, their load condition may be different. It is possible that different network slices may need to reject different sets of services based on their owe load condition. Therefore, network needs to know which network slice initiate the RRC connection establishment. PRACH partitioning is one way to provide this information.There is no need for other slice specific information.

Observation 4: For RRC connection reject, network slicing may not be visible to UE if PRACH partitioning is supported for network slicing.

Proposal 3: RACH backoff and RRC connection reject can base on PRACH partitioning for the handling of different network slices. There is no need for slice specific backoff indicator and network slice indication in the establishment cause. And in this case UE is unaware of network slicing.

RRC connection release

RRC connection release is used to release the established connection when congestion occurs. It is the last step. Before this, network may use RRC connection reconfiguration to release some low priority DRBs in a congested network slice or move them to another slice. In LTE release message, network can tell UE the time it needs to wait before initiating a connection establishment request. LTE introduces the extended wait time for delay tolerant access. For NR, such mechanism can also be used. Whether it can be used for non delay tolerant access is unclear. But Even If configuring the wait timer per service is necessary, there is no need to differentiate services from different network slices, since there will not be different network slices handle the same service in one UE.

Observation 5: For RRC connection release, network slicing may not be visible to UE.

Proposal 4: There is no need of different indication information in RRC connection release for different network slices. And in this case UE is unaware of network slicing.

Traffic transmission

In RAN2 NR ad Hoc meeting, RAN2 assumes that traffic for different slices is handled by different PDU sessions. Since RAN2 agreed before that different PDU sessions are mapped to different DRBs. It is safe to say that traffic for different slices is handled by different DRBs. Network can realize different slices by scheduling different DRBs on different carriers/RBs, which requires some enhancement to LTE, e.g. UL grant indicates the corresponding logical channel. To further reduce the impact between different DRBs, separate BSR/SR per logical channel may also be considered. All these kinds of mechanisms can work well together to support good isolation of different network slices without UE aware of network slicing.

Observation 6: For Traffic transmission for different slices, UE does not need to be aware of network slicing.

Proposal 5: For Traffic transmission for different slices, there is no need to indicate network slicing information. And in this case UE does not need to be aware of network slicing.

Mobility

Cell selection & reselection

As analyzed in [2], cell selection and reselection should take into account the availability of network slicing. To do this, one way is that the network broadcasts the supported network slices by network slice ID. So UE needs to understand the network slices to check whether the configured network slices in UE is supported by the network or not. Another way is, as suggested in [3], based on IdleModeMobilityControlInfo, where dedicated RAT/frequency selection criteria is included. In this method, the UE doesn't need to be aware of network slicing. However, we think this method works only for the case that network slicing is configured on a frequency scale not a cell level and the configuration doesn't change over time & locations. In [3], it claims to support the deployment of a large set of slices, but such a limitation fails to meet this business requirement as it requires flexible deployment.

Proposal 6: If cell selection & reselection takes into account the availability of network slicing, UE needs to be aware of network slicing.

Measurement report & handover

Similar to UE controlled mobility, network controlled mobility should also take into account the availability of network slicing. But it can be transparent to UE.

Proposal 7: For network controlled mobility, UE is unaware of network slicing.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation 1: For PRACH partitioning, whether network slicing is visible to UE depends on whether network slice ID or access category is used for partitioning.

Observation 2: For access barring, network slicing may not be visible to UE if access category includes network slicing information.

Observation 3: For RACH backoff, network slicing may not be visible to UE if PRACH partitioning is supported for network slicing.

Observation 4: For RRC connection reject, network slicing may not be visible to UE if PRACH partitioning is supported for network slicing.

Observation 5: For RRC connection release, network slicing may not be visible to UE.

Observation 6: For Traffic transmission for different slices, UE does not need to be aware of network slicing.

Based on the above discussion, we have the following proposals:

Proposal 1: For network slicing, PRACH partitioning is supported. And in this case UE is unaware of network slicing if access category containing network slicing information is used for partitioning.

Proposal 2: Access barring should take different network slices into consideration. And in this case UE is unaware of network slicing if access category contains network slicing information.

Proposal 3: RACH backoff and RRC connection reject can base on PRACH partitioning for the handling of different network slices. There is no need for slice specific backoff indicator and network slice indication in the establishment cause. And in this case UE is unaware of network slicing.

Proposal 4: There is no need of different indication information in RRC connection release for different network slices. And in this case UE is unaware of network slicing.

Proposal 5: For Traffic transmission for different slices, there is no need to indicate network slicing information. And in this case UE does not need to be aware of network slicing.

Proposal 6: If cell selection & reselection takes into account the availability of network slicing, UE needs to be aware of network slicing.

Proposal 7: For network controlled mobility, UE is unaware of network slicing.

References

[1] R2-1702441 LS on Access Control for NR.

[2] R2-1702530 Cell Selection and Reselection Consideration for Network Slicing, xiaomi.

[3] R2-1701052 Slice availability, Ericsson.