3GPP TSG-RAN WG2 Meeting #97bis	R2-1702643

Spokane, USA, 3 - 7 April 2017	R2-1700810

Agenda item:	10.3.4.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	QoS Flow Marking

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# 1	Introduction

SA2 has made the following agreements on the usage of reflective QoS for UL filter configuration [1]:

1c.	For B-type QoS profiles reflective QoS indication can be signalled to the UE via NG1.

1d.	Per packet NG3 indication can be used for Reflective QoS activation.

RAN2#96 made the following agreements on two level mapping [2]:

2:	In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow → QoSflow mapping, and AS is responsible for the QoSflow → DRB mapping (confirmation of SA2 agreement status).

3:	In UL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow → QoSflow mapping, and AS is responsible for the QoSflow → DRB mapping.

4:	DL packets over Uu are marked inband with QOS-flow-id for the purposes of reflective QoS.

5:	UL packets over Uu are marked inband with QOS-flow-id for the purposes of marking forwarded packets to the CN.

FFS for bullets 4 and 5 whether it can be semi-statically configured to not include the QOS flow ID in some cases.

RAN2 #96 also agreed on the usage of reflective QoS for AS layer filter configuration [2].

1	For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

2	The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

3    RRC can configure an uplink mapping

# 2	Discussion

## 2.1	UL flow marking

The RAN must map UL packets to transport QoS based on a QoS flow and include the flow ID in the N3 encapsulation header of uplink packets (to apply the correct charging in the CN). It can be assumed that RAN is always aware of which UL flows are mapped to which UL DRBs regardless of whether RAN has configured the AS filter (QoS flow to DRB mapping rule) or AS-reflective QoS is applied in the UE. Two situations can then be considered:

-	Only one UL QoS flow is mapped on a DRB: the RAN can set the UL N3 flow ID without indication from the radio packet (case 1)

-	Multiple UL QoS flows are mapped on DRB: the RAN needs the flow ID between the UE and gNB in order to derive which QoS flow ID to set on the G3 (case 2).

Observation 1: In the case of one to one UL QoS flow to DRB mapping, the RAN can determine the QoS flow from the UL DRB ID and attach the QoS flow ID into UL packet encapsulation header.

Based on the Observation 1 it is proposed:

Proposal 1: If a DRB carries only one QoS flow, the UE shall omit the QoS flow ID from that radio header.

NOTE:	the setting of the flow ID field in radio UL packet may also be triggered if RAN wants/needs to check the respect by the UE of the UL AS filter that RAN has decided (semi-static filter if AS configuration or dynamic filter if reflective QoS).

## 2.2	DL flow marking

DL QoS flow marking is needed for reflective QoS. One can consider two types of reflective QoS: AS-reflective QoS for the QoS flow to DRB mapping, and NAS-reflective QoS for the IP flow to QoS flow mapping. We define the AS-reflective QoS by the ability to update the UL AS filter in the UE by inband marking in DL packet and the NAS-reflective QoS by the ability to update the UL NAS filter in the UE by inband marking in DL packet.

As explained in [3] the reflective QoS on AS layer shall be decoupled from the reflective QoS on NAS layer. CN is responsible for IP flow to QoS flow mapping strategy and RAN is responsible for the QoS flow to DRB mapping strategy i.e. RAN decides whether to apply the reflective QoS or not on AS layer.

Assuming again that a flow ID is included in a DL radio packet only if needed to save bandwidth, the following combinations can apply at high level:

| AS Reflective | NAS reflective active | NAS reflective inactive |
| --- | --- | --- |
| Active | Set DL radio flow ID based on N3 flow ID | Set DL radio flow ID based on N3 flow ID |
| Inactive | Set DL radio flow ID based on N3 flow ID | DL Radio flow ID not set |

Observation 2: RAN shall set the DL radio flow ID based on NG3 flow ID if either AS- or NAS- reflective QoS is used.

As mentioned above RAN is naturally aware of when to use reflective QoS for AS filter configuration but Observation 2 necessitates that RAN is also aware when reflective QoS is used on NAS layer. SA2 has agreed in [1] that:

1c.	For B-type QoS profiles reflective QoS indication can be signalled to the UE via N1.

1d.	Per packet N3 indication can be used for Reflective QoS activation.

In the latter case (agreement 1d) RAN can determine from the N3 indication when the reflective QoS is used on the NAS layer, but if the reflective QoS is indicated over C-plane the RAN must also be informed in addition to the UE. It’s up to SA2 and RAN3 to specify details of C-plane indication to RAN.

Therefore, it is proposed:

Proposal 2: RAN shall be aware when the reflective QoS is used for a QoS flow on NAS and AS layers.

Proposal 3: RAN shall not include QoS flow IDs into DL packets if reflective QoS is not activated for NAS and AS layer UL mapping configuration.

NOTE: 	FFS if the QoS flow ID is needed in every DL packet even if the reflective QoS is activated either on NAS or AS layers

# 3	Conclusion

This contribution has made the following proposals:

Proposal 1: If DRB carries only one QoS flow, the UE may omit the QoS flow ID from the radio header.

Proposal 2: RAN shall be aware when the reflective QoS is used for a QoS flow on NAS and AS layers.

Proposal 3: RAN may not include QoS flow IDs into DL packets if reflective QoS is not activated for NAS and AS layer UL mapping configuration.

# References

- 3GPP TR23.799 v.2.0

- Draft Report of 3GPP TSG RAN WG2 meeting #96bis

- R2-1700064 Reflective QoS in AS, Nokia