3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702677

Spokane, USA 3th – 7th April 2017

Agenda Item:	10.2.3.3

Source:	Ericsson

Title:	RLM and RLF in NR

Document for:	Discussion, Decision

# Introduction

In LTE, RLM and RLF are key procedures. Thanks to RLM the UE can periodically monitor whether it is still within the coverage of its serving cell or not. Upon the detection that a radio failure occurs, UE will stop its uplink transmission and try to reselect to a new cell and re-establish its RRC connection. By this way, UE encountering RLF will not cause interference within that cell. As in LTE, for similar reasons, RLM and RLF are needed in NR.

In this contribution, we will discuss some difference in NR compared to LTE and then its impact on RLM and RLF in NR.

# Discussion

The purpose of the Radio Link Monitoring (RLM) function in the UE is to monitor the downlink radio link quality of the serving cell in RRC_CONNECTED state. Monitoring is based on the Cell-Specific Reference Signals (CRS). This in turn enables the UE when in RRC_CONNECTED state to determine whether it is in-sync or out-of-sync with respect to its serving cell as described in TS 36.213, Section 4.2.1.

The UE’s estimation of the downlink radio link quality is compared with out-of-sync and in-sync thresholds, Qout and Qin for the purpose of RLM. These thresholds are expressed in terms of the Block Error Rate (BLER) of a hypothetical Physical Downlink Control Channel (PDCCH) transmission from the serving cell. Specifically, Qout corresponds to a 10% BLER while Qin corresponds to a 2% BLER. The same threshold levels are applicable with and without DRX.

The mapping between the CRS based downlink quality and the hypothetical PDCCH BLER is up to the UE implementation. However, the performance is verified by conformance tests defined for various environments, as described in TS 36.521.

Figure 1 The whole RLM and RLF procedure in LTE

In LTE, RLM is specified largely in RAN1 specifications. Whether UE is in-sync or out-of-sync is based on hypothetical quality of PDCCH.

As in LTE, to understand the details of how RLM works in NR some progress in RAN1 is required first, such as which RS is used for RLM, how to judge UE is in-sync or out-of-sync, and the relation with lower layer procedures related to beam management such as beam recovery (details on that can be found in a companion contribution [1]). However, discussions in RAN1 seem to point that the same principle of RLM in LTE should be reused in NR, i.e. the estimated quality of downlink control channel (PDCCH in LTE) is used to judge whether UE is in-sync or out-of-sync with its serving cell.

In NR RLM should follow the same principle as in LTE, i.e. the hypothetical quality of downlink control channel in NR is used to judge if UE is in-sync or out-of-sync.

Although RAN1 has not decided which RS will be reused for RLM or whether a new RS will be defined, it is our understanding that the quality measurement on that RS should reflect the downlink control channel quality correctly. In that sense, it seems premature to make further agreements on RLM before RAN1 progresses its work on the design of DL control channels, including the aspects associated to how they would be beamformed. One of the RAN1 agreements states that NR should support UE/PDCCH-specific DMRS for PDCCH reception. At least for beamforming, UE may assume same precoding operation for PDCCH and associated DMRS for PDCCH. That is, PDCCH in NR is based on DMRS and could use UE specific beamforming. In addition to it, RAN1 states that NR should support using the same or different beams on control and the corresponding data transmissions.

PDCCH in NR is based on DMRS and should support using the same or different beams on control and the corresponding data transmissions.

Due to this reason, in our view, other RS discussed in NR, e.g. reference signals transmitted in the SS Block Set should be carefully considered since they may not be suitable as these RS may use quite different beamforming compared to PDCCH. Furthermore, still according to LTE RLM principle, the RS for RLM needs to enable a proper estimation of PDDCH quality i.e. it needs to be correlated in the frequency domain with the PDCHC quality. The SS Block Set will quite probably not expand the whole band (or at least not the PDCCH band). From this perspective, RAN1 should take a final decision on whether the SS Block Set can be used for RLM and to which extent.

RAN2 cannot assume that the SS Block Set signals can be used for RLM.

DMRS for PDCCH is quite natural to be used for RLM purpose. One drawback is that DMRS only exist when UE is scheduled. How to let UE know whether it is in-sync or out-of-sync when UE is not scheduled need be solved.

## Higher layer aspects of RLM

Regardless which RS will be defined by RAN1 to be used for RLM, RAN2 can at least make some assumptions concerning the RLM modelling and the interactions between L1 and L3. As in LTE, RAN2 should assume at least for the time being that the L1 will define Qin and Qout quality thresholds.

- Assume in NR Qin and Qout quality thresholds for RLM purposes to correspond to BLER levels of a hypothetical PDCCH to be indicated from L1 to L3.

- Send an LS to RAN1 requesting whether that assumption can be confirmed.

In scenarios where the RS used for RLM is beamformed, L1 will certainly rely on multiple beams to trigger Qout and Qin events, however, there is no reason to make that visible to the higher layers. Hence, from a L3 perspective, as in LTE, an NR UE could simply monitor the occurrences of Qin and Qout events indicated from L1 and, based on thresholds trigger entering and leaving conditions for RLF. For example, when the number of Qout events is higher than N events, RLF should be triggered and a timer initiated so the UE can detect new cells and/or be able to perform cell reselection.

- As in LTE, Qout and Qin events shall have thresholds to trigger entering and leaving conditions for RLF.

In LTE, when link starts to get deteriorated, the UE may try to regain sync with its serving cell via CRS and/or CSI-RS. That is in principle transparent to the higher layers except that if that is successful the higher layers may start receiving Qin indications that may trigger a leaving condition for RLF. In NR, a similar process will occur. In the multi-beam case, the UE will perform what is called beam recovery whose details should also be transparent to higher layers, except that it would be reflected on Qin indications that may trigger the leaving condition for RLF.

- Details on beam recovery procedures are transparent to higher layers.

When RLF is triggered in LTE, RRC connection re-establishment procedure is executed, where the UE first perform cell reselection. If the new selected cell is still a LTE cell, UE initiates random access procedure on that cell, and then send RRCConnectionReestablishmentRequest message toward NW. If the new selected cell is an inter-RAT cell, then UE should perform the actions upon leaving RRC_CONNECTED. In NR, once RLF is triggered the Ue needs to perform cell reselection and, in multi-beam deployments, the UE should perform beam selection before accessing. Notice that the UE could be performing DL beam tracking with neighbour cells to improve that process. Once cell reselection and beam selection occurs the UE can initiate random access. From an RRC perspective, there are different possibilities.

First, if the new selected cell is a NR cell, then UE should do RRC connection re-establishment procedure similar as in LTE. If the new selected cell is a LTE cell which connects to Next Generation Core, UE should continue with RRC re-establishment procedure as well instead of leaving RRC_CONNECTED even though this new selected cell is an inter-RAT cell. This is reasonable as UE can build up its context in LTE cell from old NR cell as the two cells are using same core network. If the new selected cell is a LTE cell which connects to legacy EPC or other inter-RAT cell, then UE should perform actions upon leaving RRC_CONNECTED.

When UE encounter RLF in NR and reselect to a NR cell or a LTE cell which connects to NextGenCore, RRC connection re-establishment procedure is applied. Otherwise, UE perform actions upon leaving RRC_CONNECTED.

Second, when NR works with carrier aggregation , since the carrier that NR use could be at high frequency, PCell may not always be the most robust carrier. When PCell encounter RLF, it is possible that SCell is still alive. Therefore, it is reasonable that UE does not initiate RRC connection re-establishment procedure when PCell encounters RLF but SCell still have good coverage. The UE can report such information to the network via the SCell. When the network receives such information, it can initiate a PCell change procedure to change PCell to a good SCell. The UE only initiates RRC connection re-establishment procedure when all serving cells encounter radio link issue.

For carrier aggregation in NR, when UE encounter RLF in PCell, as long as one SCell is alive, instead of initiate RRC connection re-establishment procedure, UE report such failure to NW to trigger a PCell handover procedure.

RRC connection re-establishment procedure is only initiated when all serving Cell encounter radio link issue.

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE, RLM is specified largely in RAN1 specifications. Whether UE is in-sync or out-of-sync is based on hypothetical quality of PDCCH.

Observation 2	PDCCH in NR is based on DMRS and should support using the same or different beams on control and the corresponding data transmissions.

- Observation 3    RAN2 cannot assume that the SS Block Set signals can be used for RLM.

Based on the discussion in section 2 we propose the following:

- Assume in NR Qin and Qout quality thresholds for RLM purposes to correspond to BLER levels of a hypothetical PDCCH to be indicated from L1 to L3.

- Send an LS to RAN1 requesting whether that assumption can be confirmed or not.

- As in LTE, Qout and Qin events shall have thresholds to trigger entering and leaving conditions for RLF.

- Details on beam recovery procedures are transparent to higher layers.

- When UE encounter RLF in NR and reselect to a NR cell or a LTE cell which connects to NextGenCore, RRC connection re-establishment procedure is applied. Otherwise, UE perform actions upon leaving RRC_CONNECTED

- For carrier aggregation in NR, when UE encounter RLF in PCell, as long as one SCell is alive, instead of initiate RRC connection re-establishment procedure, UE report such failure to NW to trigger a PCell handover procedure.

- RRC connection re-establishment procedure is only initiated when all serving Cell encounter radio link issue.

# References

[1] R2-1702678 Beam Recovery in NR