3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702744

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.3.1

Source:	Ericsson

Title:	PDCP TS design principles

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss specification design principles for NR PDCP. In particular, we review which improvements can be considered compared to the LTE PDCP specification.

# Discussion

In current LTE specification, the DL reception algorithms for the UE diverge for different cases of bearer configurations. The following algorithms are specified (TS 36.323, V14.1.0):

##### 5.1.2.1.2	Procedures for DRBs mapped on RLC AM when the reordering function is not used

  - Reception window handling, duplicate discard

  - Deciphering, header decompression

  - Expects in-sequence delivery from lower layers in normal operation

  - Reordering for out-of-sequence delivery at re-establishment of lower layers

##### 5.1.2.1.3	Procedures for DRBs mapped on RLC UM

  - Deciphering, header decompression

##### 5.1.2.1.4	Procedures for DRBs mapped on RLC AM and for LWA bearers when the reordering function is used

  - Reception window handling, duplicate discard

  - Deciphering

  - Reordering, reordering timer handling, in normal operation

##### 5.1.2.2	Procedures for SRBs

  - Deciphering

  - Integrity verification (duplicate discard part of it)

In NR, the PDCP reception procedure providing reordering is required when RLC delivers out of sequence, or when multiple lower layers deliver to PDCP. This will be for NR the case for both AM and UM bearers. Thus, for both AM and UM, PDCP must provide the reordering functionality.

Furthermore, in NR, also duplicate transmission of SRB data is envisaged, so that reordering/duplicate discard is also needed for SRB. The only unique function for SRB is integrity verification.

In conclusion, a common reception algorithm for all types, AM, UM, DRB and SRB should be considered as a design target for NR PDCP specification.

NR PDCP aims at defining a single algorithm for reception for a AM and UM, as well as DRB and SRB.

Whether header decompression, integrity verification and reordering (based on timer) should be used in this common algorithm should be configurable.

Another general design aspect that should be considered in the PDCP procedure design is whether description, conditions and state variables should be based on absolute SN or full COUNT. This discussion is independent on the PDCP PDU header structure, where the header should be based on SN (e.g. 12 or 18 bit, as discussed in [1]) to avoid the additional header overhead one would introduce by assigning the PDCP PDU headers the full COUNT (32bit). In the procedures however, working with COUNT, i.e. defining conditions based on comparisons in the procedure, provides better readability. Moreover, considering that COUNT it currently used in the defined reception algorithm with reordering anyway already in some state variables (Reordering_PDCP_RX_COUNT), it appears to be a straight forward and clean approach to base all state variables on COUNT.

The NR PDCP reception algorithm based on COUNT would be outlined as follow:

- COUNT determined for the received PDCP PDU.

- Duplicate discard for PDUs not in reception window (determined based on Last_Delivered_PDCP_RX_COUNT) and PDUs already received (same COUNT).

- Deciphering of received PDU based on COUNT

- Setting of Next_PDCP_RX_COUNT

- Storing of PDCP SDU associated with COUNT value

- Delivery to higher layers

- Updating of Last_Delivered_PDCP_RX_COUNT

- Maintaining of the reordering timer and Reordering_PDCP_RX_COUNT.

A draft CR text for this reception algorithm can be found in the Annex. This algorithm currently reflects the functionality used for AM with reordering in LTE, but can be extended with respect to Proposal 1.

NR PDCP procedures are based on COUNT instead of SN, which is determined in the beginning of the procedure. All NR PDCP state variables are based on COUNT.

# Conclusion

In summary, from the discussion in Section 2, we propose:

Proposal 1	NR PDCP aims at defining a single algorithm for reception for a AM and UM, as well as DRB and SRB.

Proposal 2	NR PDCP procedures are based on COUNT instead of SN, which is determined in the beginning of the procedure. All NR PDCP state variables are based on COUNT.

# References

[1] R2-1702741, PDCP SN length, Ericsson, TSG-RAN WG2 #97bis, Spokane, USA, 3rd – 7th April 2017

# Annex

##### X	Procedures for DRBs mapped on RLC AM

The procedures and most variables used herein use the sequence number space of the full COUNT, i.e., 32 bit. The sequence number in the PDCP PDU header has a sequence number length shorter than or equal to Maximum_PDCP_SN in accordance with the DRB configuration. Hence, the received PDCP SN is converted to the full sequence number space prior to any other processing.

The following definitions apply in this section:

PDCP_WINDOW_SIZE is half the PDCP Sequence Number space and hence derived as follows:

-	PDCP_WINDOW_SIZE = (Maximum_PDCP_SN + 1) / 2

###### X.1	Procedures when a PDCP PDU is received from the lower layers

Upon reception of a PDCP Data PDU with PDCP Sequence Number SN from lower layers, the UE shall:

-	set COUNT to SN + (Last_Delivered_PDCP_RX_COUNT & (Maximum_COUNT - Maximum_PDCP_SN));

-	if (COUNT <= Last_Delivered_PDCP_RX_COUNT - PDCP_WINDOW_SIZE)

[SN wraparound happened: assuming transmitter did not bring more than PDCP_WINDOW_SIZE in flight, COUNT value more than PDCP_WINDOW_SIZE behind Last_Delivered_PDCP_RX_COUNT is not possible, indicating that the previous estimated COUNT based on most significant bits of Last_Delivered_PDCP_RX_COUNT (HFN) is incorrect, i.e. must be incremented by one SN space]

-	set COUNT to (COUNT + (Maximum_PDCP_SN + 1))

-	Process the received PDCP PDU with COUNT in accordance with section X.2.

NOTE: (Maximum_COUNT - Maximum_PDCP_SN) is used to determine the Most Significant Bits of the full-size COUNT based sequence numbers (i.e. HFN) by performing a binary AND function (“&” operator).

###### X.2	Procedures for processing a received PDCP PDU with COUNT

For processing of a PDCP Data PDU with COUNT, the UE shall:

-	if (COUNT > Last_Delivered_PDCP_RX_COUNT)

-	if the PDCP PDU with the same COUNT has been received before

- 	discard the PDCP PDU;

-	else

-	if COUNT >= Next_PDCP_RX_COUNT

-	set Next_PDCP_RX_COUNT to COUNT + 1

-	perform deciphering of the PDCP PDU and store the resulting PDCP SDU;

-	if outOfSequenceDelivery is configured

-	deliver the SDU to higher layers

-	Update Last_Delivered_PDCP_RX_COUNT in accordance with section X.3

-	else

- 	discard the PDCP PDU;

###### X.3 	Updating Last_Delivered_PDCP_RX_COUNT

-	If the SDU with COUNT = Last_Delivered_PDCP_RX_COUNT + 1 has been received

-	if the SDU has not yet been delivered to higher layers

-	deliver the SDU to upper layers

-	set Last_Delivered_PDCP_RX_COUNT to Last_Delivered_PDCP_RX_COUNT + 1;

-	Update Last_Delivered_PDCP_RX_COUNT in accordance with section X.3 (recursively).

-	else

-	maintain t-Reordering in accordance with section X.4

NOTE: The recursive execution of the function in this sub-section advances Last_Delivered_PDCP_RX_COUNT beyond the sequence number of the last PDCP PDU that has been received in sequence.

NOTE: Whether to stop and or (re-)start t-Reordering should be verified when Last_Delivered_PDCP_RX_COUNT cannot be advanced further.

###### X.4	Maintaining t-Reordering

-	If t-Reordering is running:

-	if the PDCP SDU with Reordering_PDCP_RX_COUNT – 1 has been delivered to upper layers:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above):

-	if there is at least one stored PDCP SDU that has not been delivered to upper layers:

-	start t-Reordering;

-	set Reordering_PDCP_RX_COUNT to Next_PDCP_RX_COUNT.

###### X.4.1	Procedures when t-Reordering expires

When t-Reordering expires, the UE shall:

-	deliver to upper layers in ascending order of the associated COUNT value:

-	all stored PDCP SDU(s) with COUNT value(s) less than Reordering_PDCP_RX_COUNT;

-	all stored PDCP SDU(s) with consecutive COUNT value(s) starting from Reordering_PDCP_RX_COUNT;

-	set Last_Delivered_PDCP_RX_COUNT to the COUNT of the last PDCP SDU delivered to upper layers;

-	if there is at least one stored PDCP SDU:

-	start t-Reordering;

-	set Reordering_PDCP_RX_COUNT to Next_PDCP_RX_COUNT.

###### X.4.2	Procedures when the value of t-Reordering is reconfigured

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the UE shall:

-	stop and restart t-Reordering;

-	set Reordering_PDCP_RX_COUNT to Next_PDCP_RX_COUNT.