3GPP TSG RAN WG2 #97bis Meeting                                 	R2-1702840

# Spokane, USA, 3rd – 7th January 2017

# Source:			ZTE, ZTE Microelectronics

Title:	Consideration on the RACH procedure

Agenda Item:	10.3.1.4

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss the random access procedure in NR.

It has been agreed that a highly variable number of DL signals can be associated with subsets of RACH resources and subsets of preamble indices. We discuss a simple, yet flexible, framework for configuring such associations.

Measurements on DL signals form the basis of the random access procedure. The use of IDLE mode RS and CSI-RS for L3 mobility for this purpose is discussed.

DL Signals used for Random Access

In RAN1#88 and previous meetings, it was agreed that the UE uses measurements on DL signals/channels to select a subset of RACH resources and, at least for the case without gNB beam correspondence, a subset of preamble indices [1]. It is also suitable that the DL measurement is used for open-loop transmit power control for PRACH.

Agreements: - At least for the case without gNB Tx/Rx beam correspondence, gNB can configure an association between DL signal/channel, and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam. - Based on the DL measurement and the corresponding association, UE selects the subset of RACH resources and/or the subset of RACH preamble indices - A preamble index consists of preamble sequence index and OCC index, if OCC is supported - Note: a subset of preambles can be indicated by OCC indices

However, it has not yet been agreed which DL signals/channels (henceforth DL signals) that are used for this purpose. In our view, there are two main candidates for this purpose:

- IDLE mode RS

- CSI-RS for L3 mobility

It was agreed that IDLE mode RS consists of at least NR-SSS. It was also agreed that CSI-RS can be used for RRM measurements for L3 mobility, in addition to IDLE mode RS.

Agreements: - At least NR secondary synchronization signal (NR-SSS) is used for DL based RRM measurement for L3 mobility in IDLE mode - FFS in IDLE mode potentially additional use of DM-RS for PBCH (if defined) for measurement - FFS whether or not the NR-SSS alone will satisfy the requirements for RRM measurement Agreements: - For CONNECTED mode RRM measurement for L3 mobility, CSI-RS can be used, in addition to IDLE mode RS - Note that RAN1 will consider configuration overhead and possible inter-gNB signaling overhead - Detection of neighbor cell for measurement is based on NR-SS

In our view, the CSI-RS for L3 mobility should not be always-on. Instead, it could be turned on/off semi-statically and transmitted periodically when on [3][4]. When CSI-RS is turned off, it is necessary to use the IDLE mode RS as DL signal. The number of measured ports/beams/TRPs on IDLE mode RS depends on the number of transmitted SS blocks per SS burst set.

In many scenarios, it is more efficient to provide measurements on many ports/beams/TRPs using CSI-RS than using IDLE mode RS. For example in a multi-TRP multi-beam cell, it may be sufficient to transmit only a single SS block using joint quasi-omni transmission from the multiple TRPs. In this case, CSI-RS for L3 mobility could be used to measure individual TRPs/beams in the cell and used as DL signal in random access.

Proposal 1: Both IDLE mode RS and CSI-RS for L3 mobility can be used as DL signal in random access.

RA-related Configurations in Remaining Minimum SI

Overview

The SS burst set configuration and the configuration of CSI-RS for L3 mobility are both needed for random access, in our view. Therefore, we propose that they are included in the remaining minimum SI, together with the common random access configuration.

SS Burst Set Configuration

In RAN1#88, it was agreed that the maximum number of SS blocks, L, may be carrier frequency dependent [1].

Agreements: - The maximum number of SS-blocks, L, within SS burst set may be carrier frequency dependent - For frequency range category #A (e.g., 0 ~ 6 GHz), the number (L) is TBD within L ≤ [16] - For frequency range category #B (e.g., 6 ~ 60GHz), the number is TBD within L ≤ [128] - FFS: L for additional frequency range category - The position(s) of actual transmitted SS-blocks can be informed for helping CONNECTED/IDLE mode measurement, for helping CONNECTED mode UE to receive DL data/control in unused SS-blocks and potentially for helping IDLE mode UE to receive DL data/control in unused SS-blocks - FFS whether this information is available only in CONNECTED mode or in both modes - FFS how to signal the position(s)

The maximum number L should cover the most extreme deployment scenario, also taking forward compatibility into account. In practice, many cells may use only a subset of these L nominal SS blocks. Therefore, it was also agreed that the position(s) of the actually transmitted SS blocks can be informed to UEs [2], here called the SS burst set configuration. Since random access may be based on measurements on signals in SS blocks, e.g. NR-SSS, the SS burst set configuration can be seen as part of the random access configuration. Therefore, it should be available to both IDLE and CONNECTED mode UEs in the remaining minimum SI.

Proposal 2: The position(s) of actually transmitted SS blocks is included in remaining minimum SI.

CSI-RS for L3 Mobility Configuration

As discussed above, it would be beneficial to support the configurability of whether UEs use measurements on IDLE mode RS or CSI-RS for L3 mobility to select subsets of RACH resources and preambles. For this reason and to better support inter-cell measurements based on CSI-RS for L3 mobility, the configuration of CSI-RS for L3 mobility should also be included in the remaining minimum SI. Note that the configuration also indicates if the CSI-RS is transmitted or not (on/off).

Proposal 3: The configuration of CSI-RS for L3 mobility is included in remaining minimum SI.

Random Access Configuration

In addition to the SS burst set configuration and the configuration of CSI-RS for L3 mobility, the remaining minimum SI should also include the common random access configuration, which is used by UEs without a dedicated configuration, such as IDLE mode UEs. Since random access may be used to request delivery of other SI, it is not suitable to include the random access configuration in the other SI.

Proposal 4: The common random access configuration is included in remaining minimum SI.

Random Access Configuration

PRACH groups

For convenience, we can define a PRACH group as:

- A PRACH group is a subset of RACH resources and/or a subset of preamble indices.

PRACH groups are disjoint if any combination of RACH resource and preamble is contained in at most one PRACH group.

The Number of Measurement Results

The random access configuration should configure the association between DL signals and PRACH groups. Note that number of DL signals may be quite different in different scenarios. It may depend on:

- The number of actually transmitted SS blocks (if IDLE mode RS are used),

- The total number antenna ports for CSI-RS for L3 mobility (if CSI-RS for L3 mobility are used).

For example the number of SS blocks may vary between 1 and [128] for above 6 GHz. Two ways to handle this are:

Option 1: 	Associations between the maximum number of DL signals (e.g. [128] in the case of SS blocks) and RACH groups are defined.

Option 2: 	Associations between the actual number of DL signals and PRACH groups are defined.

Option 1:

In the case with gNB beam correspondence, the maximum number of PRACH groups (e.g. [128]) would have to be defined. For the scenario with analog beamforming, these nominal PRACH groups would consist of subsets of RACH resources in different time instants. Even though the gNB could schedule other transmissions during the RACH resources associated with actually not transmitted DL signals, the actually used RACH resources would be unnecessarily spread out in time, causing unnecessarily long latency until Msg1 transmission.

Option 2:

In this case, number of PRACH groups would correspond to the actual number of DL signals. For instance, if it’s indicated in the SS burst set configuration that only 16 SS blocks are actually transmitted (even though L=[128]), only 16 RACH groups would have to be defined. Compared to Option 1, the waiting time until Msg1 transmission could be reduced by a factor 8 in the example with analog beamforming and L=[128].

Hence, we propose to use the knowledge of the number of actually transmitted RS in the random access configuration, i.e. Option 2:

Proposal 5: If the number of actually transmitted DL signals (e.g. IDLE mode RS or CSI-RS for L3 mobility) is informed to UEs, then the random access configuration only defines associations for those DL signals. Otherwise, the random access configuration defines associations for the maximum number of DL signals assumed by UEs (e.g. L in the case of IDLE mode RS).

Associations between DL signals and PRACH groups

If a cell transmits CSI-RS for L3 mobility, according to the configuration in the remaining minimum SI, the random access configuration needs to indicate whether the IDLE mode RS or the CSI-RS for L3 mobility should be used for selecting PRACH group.

Proposal 6: The random access configuration can indicate if IDLE mode RS or CSI-RS for L3 mobility (if configured) should be used for selecting subset of RACH resources and preamble indices.

The associations between DL signals and PRACH groups are configured in the random access configuration. The following bullets list an example of parameters to define PRACH groups and the associations between DL signals and PRACH groups:

- The number of DL signals

  - This parameter is obtained from the SS burst set configuration or the configuration of CSI-RS for L3 mobility.

- The MSG1 preamble format

  - Including various numbers of multiple/repeated preambles within a MSG1 transmission

- The time and frequency resources for MSG1 transmission

  - In [5], RACH transmission occasion was defined as the time-frequency resource on which a MSG1 can be transmitted (if multiple/repeated preamble transmission is configured, this counts as one RACH transmission occasion).

  - Let MSG1 time instance denote the time instance of a RACH transmission occasion. In other words, there may be multiple frequency multiplexed RACH transmission occasions on the same MSG1 time instance.

- The PRACH group period, i.e. after how many MSG1 time instances the same PRACH groups are repeated.

  - One value, e.g. 1, corresponds to the same PRACH groups repeating in each MSG1 time instance. This configuration is useful in the case without TRP beam correspondence, i.e. there is no particular association from measurement result to MSG1 time instance.

  - Other values correspond to PRACH groups occuring in a subset of the MSG1 time instances, e.g. every 16th MSG1 time instance, which is useful for example in the case with TRP beam correspondence in an analog beamforming implementation, e.g. with 16 analog beams, i.e. measurement results are associated to subsets of MSG1 time instances.

- The number of PRACH groups per MSG1 time instance

  - PRACH groups on the same MSG1 time instance are separated by frequency and/or preamble subset.

- The number of preambles in a PRACH group

Further subdivision of the set of preambles in the PRACH groups can be considered, e.g. for the purpose of MSG3 transmission resource size indication as in LTE.

The parameters listed under C correspond to the PRACH resource configuration in LTE, in that they define the available RACH resources. Note that other RACH resources may be available to other UEs based on UE-specific random access configurations.

The parameters D and E define the number of PRACH groups (N), which is equal to the number of PRACH groups per MSG1 time instance times the number of MSG1 time instances with different PRACH groups, i.e. N=E*D.

Given the parameters above, an association rule can be defined that associates DL signals in the same time instance with PRACH groups in the same MSG1 time instance, as far as possible.

Proposal 7: An association rule between DL signals and PRACH groups is defined, such that DL signals transmitted in the same time instance are associated with PRACH groups in the same MSG1 time instance, as far as possible.

Naturally, both UEs and gNBs use the same association rule. This is illustrated with a few examples in the Annex.

Contention-free Random access

CONNECTED mode random access in NR may be applicable for the use-cases in LTE such as handover, establishing UL sync, scheduling request, etc, but possibly also for new use cases. The issues with analog beamforming and beam correspondence also apply to contention-free random access. Therefore, the same kind of framework with association between DL signals and PRACH groups should be applied to this case. This means that also scenarios with multiple PRACH groups, distinguished by disjoint subsets of preambles, within one MSG1 time instance should be supported. In such cases, the UE might not be configured with a single (dedicated) preamble index, but with a dedicated preamble index per PRACH group. These can be seen single-preamble PRACH groups, disjoint with the PRACH groups used for contention-based random access. The association framework enables beamformed reception of MSG1 (in the case of TRP beam reciprocity) and best DL Tx beam indication, also for contention-free random access.

Proposal 8: In contention-free random access, the UE can be configured with a (possibly different) preamble per PRACH group.

MSG1 Transmission

As discussed in more detail in [6][7], MSG1 transmission before the end of a monitored MSG2 reception window can be useful, in particular for contention-free random access. Even in the scenario with UEs with ideal Tx/Rx beam correspondence and analog beamforming, it may take extensive sweeping to find the best pair of TRP Tx beam and UE Rx beam, since only one UE Rx beam can be used at a time. The issue becomes more severe with longer DL signal periodicity, which can be up to 160 ms for SS burst set. Instead, the UE may sweep its Rx beam until it finds a beam pair that is sufficiently good, e.g. SNR above a threshold, and use that “initial” UE Rx beam also for the MSG1 transmission. In the numerical evaluation described in the appendix, we show that the likelihood is significant of finding a better UE Tx beam if multiple MSG1 transmissions with different UE Tx beams is allowed.

Proposal 9: MSG1 transmission before the end of a monitored MSG2 reception window of the previously transmitted MSG1 is supported.

As described above, a UE selects a PRACH group based on the DL signal with best measurement results, which could correspond to the best DL Tx beam. If the UE uses IDLE mode RS for this, the measurement results are SS-block-RSRP. If the UE uses CSI-RS for L3 mobility for this, the measurement results are the RSRP of the CSI-RS, e.g. “CSI-RSRP”. The power control for MSG1 transmit power control should be based on these measurement results.

Proposal 10: The MSG1 transmit power control is based on the best measurement result (e.g. SS-block-RSRP or CSI-RSRP) that was used to select subsets of RACH resources and preamble indices.

At least for UEs without beam correspondence, UL Tx beam sweeping should normally be performed before the preamble transmit power is ramped up. It is beneficial to find matching beam pairs as early as possible in the random access procedure, in order to save UE power and keep down the interference power level. In particular, if multiple MSG1 transmission before the end of a monitored MSG2 reception window is used, then the UE should not ramp up Tx power during those transmissions.

Proposal 11: UE Tx beam sweeping of MSG1, if used, should be performed before UE Tx power ramping. UE Tx power should not ramped up during multiple MSG1 transmissions before the end of a monitored MSG2 reception window.

MSG2 Reception

As discussed in [6], there are several scenarios where multiple MSG2 can be transmitted to a UE within a random access attempt, such as multiple MSG1 transmission and reception of the same MSG1 at distributed TRPs connected with non-ideal backhaul. In order to improve the subsequent communication in such scenarios, the UE shouldn’t always terminate the MSG2 reception after the first successfully received MSG2.

Proposal 12: The reception of multiple MSG2 is supported.

Conclusions

The following proposals have been made:

Proposal 1: Both IDLE mode RS and CSI-RS for L3 mobility can be used as DL signal in random access.

Proposal 2: The position(s) of actually transmitted SS blocks is included in remaining minimum SI.

Proposal 3: The configuration of CSI-RS for L3 mobility is included in remaining minimum SI.

Proposal 4: The common random access configuration is included in remaining minimum SI.

Proposal 5: If the number of actually transmitted DL signals (e.g. IDLE mode RS or CSI-RS for L3 mobility) is informed to UEs, then the random access configuration only defines associations for those DL signals. Otherwise, the random access configuration defines associations for the maximum number of DL signals assumed by UEs (e.g. L in the case of IDLE mode RS).

Proposal 6: The random access configuration can indicate if IDLE mode RS or CSI-RS for L3 mobility (if configured) should be used for selecting subset of RACH resources and preamble indices.

Proposal 7: An association rule between DL signals and PRACH groups is defined, such that DL signals transmitted in the same time instance are associated with PRACH groups in the same MSG1 time instance, as far as possible.

Proposal 8: In contention-free random access, the UE can be configured with a (possibly different) preamble per PRACH group.

Proposal 9: MSG1 transmission before the end of a monitored MSG2 reception window of the previously transmitted MSG1 is supported.

Proposal 10: The MSG1 transmit power control is based on the best measurement result (e.g. SS-block-RSRP or CSI-RSRP) that was used to select subsets of RACH resources and preamble indices.

Proposal 11: UE Tx beam sweeping of MSG1, if used, should be performed before UE Tx power ramping. UE Tx power should not ramped up during multiple MSG1 transmissions before the end of a monitored MSG2 reception window.

Proposal 12: The reception of multiple MSG2 is supported.

- References

- Chairman’s notes RAN1#88, 2017-03

- R1-1704358, “Composition of SS block, burst and burst set”, ZTE, ZTE Microelectronics

- R1-1704366, “CSI-RS for L3 mobility”, ZTE, ZTE Microelectronics

- R1-1704367, “Mobility procedure”, ZTE, ZTE Microelectronics

- Chairman’s notes RAN1 NR_AH 2017-01

- R1-1701581, “4-step Random Access Procedure”, ZTE, ZTE Microelectronics

- R1-1703399, “On multiple Msg.1 transmissions”, ZTE, ZTE Microelectronics, Lenovo, Sony, Nokia, Alcatel-Lucent Shanghai Bell

Annex A – Examples of associations between DL signal and PRACH groups

- A.1	gNB with Rx/Tx Beam Correspondence

In this example, the gNB configures a preamble (MSG1) format (parameter B above) without multiple symbols, i.e Rx beam sweeping within the configured format is not needed. Furthermore, assume that 4 RACH transmission occasions within a radio frame are defined, using parameter C above. Since a single RACH transmission occasion per time is configured, also the number of MSG1 time instances within a frame is 4. This is illustrated in Figure 1 below. Note that parameter B and C may be combined into a PRACH configuration index as in LTE (0-63 in LTE).

Figure 1: Four RACH transmission occasions (and four MSG1 time instance) per radio frame are defined, e.g. using a PRACH configuration index as in LTE.

- A.1.1	Example 1: Single DL signal (as LTE)

In this simple example, the number of DL signals is 1, i.e. A=1. For example, the SS burst set configuration informs UEs that the SS burst set contains a single SS block. Hence, the following configuration can be used:

- D = 1 (the PRACH group period)

- E = 1 (the number of PRACH groups per MSG1 time instance)

- F = all preamble indices (the number of preamble indices in a PRACH group)

The single DL signal is associated with PRACH group 0, i.e. all UEs select the same PRACH group. Note that the variable F would have to take preambles for contention-free random access into account.

Figure 2:

- A.1.2	Example 2: Four DL signals in different time instances (analog beamforming at TRP)

In this example, the number of DL signals is 4, i.e. A=4, and they are transmitted in different time instance, e.g. 4 different SS blocks. The following configuration can be used:

- D = 4 (the PRACH group period)

- E = 1 (the number of PRACH groups per MSG1 time instance)

- F = all preamble indices (the number of preamble indices in a PRACH group)

Since, D = 4, the same PRACH group occurs after four MSG1 time instances, as illustrated in Figure 3. The association between the 4 DL signals and the 4 PRACH groups can follow the index of the DL signals, e.g. the SS block index.

Figure 3:

- A.1.3	Example 3: Four DL signals – Two DL signals per time instance (Two TRPs with analog beamforming)

In this example, the number of DL signals is 4, i.e. A=4. However, two DL signals are transmitted simultaneously and the other two are also transmitted simultaneously but at another time instance. This illustrates a scenario where CSI-RS for L3 mobility is used as DL signal and two CSI-RS port are transmitted at the same time. An example of such an implementation is a cell with two TRPs with analog beamforming that transmit a CSI-RS each simultaneously, but with different DL Tx beams in the two different time instances (totally four different DL Tx beams). In this case, the following configuration can be used:

- D = 2 (the PRACH group period)

- E = 2 (the number of PRACH groups per MSG1 time instance)

- F = half of all preamble indices (the number of preamble indices in a PRACH group)

Since, D = 2, the same PRACH group occurs every second MSG1 time instances, as illustrated in Figure 4. The association rule states that the two DL signals that are transmitted in the same time instance should be associated with PRACH groups in the same MSG1 time instance. In the example implementation with two TRPs, each TRP receives a PRACH group in each MSG1 time instance, distinguished by different subsets of preambles. The TRPs can use different Rx beams in the two different MSG1 time instances. Note the configuration in Example 2 could also work in this case, but this configuration offers reduced delay until MSG1 transmission, compared to Example 2.

Figure 4:

- A.1.4	Example 4: Multiple DL signals (digital beamforming at TRP)

In this example, there are multiple DL signals , i.e. A>1. For example, the CSI-RS for L3 mobility is used, with multiple simultanous CSI-RS transmitted by the TRP using different Tx beams. The following configuration can be used:

- D = 1 (the PRACH group period)

- E = 1 (the number of PRACH groups per MSG1 time instance)

- F = all preamble indices (the number of preamble indices in a PRACH group)

Note that this configuration is the same as in Example 1 with a single DL signal. In this case, there is only one PRACH group, since D = 1 and E = 1, so all DL signals are associated with the same PRACH group 0. However, the TRP can use digital beamforming to receive all beams in each RACH transmission occasion and it can use beam correspondence to obtain the DL Tx beam to use for MSG2.

Figure 5:

- A.2	gNB without Rx/Tx Beam Correspondence

In this example, the gNB configures a preamble (MSG1) format (parameter B above) with multiple symbols, i.e Rx beam sweeping within the configured format can be performed. Furthermore, assume that 8 RACH transmission occasions within a radio frame are defined, using parameter C above. Since two RACH transmission occasions per time is configured, the number of MSG1 time instances within a frame is 4 in this example, which is is illustrated in Figure 4 below.

Figure 6:

- A.2.1	Example 5: Four DL signals in different time instances (analog beamforming at TRP)

In this example, the number of DL signals is 4, i.e. A=4, and they are transmitted in different time instance, e.g. 4 different SS blocks. The following configuration can be used:

- D = 1 (the PRACH group period)

- E = 4 (the number of PRACH groups per MSG1 time instance)

- F = half of all preamble indices (the number of preamble indices in a PRACH group)

Since, D = 1, the same PRACH group occurs in each MSG1 time instance, as illustrated in Figure 5. In each RACH transmission occasion, there are two disjoint PRACH groups, using disjoint subsets of preamble indices. The association between the 4 DL signals and the 4 PRACH groups can follow the index of the DL signals, e.g. the SS block index.

Figure 7:

- A.2.2	Example 6: 16 DL signals in different time instances (analog beamforming at TRP)

In this example, the number of DL signals is 16, i.e. A=16, and they are transmitted in different time instance, e.g. 16 different SS blocks. The same configuration as in Example 5 can be used:

- D = 1 (the PRACH group period)

- E = 4 (the number of PRACH groups per MSG1 time instance)

- F = half of all preamble indices (the number of preamble indices in a PRACH group)

In this example, there are more DL signals than RACH group, so multiple DL signals need to be associated with the same PRACH group. In this case 4 DL signals need to be associated with the same PRACH group. This means that the DL Tx beam for MSG2 is not fully conveyed through the selection of PRACH group. How to handle transmit MSG2 is up to the network implementation. In this case, MSG3 can complete the indication of the best DL Tx beam, in this case indicate one of the four remaining candidate beams for best DL Tx beam.

Figure 8: