3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702859

Spokane, US, 3rd – 7th April 2017

Agenda Item:	10.4.1.4

Source:	Ericsson

Title:	Encoding of broadcasted minimum NR system information

Document for:	Discussion, Decision

# Introduction

At the RAN2#97 meeting it was agreed that “system information valid in one cell may be valid also in other cells”. Still different cells need to have at least some parameters that are different in the broadcasted minimum SI, such as PRACH sequence index. This is discussed in [1].

Therefore, there is a need to encode the broadcasted minimum SI related to different cells without forcing all parameters to be identical; or without duplicating every parameter in every SIB in case the minimum SI contains SI related to more than one cell.

If we allow the broadcasted minimum SI to contain SI related to more than one cell, then we need a method of indicating to the UE which parts of the SI that corresponds to which cell. And we need a way to efficiently encode information that is very similar. If we would e.g. provide one copy of each SIB for each cell, then this would result in a large amount of duplication of parameters. To avoid this, we need to introduce an encoding method that does not result in excessive duplication of parameters that are the same in different cells. In this paper we discuss and propose a method to avoid such duplication.

# Discussion

If done right, the additional cost of providing the UE with SI related to one additional beam, or one additional cell, or one additional configuration, can be made very small in many deployments.

When the broadcasted minimum SI contains information valid in more than one cell then the UE need one or more identifiers in the encoded minimum SI that indicates for which cell a part of the SI corresponds to. A similar method used to encode SIB-variants valid in different cells can be used to encode SI-variants valid in different beams or to encode different SI for different cell states.

The minimum SI can encode different SI for different cells, beams, and/or states (e.g. using the PCI, TSS, or valueTag in the NR-MIB as identifiers)

One straightforward way to encode system information where only a small number of parameters have different values, e.g. for different PCIs, different TSS-values, or different valueTag-values is to first encode the common parameters jointly and then encode the parameters that are different separately. This is a very efficient way of avoiding duplication of parameters and only explicitly encoding parameters that differs for different PCI-values, TSS-values, and/or valueTag-values.

The encoding of the SIBs relating to different cells, beams or states in the minimum SI is done in a way that avoids duplication of parameters with identical values. Common parameters are encoded jointly and only parameters that differ are explicitly encoded separately.

Before the details of exactly what parameters that can be used to differentiate SIBs in the broadcasted minimum SI it is difficult to provide a concrete example. Encoding “common SIBs and delta SIBs” is one way to avoid parameter duplication, but additional methods are also possible.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The minimum SI can encode different SI for different cells, beams, and/or states (e.g. using the PCI, TSS, or valueTag in the NR-MIB as identifiers)

Proposal 2	The encoding of the SIBs relating to different cells, beams or states in the minimum SI is done in a way that avoids duplication of parameters with identical values. Common parameters are encoded jointly and only parameters that differ are explicitly encoded separately.

# References

R2-1702858, Stored system information, Ericsson, RAN2#97bis, Spokane, USA, 3rd – 7th April, 2017