3GPP TSG-RAN WG2 Meeting #97bis                              	R2-1702910

Spokane, USA, 3rd –7th April 2017

Source:	CATT

Title:	Open Issues of P2V

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN2#97 meeting [1], the agreement of P2V was:

Agreements: - eNB may provide resource pool configuration for P2V in broadcast/dedicated signalling. Whether this pool configuration points to same physical resources as V2V pool or not is eNB choice - More than one permissions which to enable “random selection”, “partial sensing”, or “either random selection or partial sensing” can be configured to associated with a P2V resource pool - One or more resource pools are allowed to be configured for P2V transmission, depends on eNB implementation. If the eNB doesn’t provide a random selection pool then UEs that only support random selection cannot perform V2P. - One or more resource pools are allowed to be configured for P2V transmission, depends on eNB implementation. - One or multiple resource pools may be configured in dedicated RRC signaling, depending on eNB implementation - P2V resource pool configuration is a separate IE from V2V pool configuration, which may contain both shared resource and/or dedicated resource information - It is not mandatory for P-UEs to support zone-based resource selection. The UE reports whether it supports zone-based resource selection. This is reported in UE capability. - Zone-based configuration cannot be configured for P2V resource in broadcast signalling. - If the UE supports zone-based resource selection, the network can provide zone-based configuration if it would like. - For P-UEs configured to allow “either random selection or partial sensing”, then it is up to UE implementation to select a resource selection method if there exist transmission resource pool(s) in which both methods are permitted - There is no need for including resource selection method in P-UE SidelinkUEinformaiton message to eNB, because P-UE has already indicated this in UE Capability - UEs (P2V and V2V) shall only use random selection in exceptional pool - If the UE is configured to do partial sensing only the UE should use partial sensing that pool (e.g. the UE is not allowed to do random selection). - As a baseline, for power saving can be achieved by UE implementation and upper layer mechanisms. - P-UEs do not performs CBR measurement. The configuration parameters can be dynamically provided to the UE via eNB RRC signaling. FFS whether a CBR value or the full parameters are provided. FFS whether it is dedicated/broadcast or both.

The concern open issues from the above agreements are:

- The behavior of P-UEs which only support random selection when the eNB doesn’t provide a random selection pool for P-UE.

- Which random selection resource pool below should be selected by the P-UE doing partial sensing when the sensing results are not available.

Option 1: random selection resource pool configured for P2V.

Option 2: exceptional pool.

In this contribution, we discuss the issues above.

Discussion

The behavior of P-UEs which only support random selection when the eNB doesn’t provide a random selection pool for P-UE.

Based on the agreement made in the last RAN2 meeting, P-UEs that only support random selection can perform P2V transmission if the eNB doesn’t provide a random selection pool. However, there are two approaches that P-UE can perform P2V transmission in this case:

- P-UEs that only support random selection perform P2V transmission using the resources of exceptional pool.

- P-UEs that only support random selection request the transmission resources from eNB.

For approach 1, RAN2 should make a decision whether introduce this case into the exceptional pool use cases.

For approach 2, it should not be restricted that UE requests the transmission resources from eNB. Hence, approach 2 should be supported in specification.

Proposal 1: P-UEs that only support random selection perform P2V transmission using the resources of exceptional pool if the random selection pool for P2V is not configured.

Proposal 2: P-UEs that only support random selection request the transmission resources from eNB if the random selection pool for P2V is not configured.

If both approach 1 and approach 2 are supported, which approach is used depends on UE implementation.

Which random selection resource pool should be selected by the P-UE doing partial sensing when the sensing results are not available.

In general, eNB configures a random selection resource pool for P-UE. For the scenario that both random selection resource pool for P2V and exceptional pool are configured, which pool should be selected by the P-UE doing partial sensing when the sensing results are not available?

Since random selection resource pool for P2V is a dedicated pool configured for P2V, it is benefit that P-UE selects the resources from P2V resource pool. It is proposed P-UE selects the resources from random selection resource pool for P2V is prioritized over from exceptional pool.

Proposal 3: P-UE selects the resources from random selection resource pool for P2V is prioritized over from exceptional pool when the sensing results are not available.

Conclusion

In this contribution, we discuss the P-UE behavior and propose:

Proposal 1: P-UEs that only support random selection perform P2V transmission using the resources of exceptional pool if the random selection pool for P2V is not configured.

Proposal 2: P-UEs that only support random selection request the transmission resources from eNB if the random selection pool for P2V is not configured.

Proposal 3: P-UE selects the resources from random selection resource pool for P2V is prioritized over from exceptional pool when the sensing results are not available.

Reference

[1] RAN2#97, chairman notes.