3GPP TSG-RAN WG2 Meeting #97 bis                                                              				          R2-1702928

Spokane, USA, 3rd-7th April 2017

Agenda item:	10.2.3.1

Source:	CMCC

Title:	RLC SDU delivery consideration

Introduction

Regarding the delivery of  PDCP PDU (i.e. RLC SDU) from RLC to PDCP layer,  the following agreements were reached at RAN2#95bis:

Agreement

1: NR specification should not prohibit out-of-order deciphering of PDCP PDUs.

Agreements

Proposal 1: Complete PDCP PDUs can be delivered out-of-order from RLC to PDCP. RLC delivers PDCP PDUs to PDCP after the PDU is reassembled

Proposal 1a: PDCP reordering is always enabled if in sequence delivery to layers above PDCP is needed (i.e. even in non-DC case)

Accordingly, the following description has been captured in TR 38.804:

The part of services and functions of the PDCP sublayer for the user plane include:

-	Reordering and duplicate detection (if in order delivery to layers above PDCP is required);

From the agreements reached in RAN2#95bis and the description captured in TR 38.804, we came to a conclusion that RLC may deliver RLC SDU out-of-order to PDCP. However, this will increase the workload for PDCP due to the reordering operation.

In this contribution, we propose to introduce an ascending order delivery mechanism in RLC to optimize the PDCP PDU delivery from RLC to PDCP, which can achieve PDCP workload reduction without any additional RLC overhead.

Discussion

From TS 36.322 [1], we obtain that it is up to RLC entity to judge whether a RLC data PDU can be placed in the reception buffer or be discarded by comparing the SN of RLC data PDU with variable-dependant receiving window.  Because this justification procedure of receiving window can help avoid SN ambiguity of RLC data PDU. So, before RLC delivers data PDUs to PDCP, all RLC data PDUs should be filtered by receiving window in RLC UM and AM.

Observation 1: Before RLC delivers data PDUs to PDCP, all RLC data PDUs should be filtered by receiving window in RLC UM and AM.

Observation 2: SN of each RLC data PDU should be recognised by RLC in UM and AM.

RLC ascending order delivery

For each data PDUs RLC received, their SN can be recognised when judging whether it meets the requirement of receiving buffer. RLC SNs are reordered in ascending order sequence without considering whether the SN is discontinuities caused either by data PDU missing or by retransmission. After the processes at RLC including removing RLC header and reassembling SDU, RLC delivers corresponding RLC SDUs to PDCP according to the ascending order of its original RLC SN. AMD PDUs and UMD PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to 1023 for AMD PDU and 0 to [2[sn-FieldLength] – 1] for UMD PDU (Sn-FieldLength gives the UM SN field size in bits)[1], so ascending order should consider the cycling of SN.  Each delivery from RLC to PDCP may contain one or more PDCP PDU(s) from one MAC SDU. RLC data PDUs are all placed in the receiving buffer, and the ascending order delivery in RLC is based on their original RLC PDU SNs, so the ascending order delivery procedure does not need any additional buffer in RLC entity. The delivery procedure is independent of ARQ function in AM, which means the RLC ascending order delivery does not require any additional latency. Comparing the situation of out-of-order delivery, part of reordering workload is moved from PDCP to RLC. Those part reordering in RLC is based on SN not including PDCP PDU, so from implementation perspective, it is more efficient than reordering in PDCP which reorders PDCP PDUs themselves.

Figure 1 provides an example of ascending order delivery without RLC data PDU missing. As shown in figure 1, MAC SDU contains four RLC PDUs, the corresponding RLC SNs are 4, 3, 5 and 6.  Although the SN order of RLC PDU received is 4, 3, 5, 6, the delivery order from RLC to PDCP is based on SN ascending order: 3, 4, 5, 6.

Figure 1 Ascending order delivery without RLC data PDU missing

Ascending order delivery with RLC PDU missing

Based on the discussion on out-of-order delivery [2], RLC SDUs from one MAC SDU can be delivered to PDCP without considering whether there is any missing RLC PDU in AM and UM. Similarly, if missing RLC PDU exists, RLC SDUs can be delivered to PDCP in the way we describe in 2.1. In addition, ARQ function in RLC AM is independent from ascending order delivery, T-reordering mechanism will used for waiting for the missing RLC PUD. From the perspective of two or more successive MAC SDUs, RLC SDUs transmitted in one MAC SDU can be ascending order delivered to PDCP. The “holes” in RLC SUD can be re-transmitted, and retransmitted RLC PUD may exist in following RLC SDU. Because the ascending order delivery is based on one MAC SUD, the order of PDCP PDU from different ascending order sequences is out-of-order which should be further solved by PDCP reordering.

Figure 2 provides an example of ascending order delivery with RLC data PDU missing. As shown in Figure 2, MAC SDU contains five RLC PDUs, the corresponding SNs are 2, 4, 5, 7 and 6. RLC PDU with SN 2 is a retransmitted RLC PDU. RLC PDU with SN 5 is missing. The order of RLC PDU with SN 7 and 6 are out of order.  According to the ascending order delivery mechanism, the delivery order of RLC PDU is based on SN ascending order: 2, 4, 6 and 7.

- Figure 2  Ascending order delivery with RLC data PDU missing

Proposal 1: Introduce an ascending order delivery mechanism in RLC to optimize the PDCP PDU delivery from RLC to PDCP.

Conclusions

In this paper, we have discussed RLC ascending order delivery.The observations and proposals are listed as follows:

Observation 1: Before RLC delivers data PDUs to PDCP, all RLC data PDUs should be filtered by receiving window in RLC UM and AM.

Observation 2: SN of each RLC data PDU should be recognised by RLC in UM and AM.

Proposal 1: Introduce an ascending order delivery mechanism in RLC to optimize the PDCP PDU delivery from RLC to PDCP.

References

[1] 3GPP TS 36.322

[2] R2-166883 Report of email discussion: [95#27] [LTE/NR] Reordering, Intel