3GPP TSG-RAN WG2 #97bis	Tdoc R2-1703023

Spokane, Washington, USA, 3rd – 7th April, 2017

Agenda Item	: 10.3.4.2 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: Location of QoS Flow ID in UL and DL packet

Document for	: Discussion and Decision

Introduction

RAN2 discussed the issue of reflective QoS at the NR AH meetings, and made following agreements.

## 3GPP RAN2 NR Ad Hoc 1 A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including: - QOS flow->DRB routing; - QoS-flow-id marking in DL packets; - QoS-flow-id marking in UL packets; 2 The new protocol layer is applicable for all cases connecting to the 5G-CN 3 Single protocol entity is configured for each individual PDU session.

Based on the agreements, it is expected that the QoS flow ID is included in the header of new layer (PDAP) PDU. One issue is where to locate the PDAP header in the PDCP PDU. In this contribution, we analyze the impact on header compression/de-compression depending on the location of PDAP header.

.

Discussion

PDAP PDU consists of a data field and a header. According to the examples mentioned in [1], there are two options to locate the PDAP header in the PDAP PDU.

- The PDAP header is located in front of the PDAP PDU.

- The PDAP header is located at the end of the PDAP PDU.

The PDAP header is located in front of the PDAP PDU

In LTE, one of the main functions of the PDCP is header compression and de-compression using ROHC. ROHC is a kind of algorithm to compress and de-compress the header of various IP packets. The PDCP entity associated with DRB can be configured by upper layer to use header compression. When ROHC is configured, the PDCP entity performs the header compression and de-compression. And the PDCP entity assumes that the PDCP SDU starts with IP header.

If the PDAP header is located in front of the PDAP PDU (PDCP SDU) and ROHC is configured, the PDCP entity can not perform the header compression and de-compression without modification. In order to perform the header compression and de-compression, the PDCP entity needs to know the PDAP header length. However, even if the PDCP entity knows the PDAP header length, the PDCP entity should perform additional processing. The figure 1 shows the additional processing for header compression in case the PDAP header is located in front of the PDAP PDU.

Figure 1 Additional processing for header compression in case PDAP header is located in front of the PDAP PDU

If the PDAP header is located in front of the PDAP PDU, the PDCP entity shall:

- Identify the PDAP header from the PDCP SDU

- Extract and save the PDAP header

- Perform the header compression.

- Reattach the PDAP header in front of PDCP PDU.

Additional processing is also required for header de-compression similar to header compression.

If the PDAP header is located in front of the PDAP PDU, additional processing is required for header compression and de-compression in PDCP.

The PDAP header is located at the end of the PDAP PDU

If the PDAP header is located at the end of PDAP PDU and ROHC is configured, the PDCP entity can perform the header compression and de-compression without any additional processing, because the first part of the PDCP SDU is the IP header same as LTE.

If the PDAP header is located the end of the PDAP PDU, no additional processing is required for header compression and de-compression in PDCP.

Based on the above discussions, it is proposed:

Proposal 1	The PDAP header is located at the end of the PDAP PDU.

Conclusion

In this document, we discuss where to locate the PDAP header in PDAP PDU and propose following:

Proposal 1	The PDAP header is located at the end of the PDAP PDU.

Reference

R2-1700029, “Impact to RAN User Plane Protocol architecture” Samsung