3GPP TSG-RAN WG2 Meeting #97bis	R2-1703121

Spokane, USA, 3rd – 7th April 2017

Source:	CATT

Title:	Discussion on SR/BSR

Agenda Item:	10.3.1.5

Document for:	Discussion and Decision

Introduction

There was an email discussion on SR/BSR enhancement in NR [97#62]. This contribution gives our understanding on this topic.

Discussion

SR

In LTE, D-SR is used to trigger eNB perform the UL scheduling in case of UL data arrival. Based on D-SR, the eNB would provide a fixed UL scheduling to UE regardless the service of the arrived data, and eNB would provide QoS based UL scheduling later based on more information provided in the BSR.

In NR SR/BSR discussion, almost all companies want to enhance the SR procedure and provide more QoS or service related information in SR in order to let the gNB perform QoS based UL scheduling earlier based on SR.

There are two issues to be considered on information indicated in SR transmission.

- Issue 1: The information indicated by SR

The potential information which can help gNB perform QoS based scheduling earlier based on SR is listed in Table-1.

Table-1 Potential information indicated in SR

| Information in SR | Usage in gNB | Potential consequence without it |
| --- | --- | --- |
| 1/ numerology/TTI on which the LCH with UL data can be transmitted | It can help gNB to perform correct scheduling on the corresponding numerology/TTI. | UL data only can be transmitted on the correct numerology/TTI after gNB obtains the BSR. - Long UL scheduling latency |
| 2/ priority of the LCH with UL data | It can help gNB to know whether the scheduling is urgent or not | Same scheduling scheme in gNB. - Scheduling based on SR always in highest priority. |
| 3/ data amount of the LCH with UL data | It can help gNB to give the suitable UL grant size | Not sufficient grant size to accommodate all UL data. - Long UL data transmission period. |

From the table it can be observed that if more information is needed to be indicated via SR transmission, then numerology/TTI is more important than others since it would improve the UL scheduling latency which is the key requirement in NR. For the priority and amount of data info, since it has no impact on the latency, it is not urgent for gNB to know and this information can be obtained via  BSR which is transmitted later.

Observation 1: if more information is indicated in SR, numerology/TTI information which can be used for the UL transmission scheduling should be considered with highest priority.

- Issue 2: The way to carry the information in SR

There are two options to carry the information in SR:

- Option 1: multi-bit carried in one SR transmission;

- Option 2: multiple SR transmission;

In Option 1, there is one SR transmission and one SR configuration. however the SR should be designed to carry multiple bits.  Due to the motivation to introduce the LCH and numerology/TTI mapping is to perform the scheduling according to different service’s latency requirement, if we use same SR configuration to adapt UE’s multiple services latency requirement, SR configuration would be based on the shortest latency requirement.

In Option 2, there are more than one SR transmissions and configurations, and the SR configuration can reflect different service latency requirements.

Comparing the two options from the SR resource utilization point of view, Option 2 would be better than Option 1.

Observation 2: To support services with different latency requirements, Option 2 (i.e. multiple SR configuration/transmission) has better resource utilization than Option 1 (i.e. multiple-bits SR with one SR configuration).

According to Observation 2 and Observation 3, it is proposed that

Proposal 1: NR SR enhancement should consider the multiple SR transmission/configuration which reflects the different service latency requirement.

In addition, there are some other solutions, e.g. SPS and grant free transmission could be used to reduce UL scheduling latency. In order to reflect the service’s different latency requirement, we can apply different schemes for different services, for example, grant free transmission can be configured for URLLC service and SR is configured for eMBB service. With the use of combination of schemes, UL scheduling latency requirement could be satisfied even without introducing enhancement to SR transmission.

Proposal 2: Different UL scheduling schemes can be applied for different services, e.g. grant free transmission for URLLC and SR for eMBB.  UL scheduling latency requirement could be satisfied even without introducing enhancement to SR transmission.

BSR format

For BSR format, in legacy LTE the motivation of LCG level BS is to reduce the BSR overhead, and also to reduce the granularity and complexity for gNB to perform QoS level UL scheduling. In NR, all of these reasoning are still applicable. Hence, 4 LCGs should be the baseline for NR BSR, and the number can be extended if it is necessary to reflect more scheduling QoS level.

Proposal 3: LCG level BS is proposed in NR BSR format and the LCG number can be extended if need is identified.

In addition, the BSR format should not be enhanced to accommodate additional numerology/TTI information in the BSR. This is because  configuration of LCH and LCG mapping can take the LCH and different numerology/TTI mapping into account.. For example, gNB can configure the LCHs which have the same numerology/TTI mapping into one LCG.

Proposal 4: No additional numerology/TTI information is included in NR BSR.

Conclusion

According to the analysis in section 2, it is proposed that:

Proposal 1: NR SR enhancement should consider the multiple SR transmission/configuration which reflects the different service latency requirement.

Proposal 2: Different UL scheduling schemes can be applied for different services, e.g. grant free transmission for URLLC and SR for eMBB.  UL scheduling latency requirement could be satisfied even without introducing enhancement to SR transmission.

Proposal 3: LCG level BS is proposed in NR BSR format and the LCG number can be extended if need is identified.

Proposal 4: No additional numerology/TTI information is included in NR BSR.