3GPP TSG-RAN WG2 Meeting #97bis	R2-1703163

Spokane, USA, 3 - 7 April 2017

Agenda item:	10.4.1.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Mobility Measurements in Connected Mode

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# 1	Introduction

As per RAN2#97 agreements:

Agreement

1	Cell quality can be derived from N best beams where value of N can be configured to 1 or more than 1.

FFS: Details of filtering to be applied

FFS: How the quality of the serving cell is determined (e.g. from serving beam only or cell quality)

FFS: Whether the agreement applies to both additional RS and idle RS.

FFS: Whether to only consider beams above a threshold ('good' beams)

In this contribution we discuss the connected mode measurements for NR and propose a measurement model for deriving beam- and cell-level qualities in NR which applies for both intra- and inter cell measurements in single and multi-beam systems.

# 2	Measurement Model for NR

In NR the mobility measurements, in addition to inter-cell mobility, should provide mechanism to handle intra-cell mobility. It would be desirable to have single model for intra/inter cell mobility measurements so that it applies for both single and multi-beam operation modes.

LTE principles could also be applied as baseline (with some enhancements or simply reused) for NR. To manage measurements and reporting in a beam-based system the following high level functions are considered in the proposed model:

- Physical layer measurements (L1) on reference signals

- Higher layer filtering (e.g. moving average) of the L1 measurements to determine beam quality/qualities

  - Layer 2 filtering

  - Signal quality threshold levels for beam level reporting actions and possibly time/power domain hysteresis

- Higher layer filtering (e.g. moving average) and mechanism to determine cell quality based on individual beam qualities

  - Layer 3 filtering

  - Signal quality threshold levels for cell level reporting actions and possibly time/power domain hysteresis

Observation 1: Cell quality based mobility (called ”L3 mobility”) seems to be analogous with LTE mobility defined in RRC [2]

Proposal 1: Define Cell-level quality based mobility in NR RRC

Observation 2: Beam level mobility (called “L2 mobility”) is a new level of mobility which is not defined in LTE

As the beams qualities and best beams may be changing very rapidly, especially in the higher frequencies, RRC level reporting and events are most likely going to be too cumbersome and slow to handle that. On the other hand, in lower frequency bands that are also used in LTE there is no immediate need to define beams but it would be better to have just one measurement model defined for both L2 and L3 mobility. Of course, in some scenarios L2 mobility may consist of single beam per cell i.e. there is no “intra-cell mobility” but it seems beneficial to define just one model for each and every beam scenario.

Proposal 2: Define single measurement model that handles single and multi-beam [3].

Proposal 3: Discuss which specification beam level mobility should be defined in.

### 2.1 Beam Quality vs. Cell Quality

In Figure 1 we provide the example measurement model to cover intra and inter cell measurements in NR. Input for the model are the beam specific L1 measurement samples that are L1 filtered. Input signals may be e.g. CSI-RS for mobility measurements or SS block measurements (measuring SS block RSRP using NR-SSS).

The model follows LTE principles: for each L1 measurement there is a corresponding higher layer (L2) filtered beam level sample and higher layer filtered cell quality sample. Measurement model provides one quality value for each beam and one quality value per cell. Two levels of filtering are considered to cover the intra-cell and inter-cell mobility.

As intra cell mobility is considered to have minimum involvement of RRC signaling it can be done quicker and it allows to base the mobility decisions on shorter term evaluations to be more reactive to the changing channel conditions. In contrast, the inter-cell handover is RRC-based and therefore most likely requires a “heavier” procedure at both UE and network side. It means the handover decision for inter-cell mobility would require credible, longer term evaluation of the cell to minimize the ping-pong effect and handover failure rates.

Figure 1. Example measurement model for beam and cell quality measurements

Figure 1 consists of two functions using the same measurement samples provided by L1 filtering entity. Below we depict a high level description of how the measurement model works:

Beam Quality

- L1 measurements done per physical layer signal per beam reference signal

- Higher layer filtering such as moving average (L2 / MAC layer is assumed to have filtering as RRC is not involved) is applied per DL beam at L2

  - L2 filtering parameters are configured by network

  - L2 filtering is used to determine that criteria related to beam measurements are evaluated based on stable measurements.

  - L2 filtering stabilizes the beam measurements for intra-cell beam events (e.g. to evaluate whether to trigger beam management actions such as beam reporting or beam recovery) and measurements provided to the selection function

- Filtered Beam measurements are used to evaluate criteria, e.g. for beam management actions of the serving cell and for e.g. beam quality reporting in an inter-cell measurement report (beam report)

Cell Quality

- Beam specific measurement samples are reused for determining cell quality

  - Filtered values provide further input to measurement report triggered by cell-level reporting criteria

- Selection function is applied to filtered beam measurements (i.e. considering N-highest quality beams where N is configured by network, e.g. N=1).

- UE averages selected N beams.

- Averaged value is further filtered to obtain long term cell quality value. A single value is derived for the cell and used to evaluate criteria for inter-cell event and potential action such as measurement reporting.

- Similar selection function is run for serving cell and neighbor cell measurements. UE derives a single cell quality value out of filtered beam measurement results. The same value of N should be used for serving and neighbouring, to ensure fair and credible comparison.

Observation 3: Same measurement samples can be used for determining individual beam quality levels and cell quality

Observation 4: Having two levels of filtering provides network the means to optimize handover performance by configuration of measurement report triggering

Proposal 4: Define L2 filtering of beam level measurement samples for intra-cell beam management (i.e. mobility without RRC involvement)

Proposal 5: Define L3 filtering and selection function for inter-cell mobility (i.e. with RRC involvement).

Proposal 6: Select N-highest L2 filtered RSRP values and average them for determining the cell quality. N is configured by the network.

### 2.2 Time-To-Trigger

Time to trigger parameter (TTT) is used in LTE to evaluate measurement reporting criteria for configured time. Similar principle can be reused in NR with some enhancement or options for network to optimize the UE reporting. Separate TTT parameters should be used for intra-cell and inter-cell reporting criteria/action. One way to configure the values would be to use:

- Shorter TTT values for evaluating the trigger for beam level reporting (i.e. to quickly react to beam level events such as reporting new beams or quality change on the current serving beam/set of serving beams etc..)

- Longer TTT values for evaluating the trigger for cell level events (e.g. triggering measurement reports)

The used filtering coefficients, thresholds and TTT values are configured by network using RRC signaling.

Observation 5: Separate Time to Trigger parameters for triggering beam level reporting and cell level evaluation of reporting/action criteria would be beneficial in NR.

Proposal 7: Reuse the TTT concept of LTE, i.e. trigger events after L3 filtered results fulfill event criteria for duration of TimeToTrigger for cell quality based events.

Proposal 8: Define separate events and TTT values for beam based events.

Proposal 9: Proposal 7 and Proposal 8 can be achieved by allowing to configure TTT per measurement event.

### 2.3 Example Operation of the Measurement model

Figure 2 illustrates the derivation of cell quality based on the beam RSRP measurements. Two beams are illustrated in this scenario: Beam1 and Beam2. In the figure the selection function utilizes the highest measured RSRP to represent the cell quality (N=1): Time-to-Trigger runs based on the Cell RSRP. Beam1 is above the TH1 what initiates the TTT for the cell. After a while, measurements indicate that Beam1 level falls below the TH1 but Beam2 value is above. This keeps the TTT running for cell level measurement. Once the TTT timer has elapsed, criteria for cell level measurement event can be triggered e.g. measurement report. Furthermore, similar evaluation is run per cell - based on beam level measurements.

Figure 2. Example of evaluating a cell quality with the proposed model

Figure 3 illustrates one example of beam level measurements and event/action triggering based on beam level threshold (TH2) and TTT for determining beam level events/actions. Three beams (1, 2, 3) are considered in this example. For the Beam 3, which is constantly above a specific TH2, no actions are determined. For Beam 2 the RSRP is observed to be above the TH2 for the duration of TTT (beam) it may trigger specific actions, e.g. beam management related reporting. After a while it is observed that the Beam2 RSRP level falls below TH2 and stays there for the duration of TTT, another action may be triggered. Beam 1 quality exceeds the TH2 but does not stay above the threshold for the duration of TTT thus, no actions are triggered.

Figure 3. Example of evaluating beam qualities with the proposed model

# 3 	Conclusions

The following Observations and Proposals have been made in the course of this paper:

Observation 1: Cell quality based mobility (called ”L3 mobility”) seems to be analogous with LTE mobility defined in RRC [2]

Observation 2: Beam level mobility (called “L2 mobility”) is a new level of mobility which is not defined in LTE

Observation 3: Same measurement samples can be used for determining individual beam quality levels and cell quality

Observation 4: Having two levels of filtering provides network the means to optimize handover performance by configuration of measurement report triggering

Observation 5: Separate Time to Trigger parameters for triggering beam level reporting and cell level evaluation of reporting/action criteria would be beneficial in NR.

Proposal 1: Define Cell-level quality based mobility in NR RRC

Proposal 2: Define single measurement model that handles single and multi-beam mobility

Proposal 3: Discuss which specification beam level mobility should be defined in.

Proposal 4: Define L2 filtering of beam level measurement samples for intra-cell beam management (i.e. mobility without RRC involvement)

Proposal 5: Define L3 filtering and selection function for inter-cell mobility (i.e. with RRC involvement).

Proposal 6: Select N-highest L2 filtered RSRP values and average them for determining the cell quality. N is configured by the network.

Proposal 7: Reuse the TTT concept of LTE, i.e. trigger events after L3 filtered results fulfill event criteria for duration of TimeToTrigger for cell quality based events.

Proposal 8: Define separate events and TTT values for beam based events.

Proposal 9: Proposal 7 and Proposal 8 can be achieved by allowing to configure TTT per measurement event.

# 4 	References

- RAN2#97 Chairman Notes

- TS 36.331-e21, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 14), 3GPP

- R2-170xxxx Beam management in NR, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP TSG-RAN WG2 Meeting #97bis Spokane, USA, 3 - 7 April 2017