3GPP TSG-RAN WG2 Meeting #97bis	R2-1703239

Spokane, USA, 3 - 7 April 2017

Source:	Rapporteur (ZTE)

Title:	On the scope of TS 37.340 on Multi-connectivity

Agenda Item:	10.2.1

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# Introduction

This document discusses some issues related to the scope of the new TS 37.340 on “Multi-connectivity” and the possible work-split with TS 36.300 and TS 38.300.

# Discussion

As part of the agreed NR WID [1] it has been decided to create a new multi-RAT Stage 2 (TS 37.340) on “Multi-connectivity” to cover the high level description of the Dual Connectivity options included in the WID, as well as possible future extensions to the multi-connectivity concept.

However the work-split with the other Stage 2 specs, namely TS 36.300 and TS 38.300, is not entirely clear.

One first simple issue is about “legacy” intra-LTE Dual Connectivity (with connection to the EPC). The assumption is that intra-LTE DC (and related possible enhancements) will continue to be addressed in TS 36.300, while TS 37.340 may just have a pointer to TS 36.300 for this.

Proposal 1: Stage 2 description for intra-LTE DC (with connection to the EPC) will continue to be addressed in TS 36.300.

Another key issue is about E-UTRA-NR Dual Connectivity (EN-DC), a.k.a. “option 3/3a/3x”. Here the assumption is that this will definitely be covered in TS 37.340, but it would make sense to formally agree on this.

Proposal 2: Full Stage 2 description for E-UTRA-NR Dual Connectivity (EN-DC), a.k.a.“option 3/3a/3x”, will be addressed in TS 37.340.

However we believe that EN-DC should also be mentioned in TS 36.300, as in a sense this option would fall into the E-UTRAN scope, since the connectivity towards the Core Network is still provided via S1 interface to the EPC. The simplest way to achieve this is a pointer in TS 36.300 to TS 37.340 for the description of EN-DC.

Proposal 3: E-UTRA-NR Dual Connectivity (EN-DC) will also be mentioned in TS 36.300.

Another possible issue is the work-split with TS 38.300. For instance there might be different views regarding where to cover intra-NR DC. In theory this could also be covered by TS 38.300. However it’s not clear what would happen if intra-NR Multi Connectivity operation will be introduced in the future: should this be described in TS 38.300 or in the Stage 2 TS dealing with “Multi-connectivity”? Even more, what about intra-LTE DC with connection to the 5G-CN? If intra-NR DC will be covered in TS 38.300, should intra-LTE DC with connection to the 5G-CN be covered there as well?

Proposal 4: Discuss where to cover intra-NR DC and intra-LTE DC with connection to the 5G-CN.

Note: in the draft Skeleton and initial TP in [2] and [3] the current assumption is that all the DC options (apart from intra-LTE DC with connection to the EPC) are to be covered in TS 37.340.

One final issue is about "LTE-WLAN Aggregation" and possible adaptation to 5G-CN, or the extension of the concept to "NR-WLAN Aggregation". Should these solutions be considered as part of TS 37.340 or TS 38.300?

Proposal 5: Discuss where to cover (future) solutions like “LTE-WLAN Aggregation with connection to the 5G-CN” and "NR-WLAN Aggregation".

# Conclusion

The following proposals were suggested:

Proposal 1: Stage 2 description for intra-LTE DC (with connection to the EPC) will continue to be addressed in TS 36.300.

Proposal 2: Full Stage 2 description for E-UTRA-NR Dual Connectivity (EN-DC), a.k.a.“option 3/3a/3x”, will be addressed in TS 37.340.

Proposal 3: E-UTRA-NR Dual Connectivity (EN-DC) will also be mentioned in TS 36.300.

Proposal 4: Discuss where to cover intra-NR DC and intra-LTE DC with connection to the 5G-CN.

Proposal 5: Discuss where to cover (future) solutions like “LTE-WLAN Aggregation with connection to the 5G-CN” and "NR-WLAN Aggregation".

# References

[1]	RP-170855		Work Item on New Radio (NR) Access Technology

[2] R2-1703241		TS37.340 – Draft Skeleton

[3] R2-1703242		TS37.340 – Initial Text Proposal