3GPP TSG RAN WG2 Meeting #97bis    	                                    R2-1703277

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.2.2.2

Source: 	Sony

Title:	Path selection for transmission of RRC messages

Document for:	Discussion

# Introduction

In last RAN2#97 meeting, the agreements regarding transport for the CP signalling have been reached as follows.

Agreements

1:	For the SN/MN RRC reconfiguration requiring also MN/SN RRC reconfiguration, a MN RRC message is delivered with an embedded SN RRC message.

2	UE can be configured with an SCG SRB to allow SN RRC messages to be sent directly between UE and SN.

3:	For SN RRC reconfigurations not requiring any coordination with MN then SN RRC messages can be transported directly to the UE (or eNB implementation can be deliver it embedded within a MN RRC message)

4	Measurement reporting for mobility within the SN can be transported in SN RRC messages directly from UE to SN, if SCG SRB is configured. Detail rules for UE to select transmission path for UL message to be defined in WI.

5	These agreement do not imply that the UE has to do any reordering of RRC messages.

In this contribution, we further elaborate our perspectives on how to select the transmission path for UL/DL RRC messages.

# Discussion

In order to consider the trade-off between control plane signalling reliability and efficient radio resource utilization, the dynamic transmission path selection for both uplink and downlink RRC signalling via single link or both links should be supported, e.g. dynamic switch from single RRC to RRC diversity and vice versa.

Proposal 1: Dynamic transmission path selection for RRC signalling via single link or both links should be supported.

For the selection rules, the conventional criteria e.g. link quality, coverage, UE mobility status could be considered. Take the link quality as an example, the dual path RRC signalling transmission will be triggered if the UE is experiencing link quality deterioration with MN while maintaining a relative better link quality with SN. As for the coverage criteria, e.g. the RRC diversity will be activated only when the SN has relative wide coverage. Regarding the UE mobility status, e.g. the RRC diversity will be activated for the UEs with lower moving speed. It should be noted that, the combination of these criteria is not precluded, e.g. the RRC diversity will not be activated for the SN with narrow coverage while UE moves with high speed.

Proposal 2: Link quality, coverage and UE mobility status could be considered as the candidate transmission path selection criteria.

It is envisaged that dual path RRC signalling transmission will not be triggered for every RRC message. The RRC diversity is applicable for UE specific as well as RRC message (or RRC message group) specific. The details on which RRC messages are necessary for RRC diversity needs more considerations although it is not expected that RRC specification will be impacted.

Proposal 3: RRC diversity is UE specific as well as RRC message specific.

# Conclusion

Proposal 1: Dynamic transmission path selection for RRC signalling via single link or both links should be supported.

Proposal 2: Link quality, coverage and UE mobility status could be considered as the candidate transmission path selection criteria.

Proposal 3: RRC diversity is UE specific as well as RRC message specific.