3GPP TSG-RAN WG2 Meeting #97bis	R2-1703307

Spokane, USA, 3 - 7 April 2017

Agenda item:	8.26

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Role of drx-Config-r13 with UL LAA (N.104)

WID/SID:	ASN.1 review – Release 14

Document for:	Discussion and Decision

# 1	Introduction

During asn.1 review process following point was raised:

| N.104 | 6.3.2 MAC-MainConfig::drx-Config | The role of drx-Config-r13 is unclear (with regards to UL LAA specifically, but also in general) | 3 | It should be clarified whether only the UL retransmission timer (which relates to asynchronous HARQ) applies for UL LAA, or also the OnDuration and ReTx timers Further, there is a potential Rel-13 issue: The drx-Config-r13 is a critical extension, but it seemsit doesn’t contain all the content of DRX-Config (without suffix), and requires presence of that should this have been a non-critical extension anyway originally?? Coordinator (Ericsson): The first question should be clarified or confirmed. The Rel-13 issue needs to be handled in a separate maintenance CR. |  |
| --- | --- | --- | --- | --- | --- |

In this paper, we discuss these points in more detail.

# 2	Discussion

DRX-config-r13 is defined like this:

DRX-Config-r13 ::=					SEQUENCE {

onDurationTimer-v1310					ENUMERATED {psf300, psf400, psf500, psf600,

psf800, psf1000, psf1200, psf1600}																		OPTIONAL,	--Need OR

drx-RetransmissionTimer-v1310			ENUMERATED {psf40, psf64, psf80, psf96, psf112,

psf128, psf160, psf320}

OPTIONAL,	--Need OR

drx-ULRetransmissionTimer-r13			ENUMERATED {psf0, psf1, psf2, psf4, psf6, psf8, psf16,

psf24, psf33, psf40, psf64, psf80, psf96,

psf112, psf128, psf160, psf320}

OPTIONAL	--Need OR

}

For release 14 UL LAA, the UL retransmission timer is applicable but r13 versions of DL retransmission timer as well as onDurationTimer are not relevant for UL LAA. But this is not currently clear from the field descriptions, and since the DRX-Config-r13 is a critical extension of DRX configuration, this seems to mean that UE configured with DRX-Config-r13 also overrides the legacy OnDuration and (DL) retransmission timers.

drx-Config Used to configure DRX as specified in TS 36.321 [6]. E-UTRAN configures the values in DRX-Config-v1130 only if the UE indicates support for IDC indication. E-UTRAN configures drx-Config-v1130, drx-Config-v1310 and drx-Config-r13 only if drx-Config (without suffix) is configured. E-UTRAN configures drx-Config-r13 only if UE supports CE or if the UE is configured with uplink of an LAA SCell.

Does this mean UE supporting UL LAA also automatically supports the extended OnDuration and retransmission timer values (which were originally only meant for UEs supporting CE)?

QUESTION: Does UE supporting UL LAA also support extended onDurationTimer and drx-RetransmissionTimer values?

Depending on the answer, we propose to clarify this in the 36.331/36.306.

Proposal 1: Clarify whether UE supporting UL LAA also supports extended onDurationTimer and drx-RetransmissionTimer values.

# 3	Conclusion

In this paper, we provided question on UL LAA supporting UE need to support release 13 extensions of onDurationTimer-v1310 and drx-RetransmissionTimer-v1310.  Depending on the group understanding a clarification may be necessary.

Proposal 1: Clarify whether UE supporting UL LAA also supports extended onDurationTimer and drx-RetransmissionTimer values.