3GPP TSG-RAN WG2 #97bis	R2-1703465

Spokane, Washington, USA, 3 – 7 April 2017

Agenda Item:	10.4.1.8

Source:	Huawei, Hisilicon

Title:	Automatic Neighbour Relation in NR

Document for:	Discussion, Decision

# Introduction

The function of Automatic Neighbour Relation (ANR) was specified at Rel-8 to automatically establish the relations between radio network nodes. In RAN#75, Automatic Neighbour Relation (ANR) was agreed to put into the scope of the NR WI (to be Rel-15).

# Discussion

As one of the SON (Self-organization network) feature set specified for LTE, ANR enables the automatic discovery and establishment of the neighbour relations between the radio network nodes, which can be used to construct the neighbour cell list, to establish the connections between Base Stations, and to support the mobility management, and mobility optimization and load balancing procedures.

The purpose of the Automatic Neighbour Relation (ANR) function is to relieve the operator from the burden of manually managing Neighbour Relations (NRs).

# Automatic Neighbour Relation Function as described with TS36.300

In LTE, the ANR function resides in the eNB and manages the conceptual Neighbour Relation Table (NRT). Located within ANR, the Neighbour Detection Function finds new neighbours and adds them to the NRT. ANR also contains the Neighbour Removal Function which removes outdated NRs.

The ANR (Automatic Neighbour Relation) function relies on cells broadcasting their identity on global level, E-UTRAN Cell Global Identifier (ECGI).

The ANR function works as follows:

The eNB serving cell A has an ANR function. As a part of the normal call procedure, the eNB instructs each UE to perform measurements on neighbour cells. The eNB may use different policies for instructing the UE to do measurements, and when to report them to the eNB.

- The UE sends a measurement report regarding cell B. This report contains Cell B’s PCI

- The eNB instructs the UE, using the newly discovered PCI as parameter, to read the ECGI, the TAC and all available PLMN ID(s) of the related neighbour cell.

- When the UE has found out the new cell’s ECGI, the UE reports the detected ECGI to the serving eNB.

- The eNB decides to add this neighbour relation, and can use PCI and ECGI to lookup a transport layer address to the new eNB, to update the Neighbour Relation List and to setup a new X2 interface towards this eNB.

Challenges of ANR for NR

In the framework of LTE, the ANR functions works based on the mapping between PCI and ECGI of a particular cell. UE measures the cell specific CRS before its report. With regard to NR, as discussed in RAN1, the idle mode RS (within the synchronization signal) is used to distinguish the cell identity. The legacy ANR mechanism in LTE can be reused for NR for cell level relation.

Proposal 1: In NR, the legacy ANR mechanism in LTE can be reused in NR for cell level relation as the baseline.

In the beamformed evironment for NR, on top of Idle mode RS, there was discussion on additional RS (e.g. CSI-RS) also, which is strongly related to the beam management and the provision of the beam-level information from the perspective of common control channel (xSS, PBCH). When the UE reads the system information from different beams of the cell, both Cell Identity and the beam specific information can be acquired by the UE.  It is discussed in RAN1 that specific RACH resources can be designed per beam. In order to speed up the RACH procedure in some cases (e.g. Handover), if the beam information can be provided to the UE, it may speed up the UE access to save the time for beam finding and beam determination.

In addition, it would often be the case that not all beams of a nearby cell are actually neighbours to the current gNB, e.g. some of the beams may be directed away from the gNB and not have overlapping coverage.  Depending on the final structure of xSS, it could be useful for the ANR function in the gNB to be aware of which beams are detected by the UE,

so that UEs in the future can be configured to measure at appropriate times, e.g. when the neighbour beam is expected to transmit its reference signals (time index of the SS block), and when the neighbour beam is actually active in case of beam sweeping.  This would be especially important in cases where measurement gaps need to be scheduled e.g. inter-frequency, it would be important to align the measurement gap with the expected xSS transmission by the concerned beam.  Otherwise there is a risk that the neighbour beam is active and transmitting its reference signals outside the measurement gap, making it undetectable for the UE.

The exact content of the beam information as mentioned above can be subject to the RAN1 discussion.

Following the procedure of ANR, it would be helpful to put the beam-specific information into the neighbour list of the cell.

Proposal 2: It is proposed to allow the UE to report beam specific information of common channel of the neighbours together with the cell identity during the ANR report procedure.

From RRC-connected state perspective, the RRM measurement on the additional RS was discussed in RAN2 also. There is a possibility for the network to rely on the pattern of CSI-RS to distinguish the beams transmitting the data channel (e.g. PDSCH) within a particular cell. The definition of Beam level identifiers for data channel is subject to the RAN1 discussion. The beam based management is being discussed in RAN1 and it is understood that the beam management is not across the cell boundary. So then there is no requirement to ask the UE to report the beam information of data channel during ANR procedure.

Proposal 3: It is proposed to not report the data channel specific beam information to the network during the ANR procedure.

# Conclusion

Proposal 1: In NR, the legacy ANR mechanism in LTE can be reused in NR for cell level relation as the baseline.

Proposal 2: It is proposed to allow the UE to report beam specific information of common channel of the neighbours together with the cell identity during the ANR report procedure.

Proposal 3: It is proposed to not report the data channel specific beam information to the network during the ANR procedure.

# References

3GPP TS 36.300 V13.4.0 (2016-06), “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description”