3GPP TSG-RAN WG2 Meeting #97bis	R2-1703492

Spokane, USA, 3rd – 7th April 2017	Revision of R2-1701535

Agenda Item	: 10.3.1.5 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: BSR enhancement for New RAT

Document for	: Discussion and Decision

1.	Introduction

RAN2 had an e-mail discussion [97#62][NR] to discuss possible SR/BSR enhancement in NR. Regarding BSR, RAN2 mainly discussed the granularity aspect. As a result of e-mail discussion, it was proposed regarding BSR that,

- Proposal 4. The existing LTE BSR framework is used as baseline for NR BSR framework.

- Proposal 5. The NR BSR framework shall provide the gNB with information such that it can select on which numerology to grant resources.

- Proposal 6. RAN2 to discuss whether to increase the granularity and if so how. Examples to increase granularity include increasing the number of bits in the BS field, increasing the number of LCGs, reporting BSR per logical channel, and revisiting BSR triggers to avoid mismatch between UE and gNB.

In this contribution, we present our view on granularity aspect of BSR enhancement and also other possibilities for BSR enhancement.

2.	Discussion

2.1. BSR with finer granularity

In order to avoid resource waste in scheduling, it would be desirable that BSR is performed based on a granularity which is finer than the granularity of scheduling. In this sense, in LTE, BSR per LCG is sufficient because the uplink resource is allocated per UE and the UE will multiplex data from all logical channels into one MAC PDU in LCP procedure.

In NR, numerology concept is introduced. From scheduling perspective, the implication of introducing numerology would be that:

- UL grant indicates specific numerology/TTI length;

- The UE multiplex data from logical channels that is allowed to be transmitted by using the numerology/TTI length indicated by UL grant;

In this case, BSR per LCG may lead to waste of uplink resource. As the eNB cannot tell the exact amount of data available for transmission for e.g., each logical channel or each numerology, the eNB may provide a larger amount of uplink resource based on BSR per LCG. To avoid waste of uplink resource, the UE needs to inform the eNB of the amount of data with finer granularity than LCG. For this, one way is to extend the number of LCG and use BSR per LCG. Another way is to introduce BSR per logical channel.

Option 1. BSR per LCG with increased number of LCG

In Option 1, the intention is to put logical channels with the same numerology into one LCG. However, if a logical channel is mapped to multiple numerologies, the logical channel may need to belong to multiple LCGs. Then, the UE would report more buffer size than it really has. In LTE, although there is no rule in allocating LCG, it is likely that the eNB allocates LCG by considering the priority of logical channels, e.g., grouping logical channels with similar priorities. In NR, it may not be easy for the gNB to allocate LCG nicely in consideration of numerology as well as priority.

Option 2. BSR per logical channel

In Option 2, buffer size is reported per logical channel. One may have concern about the overhead. However, the gNB can control the overhead by UL grant because the UE cannot transmit the BSR larger than the received UL grant, i.e., the UE will truncate the BSR. Note that in ProSe, SL BSR MAC CE is newly introduced where the buffer size of LCGs are included in decreasing order of the highest priority of the sidelink logical channel belonging to the LCG. Similar approach can be applied to NR BSR, i.e., the buffer size of logical channels are included in decreasing order of the priority of logical channel.

Proposal 1. In NR, buffer size is reported per logical channel.

2.1. BSR avoiding over/under-scheduling

In NR, if multi-connectivity is supported, threshold based BSR may be used as in LTE Dual Connectivity (DC).

In threshold based BSR, the MAC triggers BSR when amount of data becomes larger than the given threshold by considering that it was empty buffer when amount of data was less than the threshold.

In multi-connectivity, the buffer size would fluctuate around the threshold due to scheduling by multiple nodes. Using threshold based BSR, fluctuating buffer size may trigger BSR unnecessarily. Rather than triggering BSR every time the buffer size exceeds the threshold, it would be good to trigger BSR only when the UE suspects that the UE is under-scheduled.

In the meanwhile, the MAC today triggers BSR if data becomes available in empty buffer regardless of the amount of data. Thus, even a very small amount of data could request UL resource, and the UE may get only a small amount of data even though there are more data coming, i.e., under-scheduling. We think it is not inefficient to request UL resource too early only to report very small amount of data. Instead, it would be good to wait for a while and then request UL resource expecting that there would be more data coming.

For this, it could be discussed in RAN2 that the MAC triggers BSR only if the UE is not scheduled for a long time and certain amount of data is stuck in the UE side. One may think it delays the BSR, but benefit is to avoid over-scheduling and receive proper amount of uplink resource efficiently at the cost of patience.

Proposal 2. In NR, in order to avoid over-scheduling and facilitate efficient scheduling, the UE triggers BSR if data exceeding a certain amount is not scheduled for a long time.

In BSR operation, the buffer size refers to the amount of data available for transmission in both of PDCP and RLC. In other words, the buffer size is calculated as a sum of PDCP data amount and RLC data amount, and then reported via one BSR MAC CE. Accordingly, the eNB cannot tell the PDCP data amount and RLC data amount separately.

In LTE DC, over-scheduling problem was pointed out regarding split bearer because two eNBs may over allocate uplink resource for the PDCP data of split bearer. In the end, it was considered that good eNB coordination could avoid over-scheduling problem.

However, the situation in E-UTRA-NR Dual Connectivity (EN-DC) may be different from LTE DC. For instance, tight coordination between eNB/gNB may become more complex. Considering that tight coordination may require each node’s scheduling result dynamically, it may bring additional signaling overhead across Xx interface when short TTI is used in NR. In addition, it wouldn’t be desirable to force tight coordination between different RATs.

In order to make the coordination between eNB/gNB easier, it could be considered to report PDCP data amount and RLC data amount separately. By having PDCP data amount separately, each scheduler, i.e., eNB/gNB, could allocate uplink resource without tight coordination by having a general principle in scheduling PDCP data.

Reporting PDCP data amount separately would also be beneficial in case multiple nodes are assumed, i.e., multi-split bearer. For example, for multi-split bearer, only some eNB/gNBs may mainly serve the multi-split bearer in order to avoid resource waste [R2-166645]. In this case, reporting PDCP data amount only to some eNB/gNBs, coordination effort could be reduced.

Proposal 3. In NR, in order to avoid tight scheduling coordination in EN-DC, the UE reports PDCP data amount separately.

3.	Conclusion

In this contribution, we discussed BSR enhancement in New RAT WI, and propose:

Proposal 1. In NR, buffer size is reported per logical channel.

Proposal 2. In NR, in order to avoid over-scheduling and facilitate efficient scheduling, the UE triggers BSR if data exceeding a certain amount is not scheduled for a long time.

Proposal 3. In NR, in order to avoid tight scheduling coordination in EN-DC, the UE reports PDCP data amount separately.