3GPP TSG-RAN WG2 Meeting #97bis	R2-1703493

Spokane, USA, 3rd – 7th April 2017	Resubmission of R2-1701539

Agenda Item	: 10.3.1.4 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: RA enhancement for New RAT

Document for	: Discussion and Decision

1.	Introduction

In LTE, the UE performs only one Random Access (RA) procedure at one point in time, meaning that the UE doesn’t transmit multiple RA preambles (RAPs) in parallel.

In this contribution, we discuss a potential aspect of random access procedure enhancement in New RAT.

2.	Discussion

In LTE, the UE performs only one Random Access (RA) procedure at one point in time. If an RA procedure is initiated while another one is on-going, the UE selects one RA procedure. This could happen when, for instance, the UE receives a PDCCH order for RA initiation while performing a RA procedure initiated by MAC itself.

The reason of restricting to only one RA procedure was that there wouldn’t be any difference in RA procedures with different purposes. In other words, regardless of what initiates RA procedures, the UE will in the end get uplink grant as a result of RA procedure. Moreover, multiple RA procedures would require more complex UE/eNB behavior managing simultaneous RA procedures.

Using one RA procedure, however, means that another RAP can only be transmitted after RAR reception failure or Contention Resolution failure. As it would take a certain time duration for RAR reception and Contention Resolution, allowing only one RAP transmission may take away a possibility that the UE gets uplink grant earlier or the UE enters in RRC_CONNECTED earlier as a result of early RA procedure completion.

In NR, there is an intention to complete RA procedure quickly in order to make the RRC Connection as early as possible or to be scheduled with reduced latency. Therefore, it would be worthwhile to have means for the UE to reduce the time required for RA procedure completion. One way to achieve this purpose would be to allow a UE to transmit multiple RA preambles simultaneously. Then, the UE will be e.g., in RRC_CONNECTED or scheduled upon the earliest RA procedure completion among the multiple RA procedures. Considering that multiple RA procedures may bring a complexity in UE/eNB, we think it would be sufficient to allow multiple RA procedures only for some specific events such as RRC Connection establishment or arrival of data with high priority.

Proposal. In NR, in order to reduce latency in RA completion for high priority of event/data, multiple RA procedures is allowed.

3.	Conclusion

In this contribution, we discussed possible RA enhancement in consideration of high priority of event or data and propose:

Proposal. In NR, in order to reduce latency in RA completion for high priority of event/data, multiple RA procedures is allowed.