3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702719

# Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.2.3.5

Source:	Ericsson

Title:	0 ms support during handover procedure with dual Tx_Rx

Document for:	Discussion, Decision

# Introduction

In RAN2#97, the following has been agreed.

Agreements

1	We will aim to define HO for NR with an interruption as close to zero as possible while only having single Tx/Rx in the UE, and 0ms interruption at least for the case that the UE supports simultaneous Tx/Rx with source cell and target cell during HO

That is, we need to find solution to support 0 ms interruption during handover from source to target cell when UE support dual Tx/Rx.

In this contribution, we will discuss how this target can be fulfilled.

# Discussion

The definition of 0 ms interruption is from 38.913 which is listed below

7.7	Mobility interruption time

Mobility interruption time means the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The target for mobility interruption time should be 0ms.

According to this requirement, it means UE need to at least transmit/receive PDCP packet with source at subframe N, and then transmit/receive PDCP packet with target at subframe N+1, otherwise 0ms interruption is not satisfied.

To support 0ms interruption, apparently both source cell and UE should continue communicate with each other after the source node sends HO command to the UE. This connection between the source and the UE can be released only after UE can receive PDCP packet from the target cell.

In normal handover procedure in LTE, the source cell will not serve UE after it has sent HO command to UE, and UE will detach from source cell and try to connect to the target cell once it receives HO command from the source.

A new feature, make-before-break, has been introduced in LTE release 14, which can reduce the interruption time during HO. With this new feature, UE will not release its connection with source cell until it either

- Transmits the RACH preamble;

- Transmits the HO Complete message on the PUSCH (if RACH-less HO with UL grant received in HO Command message); or

- Monitors the PDCCH in the target cell (if RACH-less HO without UL grant in HO Command message. The UE instead receives UL grant through the PDCCH).

As can be seen, although this feature can reduce HO interruption time, it cannot meet 0ms interruption requirement as UE need to release connection before it can receive packet from target cell.

Therefore, to support 0 ms interruption, after receiving HO command, UE should only release its connection with source cell until it can receive packet from target cell.

Correspondingly after sending HO command to UE, source cell should continue serve UE in parallel to forward data to target cell so that target cell is ready to serve UE when UE connect to target cell.

The whole signalling procedure to support 0ms interruption is illustrated in Figure 1 below.

Figure 1 Flow chart during handover to support 0ms interruption

Step 1 to 5, same as those step in LTE, so no further explanation.

Step 6: Source send UE a handover command, i.e. RRCConnectionReconfiguration with MobilityControlInfo, where security KEY for target cell is included and also an indicator to inform UE to do 0ms interruption handover.

Note 1: Source can start to forward PDCP packet to target

Step 7: UE then start to synchronize with target cell, but keep its connection with target

Step 8: UE synchronized with target cell

Step 9: Target cell start to schedule UE in uplink

Step 10: UE send RRCConnectionReconfigurationComplete message to UE

Note 2: Now target can start to send UE PDCP packet. At the same time, source may continue send UE PDCP packet

Step 11: UE needs to distinguish PDCP packet from source and target cell

Note 3: The PDCP packet received by UE from target may be a duplication from source. But this still meet 0ms interruption requirement. Avoiding duplication over the air can be left to further NW implementation

Step 12: UE detach from source cell

Step 13: Source cell send SN status transfer message to target cell

Step 14: Target inform source to release UE context

Step 15: Source release resource for UE

As can be seen in the figure, there could be some overlap duration that UE will receive PDCP packet from source cell and the PDCP packet received from target cell simultaneously. This is because UE cannot know exactly when target will send PDCP packet to it so that it can detach from source right before that time to avoid receiving PDCP packet from both source and target concurrently and still meet 0 ms interruption requirement.

As different security KEYs are used at the two cells and only one PDCP entity is used to handle these packets during handover, UE need to identify which PDCP packet is from which cell so that it can decipher packets correctly.

One solution is that two RLC/MAC entities are setup during this transition period with one corresponds to data transmission/reception with source and the other corresponds to data transmission/reception with the target. UE PDCP entity then can identify where the packet comes from by checking which RLC entities deliver packets to PDCP. This is illustrated in Figure 2 below.

PDCP

RLC

MAC

PHY

Source

PDCP

RLC_S

RLC_T

MAC_S

MAC_T

PHY_S

PHY_T

PDCP

RLC

MAC

PHY

Target

PDCP packet

SN=X,X+1,X+2

UE

SN=X,X+1

SN=X+1,X+2

Figure 2 UE internal protocol stack to support 0 ms interruption

Based on the above analysis, we have the following proposals.

- After receiving HO command, UE should not release its connection with the source cell until it can receive PDCP packet from target cell to meet 0 ms interruption requirement

- How to receive PDCP packet from both source and target concurrently is an implementation issue in UE.

Besides UE, the source cell also needs to know when it does not need to serve this UE anymore. The source cell can know such information via legacy LTE procedure, i.e. when the target cell requests the source cell to release UE context after the target cell gets Path Switch Request Ack from the core network. If such delay is regarded as a bit long, source can also sense that UE has left it by checking the HARQ/RLF feedback from UE. That is, it is up to source node implementation, as was also the approach selected for the Rel-14 solution.

RAN2 does not need to specify the solution about how source node know when not to schedule UE anymore

Another impact at the source cell to support 0ms interruption is when/how to transfer SN status transfer message. In normal handover, the source cell will send to the target cell a SN status transfer message once it sends HO command to the UE. As the source cell will not serve the UE after it has sent HO command to the UE, the SN in the SN status transfer message can be determined. To support 0ms interruption, the source needs to continue serve the UE after send HO command. Then it is not possible to determine which SN it will not use for the PDCP packet.

Source gNB cannot determine the SN to be assigned to PDCP SDU for target cell when it send HO command to the UE as it need to continue serve UE.

As in LTE, the timing of the SN status transfer message can be left for network implementation. The source cell will not send status transfer message to the target cell after it send a HO command to the UE. The source cell just forwards PDCP SDU with SN assigned by it to the target cell. Source cell sends SN status transfer message to target cell after it know it does not need to serve UE anymore. This is of course still need RAN3 to decide.

Transmission timing of the SN status transfer message is left for network implementation, as in Rel-14.

# Conclusion

Based on the discussion in section 2, we have the following observation and proposals.

Observation 1	RAN2 does not need to specify the solution about how source node know when not to schedule UE anymore

Observation 2	Source gNB cannot determine the SN to be assigned to PDCP SDU for target cell when it send HO command to the UE as it need to continue serve UE.

Proposal 1	After receiving HO command, UE should not release its connection with the source cell until it can receive PDCP packet from target cell to meet 0 ms interruption requirement

Proposal 2	How to receive PDCP packet from both source and target concurrently is an implementation issue in UE.

Proposal 3	Transmission timing of the SN status transfer message is left for network implementation, as in Rel-14.